Course / Subject Name: NQ-ET 283 Basic Electronics

- MCQs

| Formative Assessment (Part A) LU (1 to 3) | Formative Assessment (Part B) LU (4 to 5) | Formative Assessment (Part C) LU (6 to 8) |
| --- | --- | --- |
| - Types of electron emissions are? a) 2 b) 3 c) 4 d) 1 - Quick electron emissions is in? a) Photoelectric emission b) Field emission c) Secondary emission d) Thermionic emission - Electron tubes were invented by - Sir Henry W. Seelay - Sir Schuyler Skaats Wheeler - Sir Samuel W. Francis - Sir John Ambrose - The first vacuum tube was invented by an English electrical engineer in - 1911 - 1904 - 1978 - 1933 - Current carriers in semiconductors are? a) Electrons and Protons b) Electrons and Nucleons c) Electrons and Photons d) Electrons and Holes - The concentration of doping is kept below ______________ a) 1 % b) 5 % c) 10 % d) 50 % - Which of the following is doped to create a P-Type Semiconductor? a) P b) Sb c) Ga d) As - Which one of the following is not an intrinsic semiconductor? a) Carbon b) Silicon c) Germanium d) Lead #### Which factor primarily influences the conductivity of an intrinsic semiconductor? - Dopant concentration - Temperature - Voltage applied - Magnetic field #### What type of bonding is formed between dopant atoms and the host atoms in a doped semiconductor? - Covalent bonding - Ionic bonding - Metallic bonding - Hydrogen bonding #### In a P-type semiconductor, the Fermi level moves: - Closer to the conduction band - Closer to the valence band - Away from both the conduction and valence bands. - To the center of the band gap. #### What is an intrinsic semiconductor? - A semiconductor doped with impurities. - A pure semiconductor without impurities - A metal with high conductivity - An insulator at room temperature. ### Which +of the following is an active element in electronics? - Diode - Resistor - Transformer - Capacitor ### Which active component is primarily responsible for signal amplification in an electronic circuit? a) Inductor b) Diode c) Transistor d) Capacitor #### What is the purpose of doping a semiconductor? a) To reduce its conductivity. b) To increase the size of its band gap. c) To introduce charge carriers for enhanced conductivity. d) To make it act as an insulator. ### In a half-wave rectifier, how many diodes are used? a) One b) Two c) Three d) Four ### In a full-wave rectifier using a center-tap transformer, how many diodes are required? ### a) One b) Two c) Three d) Four ### What is the primary function of a filter in a rectifier circuit? ### a) To increase the output voltage b) To smooth the output and reduce ripple c) To amplify the output signal d) To regulate the input current ### The efficiency of a half-wave rectifier is approximately: ### a) 50% b) 60.8% c) 40.5% d) 81.2% ### Which of the following rectifiers has the highest efficiency? a) Half-wave rectifier b) Full-wave rectifier c) Bridge rectifier d) None of the above ### Which of these components can act as both an active and passive element depending on the context? a) Transformer b) Diode c) Capacitor d) Battery ### What is the primary difference between intrinsic and extrinsic semiconductors? a) Intrinsic semiconductors have no charge carriers. b) Extrinsic semiconductors are pure and undoped. c) Intrinsic semiconductors are pure, while extrinsic are doped with impurities. d) Both are the same, with different conductivity levels. ### What type of impurity is added to form an n-type semiconductor? a) Pentavalent impurity b) Trivalent impurity c) Tetravalent impurity d) No impurity ### Which material is commonly used as a semiconductor in electronic devices? ### a) Copper b) Silicon c) Iron d) Aluminum ### What is the energy gap (bandgap) for a typical silicon semiconductor? a) 0.3 eV b) 1.1 eV c) 2.4 eV d) 5.0 eV ### What is the role of a trivalent impurity in a semiconductor? a) It creates additional electrons. b) It increases resistance. c) It creates holes. d) It makes the semiconductor a conductor. ### In a p-n junction, what forms at the interface between the p-type and n-type regions? a) Depletion region b) Conductive layer c) Hole-electron layer d) Neutral zone #### What happens when a PN junction is forward biased? a) The depletion region widens. b) The depletion region narrows. c) The current stops flowing. d) The diode becomes an insulator. #### In reverse bias, the current through a diode is typically: a) Very high b) Zero c) Leakage current (very small) d) Equal to the forward current #### What is the primary purpose of a PN junction diode? a) To amplify signals b) To rectify AC to DC c) To store energy d) To generate electromagnetic waves Answer Keys: - c; 6.a; 11.b; 16.a; 21.b; 26.c. 2.b; 7.c; 12.b; 17.b; 22.c; 27.a. 3.d; 8.a; 13.a; 18.b; 23.a; 28.b. 4.b; 9.b; 14.a; 19.c; 24.b; 29.c. 5.d; 10.a; 15.c; 20.b; 25.b; 30.b. | - Which regions are present in a bipolar junction transistor? a) Base and Emitter b) Base, Collector, and Emitter c) Anode and Cathode d) Drain, Source, and Gate - In a BJT, the base is: a) Heavily doped b) Lightly doped c) Moderately doped d) non doped - What happens to the majority carriers when a BJT operates in active mode? a) They recombine in the base region b) They travel directly to the collector c) They are stored in the emitter region d) They are blocked by the base-emitter junction - What type of charge carriers are predominant in an NPN transistor? a) Holes b) Electrons c) Ions d) Photons - In a BJT, when the base-emitter junction is forward biased, the transistor is: a) Off b) Saturated c) In active mode d) In reverse cutoff mode - What is the main function of the base in a BJT? a) To control the flow of electrons from the collector b) To amplify signals c) To control the current flow from emitter to collector d) To store charge carriers - In a BJT, the emitter: a) Emits photons b) Supplies majority carriers c) Acts as a control terminal d) Blocks current flow - If the collector current is 9.8 mA and the base current is 0.2 mA, what is the current gain (β)? a) 49 b) 50 c) 48 d) 52 - Which of the following conditions makes the BJT operate in the cutoff region? a) Base-emitter junction forward biased b) Base-emitter junction reverse biased c) Collector-emitter voltage at zero d) High current through the base - Which region of a BJT has the largest physical size? a) Emitter b) Base c) Collector d) Gate - BJTs are commonly used in: a) Light generation b) Amplification and switching c) Digital storage d) Rectification - Which type of BJT is preferred for low-noise amplification? a) PNP b) NPN c) Both d) None - What is a typical application of BJT in analog circuits? a) Voltage regulation b) Signal switching c) Rectification d) Signal amplification - Which circuit configuration is widely used for voltage amplification? a) Common emitter b) Common base c) Common collector d) Emitter follower - In a relay circuit, the BJT is often used as: a) A rectifier b) A switch c) A sensor d) A load resistor - The primary current in an FET flows through the: a) Gate b) Source and Drain c) Collector d) Base - The gate of an FET controls: a) Voltage b) Current flow c) Resistance d) Inductance - Which of the following is a key characteristic of FETs? a) High input impedance b) Low input impedance c) Low current gain d) High noise levels - What is the depletion region in an FET? a) A region with high carrier concentration b) A region devoid of carriers c) A region where recombination dominates d) A region of maximum conductivity - Which type of FET has a gate insulated from the channel? a) JFET b) MOSFET c) BJT d) Triac - In a MOSFET, the threshold voltage is the voltage at which: a) Current stops flowing b) The channel is fully depleted c) The channel starts conducting d) Drain voltage equals gate voltage - In an N-channel MOSFET, when the gate-to-source voltage is increased, the channel conductivity: a) Decreases b) Increases c) Remains constant d) Stops completely - Which of the following is true about JFET operation? a) The gate current is high b) The gate current is nearly zero c) It is current-controlled d) It has no depletion region - The pinch-off voltage in an FET refers to the: a) Voltage at which current ceases b) Voltage at which the channel is completely pinched c) Voltage required for full conduction d) Reverse bias voltage of the source - FETs are predominantly used in: a) High-current applications b) Low-power signal processing c) Rectifiers d) Induction heating - FETs are often preferred over BJTs in which applications? a) High noise environments b) Low impedance circuits c) High input impedance applications d) Power amplification - MOSFETs are widely used in: a) Rectifiers b) Digital circuits c) Transformers d) Mechanical relays - FETs are particularly suitable for radio-frequency (RF) applications due to their: a) Low gain b) High-frequency response c) Large size d) High power consumption - Which of the following circuits commonly employs FETs? a) Inverters b) Rectifiers c) Light-emitting diodes d) Motors - In power electronic circuits, which type of FET is commonly used? a) JFET b) MOSFET c) Triac d) BJT Answer Keys: 1.b; 6.c; 11.b; 16.b; 21.c; 26.c. 2.b; 7.b; 12.b; 17.b; 22.b; 27.b. 3.a; 8.b; 13.d; 18.a; 23.b; 28.b. 4.b; 9.b; 14.a; 19.b; 24.b; 29.a. 5.c; 10.c; 15.b; 20.b; 25.b; 30.b. | - What is the function of a thyristor? a) Signal amplification b) Switching and rectification c) Current limiting d) Voltage regulation - Which terminal is NOT present in a Thyristor? a) Anode b) Cathode c) Gate d) Drain - A DIAC is primarily used for: a) Amplification b) Triggering TRIACs c) Voltage regulation d) Current amplification - The DIAC operates in which mode? a) Reverse-biased only b) Forward-biased only c) Bidirectional d) Unidirectional - Which of the following is a characteristic of a TRIAC? a) Conducts in only one direction b) Conducts in both directions c) Operates only in DC circuits d) Requires no gate for triggering - TRIACs are most commonly used in: a) RF amplifiers b) AC power control c) Rectifiers d) Digital logic circuits - What type of device is a UJT? a) A diode b) A unidirectional transistor c) A switching device d) A voltage regulator - The UJT has how many terminals? a) 2 b) 3 c) 4 d) 5 - A photo transistor operates by: a) Heat stimulation b) Light stimulation c) Voltage variation d) Magnetic field - Which is NOT a component of a photo transistor? a) Base b) Collector c) Cathode d) Emitter - LASCR stands for: a) Light Activated Silicon Controlled Rectifier b) Light Amplification Silicon Control Relay c) Linear Active Semiconductor Current Regulator d) Laser Assisted Semiconductor Circuit Repeater - LASCR is triggered by: a) Voltage pulses b) Heat c) Light signals d) Mechanical force - Optocouplers are used to: a) Amplify electrical signals b) Transfer electrical signals using light c) Generate radio frequencies d) Provide AC to DC conversion - An optocoupler consists of: a) A light-emitting diode and a photo detector b) A thyristor and a TRIAC c) Two photo diodes d) A transistor and a diode - What is the primary application of opto coupler? a) Noise amplification b) Electrical isolation c) Voltage regulation d) Power amplification - Which is a characteristic of an ideal operational amplifier? a) Infinite gain and bandwidth b) High noise levels c) Zero input impedance d) Limited output voltage - The inverting input of an operational amplifier: a) Increases the input signal b) Inverts and amplifies the input signal c) Reduces signal noise d) Limits the bandwidth 18. An operational amplifier in a voltage follower configuration has a gain of: a) 0 b) 1 c) 10 d) Infinity 19. The difference between LSI and VLSI is: a) LSI has more transistors than VLSI b) VLSI contains more transistors than LSI c) LSI is used for low-power applications only d) VLSI is less complex than LSI 20. VLSI technology is primarily used in a) Simple calculators b) Large-scale processors c) Analog meters d) Electro-mechanical relays 21. SSI circuits contain: a) A few gates per chip b) Thousands of gates per chip c) Millions of gates per chip d) Only analog components 22. Which generation of integrated circuits introduced MSI? a) First generation b) Second generation c) Third generation d) Fourth generation 23. An MSI circuit typically has: a) Less than 10 gates b) Between 10 and 100 gates c) More than 100 gates d) Billions of transistors 24. Which component in an op-amp is responsible for stabilizing the output? a) Resistors b) Feedback network c) Diodes d) Transistors 25. LASCRs are primarily used in: a) Power control circuits b) Communication circuits c) Signal processing circuits d) Audio amplifiers 26. A DIAC can trigger a TRIAC when: a) Voltage across it exceeds a threshold b) Current through it is high c) Light falls on it d) A magnetic field is applied 27. What distinguishes a photo transistor from a regular transistor? a) It has a light-sensitive base b) It operates on magnetic fields c) It lacks an emitter terminal d) It is unidirectional 28. Which device provides electrical isolation in high-voltage circuits? a) TRIAC b) UJT c) Optocoupler d) LASCR 29. VLSI design is crucial in which field? a) Power grids b) Microprocessor fabrication c) High-frequency radio d) Simple analog devices 30. The gate current required to trigger a thyristor is: a) Very high b) Zero c) Moderately low d) Negligible Answer Keys: 1.b; 6.b; 11.a; 16.a; 21.a; 26.a. 2.d; 7.c; 12.c; 17.b; 22.c; 27.a. 3.b; 8.b; 13.b; 18.b; 23.b; 28.c. 4.c; 9.b; 14.a; 19.b; 24.b; 29.b. 5.b; 10.c; 15.b; 20.b; 25.a; 30.d. |

- Limited Response Questions

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) |
| --- | --- | --- |
| LU (1 to 3) | LU (4 to 5) | LU (6 to 8) |
| - Write names of 3 passive electrical components? - Write abbreviations of LED, PNP and NPN? - Name the Majority Charge carriers in P-type and N-Type semiconductor? - Define Ripple Factor? - What is Doping? - Write name of any 2 semiconductor material? - Pentavalent elements are doped with Si or Ge to form which type of semiconductor? - Trivalent elements are doped with Si or Ge to form which type of semiconductor? - Name types of electron tubes? - What should be the connection between diode and battery in reverse bias? | - JFET and MOSFET are the acronym for? - What is transistor? - Draw NPN and PNP Transistor? - Define current gain of Transistor? - Define voltage gain of Transistor? - Define power gain of Transistor? - Name types of Field Effect Transistor? - Mention types of Metal Oxide Silicon Field Effect Transistor (MOSFET)? - Draw symbol of IGFET and MOSFET? - Write 3 uses of Transistor? | - Define Thyristor? - What does the term DIAC, TRIAC and SCR means? - Draw symbol of Op-Amp and label it? - Define SSI, MSI and LSI? - Enlist 3 applications of UJT? - What is Opto-Coupler? - Write applications of DIAC and TRIAC? - State Peak Firing Voltage equation? - Write 3 uses of photo transistor? - Draw characteristic curve of UJT? |

- Assessment Tasks

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) |
| --- | --- | --- |
| LU (1 to 3) | LU (4 to 5) | LU (6 to 8) |
| Construct a voltage regulator circuit with a 220V AC input using bridge rectifier and draw output waveform of the circuit. - Adhere to all health and safety guidelines relevant to the task. - Set up the workstation to meet the task requirements. - Collect the required tools, equipment, and components for the job. - Construct a bridge rectifier circuit using four diodes. - Connect the step-down transformer (220V to 13V) output to the bridge rectifier. - Attach a resistor (Rs) in series to control the input current. - Connect a capacitor in parallel with the bridge rectifier circuit to reduce the riples / fluctuations in output voltage by charging and discharging during the crest and troughs of the input wave. - Connect the circuit with 220V AC input. - monitor the circuit's output waveform with the help of an oscilloscope. - Document the observed waveform with the oscilloscope. - Connect a zener diode in parallel with the load after the capacitor filter. It will provide regulated and constant dc voltage to the load irrespective of the fluctuations in input voltage. - Measure the output voltage using a multimeter. - Record the voltage waveform using the oscilloscope. - Verify the filtered and regulated output voltage with a multimeter to ensure compliance with design specifications. - Test the circuit under various load conditions, assessing stability, ripple voltage, efficiency, and heat dissipation. - Record the performance data and output readings. | Demonstrate the operation and determine the voltage gain of a typical JFET voltage amplifier also demonstrate and measure the Zero bias characteristics of a metal oxide semiconductor field effect transistor (MOSFET). - Select the components and tools required to perform the task. - Set up the JFET amplifier circuit with the JFET in a common-source configuration. - Connect resistors Rs to the source terminal and Rd to the drain terminal. - Place a bypass capacitor across Rs​ to enhance the voltage gain by bypassing the AC signal around the source resistor. - Connect a resistor Rs to the gate terminal to provide biasing and ensure proper operation of the JFET. - Apply a small input signal to the gate terminal using a signal generator. - Attach a suitable load resistor to the output at the drain terminal. - Connect the circuit with a suitable DC supply voltage, ensuring proper polarity and voltage levels. - Measure the input voltage Vin using an oscilloscope/ multimeter at the gate terminal. - Measure the output voltage Vout​ across the load resistor connected to the drain terminal.C - Calculate the voltage gain using the formula Av​=Vout​/Vin​ and noted the result. - Set up a simple circuit to measure the zero-bias characteristics of a MOSFET in a common-source configuration. - Connect the gate terminal of the MOSFET directly to the source terminal to achieve a zero-bias condition. - Connect circuit with a DC supply voltage between the drain and source terminals, ensuring proper connections. - Connect a multi-meter in series with the drain terminal to measure the drain current Id. - Gradually increase the DC supply voltage while keeping the gate-to-source voltage VGS at zero. - Record the corresponding drain current Id for each increment in drain-to-source voltage VDS. - Plot the drain current ID versus the drain-to-source voltage VDS​ on a graph to observe the zero-bias characteristics of the MOSFET. - Analyze the plot to identify key characteristics such as the threshold voltage and saturation region. | Demonstrate the four triggering modes of a TRIAC also Measure the peak emitter firing voltage of a uni-junction transistor. - Connect the TRIAC in a basic test circuit with a load connected between the TRIAC's MT2 and the AC supply. - Provide a gate triggering circuit to control the TRIAC's operation. - Mode 1: (MT2 positive, Gate positive), apply a positive voltage to MT2 and a positive trigger pulse to the gate. - Observe the TRIAC conducting and lighting the load during the positive half-cycle of the AC voltage. - Mode 2: (MT2 positive, Gate negative), apply a positive voltage to MT2 and a negative trigger pulse to the gate. - Observe the TRIAC conducting and lighting the load during the positive half-cycle. - Mode 3: (MT2 negative, Gate positive), appliy a negative voltage to MT2 and a positive trigger pulse to the gate. - Observe the TRIAC conducting and lighting the load during the negative half-cycle. - Mode 4: (MT2 negative, Gate negative), apply a negative voltage to MT2 and a negative trigger pulse to the gate. - Observe the TRIAC conducting and lighting the load during the negative half-cycle. - Record observations for all four modes to confirm the triggering behavior. - Set up a circuit to measure the peak emitter firing voltage of a uni-junction transistor (UJT) using a resistor and capacitor connected in a relaxation oscillator configuration. - Connect the UJT emitter terminal to the junction of the resistor and capacitor. - Power the circuit with a suitable DC supply voltage across the UJT. - Attach an oscilloscope probe to the emitter terminal to measure the voltage waveform. - Gradually increase the supply voltage and observed the point at which the emitter voltage peaks and the UJT starts conducting. - Record the peak emitter voltage observed on the oscilloscope at the firing point. - Repeat the measurement to ensure accuracy and consistency of the peak emitter firing voltage. |