Course / Subject Name: NQ-ET 203 DC MACHINES & BATTERIES

- MCQs

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) |
| --- | --- | --- |
| LU (1-9) | LU (10-18) | (LU 19-23) |
| - The induced E.M.F by electromagnetic induction was firstly stated by the scientist. a) Heinrich Lenz b) Charles Augstin De-Coulomb c) Michael Faraday d) Joseph Henry - According to Faraday’s laws of electromagnetic induction, the induced E.M.F is a) N dt. dϕ b) N dϕ /dt c) N dϕ. dt d) N + dt dϕ - According to Fleming’s right hand rule the thumb points a) Movement of conductor b) Direction of magnetic field c) Direction of current d) None of these - The purpose of commutator in a DC Machine is to a) Spread out the flux in air gap b) Convert A.C into pulsating D.C c) Reduce effect of armature reaction d) Change the position of carbon brushes - ___ insulation is usually used between commutator segments. a) Paper b) Mica c) Fabric d) Tape of glass - Brushes are made up of a) Copper b) Cast iron c) Carbon d) Mica - A coil with any number of turns has a) 1 coil side b) 2 coil sides c) 4 coil sides d) Any number of sides - Winding pitch for lap winding is a) Y = YB – YF b) Y = YB + YF c) Y = YB x YF d) Y = YB / YF - If the coil-span (or coil pitch) is equal to pole pitch, it is called a) Single layer winding b) Full pitched winding c) Simplex winding d) Short pitched winding - This is not the part of D.C machine a) Armature b) Commutator c) Damper Winding d) Field Winding - In retrogressive lap winding, commutator pitch (YC) will be equal to a) + 1 b) – 1 c) +2 d) 0 - The lap winding is suitable for a) High current, high voltage b) High current, low voltage c) Low current, low voltage d) Low current, high voltage - The basic requirement of a D.C Armature is that it must be a) Close one b) A lap winding c) A wave winding d) Either b or c - E.M.F Equation of a D.C Generator is a) E= ϕ Z N P /60 A b) E= ϕ Z/ N P 60 A c) E= ϕ Z N P /60 A d) Z E= ϕ N P 60 A - Fleming’s right hand rule is used for a) DC generators b) DC Motors c) Induction Motors d) Transformers Answer Keys: 1.c; 2.b; 3.a; 4.b; 5.b. 6.c; 7.b; 8.a; 9.b; 10.c. 11.b; 12.b; 13.a; 14.a; 15.a. | - An electrical motor is a machine which converts a) Electrical energy into mechanical energy b) Chemical energy into electrical energy c) Mechanical energy into electrical energy d) Electrical energy into chemical energy - The direction of mechanical force produced in a DC motor can be determined by a) Faraday’s Law b) Lenz’s Law c) Coulomb’s Law d) Fleming’s Left-Hand Rule - Back e.m.f. is induced in a) Shunt field b) Series field c) Commutator d) Armature - Formula to obtain armature current of D.C motor is a) 𝑉+ 𝐸𝑏 /𝑅𝑎 b) (𝑉− 𝐸𝑏)/ 𝑅𝑎 c) 𝑉+ 𝐸𝑏 /𝑅𝑎+ 𝑅𝑠ℎ d) 𝑉 /𝑅𝑎 - Field winding of shunt motor consists of a) Few turns of thin wire b) More turns of thin wire c) Few turns of thick wire d) More turns of thick wire - Compound motor consists of a) Series field winding b) Shunt field winding c) Adjusting winding d) Both (a) and (b) - The direction of d.c. series motor can be reversed by interchanging the connection of a) Series field winding b) Armature winding c) Only (a) or (b) at a time d) Both (a) and (b) at a time - If armature current is increased then speed of series motor will be a) increased b) decreased c) constant d) zero - Which motor is considered as constant speed motor a) Series b) Shunt c) Compound d) Both (a) and (b) - This is not a part of three-point starter a) Variable tapped resistor b) Magnetic holding coil c) Magnetic contactor d) Moving system with handle - Terminal ‘A’ of 3-point starter is connected to a) Main supply switch b) Field terminal c) Armature d) Both (b) and (c) - Which of the following test can be performed for any DC Machine a) Brake test b) Swinburne’s test c) Hopkinson’s test d) None of these - 3 point and 4-point starters are used for a) Series motor b) Shunt motor c) Compound motor d) All of these - The maximum torque of DC Motor is limited by a) Commutation b) Heating c) Speed d) Armature current - Formula to find commercial efficiency of DC Motor is a) 𝐵𝐻𝑃 x 746 / 𝑉. 𝐼 b) 𝐵𝐻𝑃 x 746 /𝐸𝑏. 𝐼𝑎 c) 𝐸𝑏. 𝐼𝑎 / 𝑉. 𝐼 d) 𝐻𝑃 x 𝐸𝑏. / 𝐼𝑎 𝑉. 𝐼 Answer Keys: 1.a; 2.d; 3.d; 4.b; 5.b. 6.d; 7.c; 8.b; 9.b; 10.c. 11.c; 12.a; 13.b; 14.d; 15.a. | - A cell or battery converts a) Electrical energy into mechanical energy b) Chemical energy into electrical energy c) Mechanical energy into electrical energy d) None of these - Two different metal conductors in a battery are called a) Electrodes b) Electrolytes c) Separators d) Cell connectors - The specific gravity of water is a) 1.000 b) 1.265 c) 1.835 d) 2.030 - The capacity of a cell is measured in a) watt-hours b) watts c) amperes d) ampere-hours - In a Lead acid battery, the No. of negative plates are always____than positive plates a) Equal b) 1 more c) 1 less d) 2 more - The state of fully charged lead acid battery can be determined by a) Voltage b) Specific gravity c) Temperature d) Both (a) and (b) - Battery is a ______device a) Mechanical b) Electro-chemical c) Electro-mechanical d) Electrical - The separators of Li-ion battery are made up of a) PVC b) Porous polymer c) Plastic d) Rubber - The open circuit voltage of new Li-ion cell is about a) 1.36 V b) 2.2V c) 3.6V d) 4.8V - This is not the advantage of Li-ion cell a) Small size b) Light weight c) Long service life d) Cheap - In a lead acid cell, PbSO4 forms during a) Charging process b) Both charging & discharging process c) Discharging process d) Manufacturing process - The cell which cannot be recharged after discharging is called a) Primary cell b) Secondary cell c) Lead acid cell d) Edison’s cell - In any condition, the temperature of battery should not be greater than a) 37oC b) 50 oC c) 75 oC d) 100 oC - Due to overcharging of Lead-acid battery a) electrolyte is reduced b) Internal resistance is increased c) None of (a) and (b) d) Both (a) & (b) - The cell which can be recharged after discharging is called a) Primary cell b) Secondary cell c) Lead acid cell d) Edison’s cell Answer Keys: 1.b; 2.a; 3.a; 4.d; 5.b. 6.d; 7.b; 8.b; 9.c; 10.d. 11.c; 12.a; 13.b; 14.d; 15.b. |

- Limited Response Questions

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) |
| --- | --- | --- |
| LU (1-9) | LU (10-18) | (LU 19-23) |
| - Define Separately Excited D.C Generator. - Write the names of eight important parts of a D.C Machine. - O.C.C Stands for? - What is the function of brushes in a D.C machine? - Write the formula of induced E.M.F. in lap wound generators. - What is the function of Interpoles in a D.C Machine? - Define simplex lap winding - Draw the power stages of DC Generator. - Enlist the variable losses of DC Generator. - Define constant losses of DC Generator. | - State the basic working principle of DC Motor. - State back EMF of DC Motor. - State the names of different types of DC Motors. - Write the basic function of DC Motor Controller. - Define efficiency of Motor. - Which motor is suitable for lathe machine and why? - Enlist the speed control methods of DC Series Motor. - What are three terminals of 3-Point Starter? - What is meant by Motor Controller? - Write two differences between Series Motor and Shunt Motor. | - Define a Battery. - Define primary cell. - Name the factors on which output of cell depends. - State two features of zinc-carbon dry cell. - State differences between primary cell and secondary cell. - What is meant by separator in batteries? - What is connecting bar in a battery.? - What is meant by battery formation? - Write the chemical equation for Lead-acid battery. - What is the function of vent plugs in a battery? |

- Assessment Tasks

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) |
| --- | --- | --- |
| LU (1-9) | LU (10-18) | (LU 19-23) |
| Calculate and record DC machine Armature data and also plot open circuit characteristics of DC shunt generator. Performance Criteria: - Wear PPEs for ensuring health and safety - Select the required tools/equipment/materials and DC Machine. - Place DC machine on workbench - Disassemble the DC Machine. - Remove the armature from DC Machine. - Count number of slots in armature core. - Count total number of Coils in Armature winding. - Record the armature data - Check the commutator surface - Clean the commutator - Place carbon brushes in correct position - Reassemble the DC Machine. - Connect DC supply to the field coil of DC shunt generator - Connect rheostate, ampere meter in series with field winding - Connect voltmeter across armature terminals to find out induced EMF in armature winding - Couple armature shaft with prime mover - Rotate the armature at constant speed - Switch on the DC power supply - Record induced EMF and current by Decreasing or increasing the resistance by rheostate. - Repeat the process till magnetic field of field winding reaches its saturation value. - Draw graph between field current and armature voltage from the recorded values. | Operate DC Shunt motor , measure the speed by techmeter and also control the speed of motor by applying field control method. Performance Criteria: - Wear PPEs for ensuring health and safety - Select the required tools/equipmengt/MATERIALS and instruments - Install a shunt motor - connect both field winding & Armature winding in parallel connection. - Apply rated voltage - Place techmeter near the motor - Measure the speed of DC shunt motor. - Switch off the DC shunt motor - Construct the circuit to contorl the speed of motor. - Connect rheostate with field coil of motor in series in the circuit . - Switch ON supply - Increase the voltage gradually to its rated voltage. - Record speed at different supply voltages. - Make graph between speed and applied voltage. - Switch off the motor. - Disassamble the circuit. | Prepare Electrolyte for Lead Acid Battery, measure gravity of electrolyte, connect the battery to the battery charger and record the charging voltage. Performance Criteria: - Wear PPEs for ensuring health and safety - Select required Tools / Equipment/materials - Select proper container to prepare electrolyte. - Identify the required Acid(H2SO4) - Put H2SO4 in the container as per requirment - Add water drops to the container as required and slowly stir the mixture with a wooden rod until solution of water and H2SO4 incorporate completely. - Measure Specific Gravity of Electrolyte using Hydrometer. - Pour electroltye in the battery container carefully - Check the volume of electrolyte in a battery container. - Close the vent plug of lead acid battery - Set voltage and current setting of charger. - Connect battery at the output terminals of charger. - Turn on supply and note the current reading of ammeter from charger. - Keep battery connected with charger till the ammeter shows minimum or zero reading. - Disconnect battery form the charger. - Remove battery charger and palace it in the specified area. |