### Course / Subject Name NQ-ET 273 Electrical Instruments and Measurements

### MCQs

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) |
| --- | --- | --- |
| LU (1 to 9) | LU (10 to 16) | LU (15) |
| - What is the primary safety precaution when using a multimeter for measurements? a) Ensure the multimeter is turned off when connecting probes b) Wear rubber gloves only c) Ensure the multimeter is calibrated d) Always use the highest range setting - The internal resistance of an ampere meter is - Very low - Very high - Medium - High - Which of the following is the correct setting for measuring current using a multimeter? a) Voltage mode b) Resistance mode c) Current (Amps) mode d) Continuity mode - When measuring resistance with a multimeter which of the following is required? a) Power must be turned on in the circuit b) The multimeter should be set to voltage mode c) The multimeter should be set to resistance mode d) The circuit should be disconnected from the power supply - How should the tachometer be positioned when measuring RPM? a) At a distance from the rotating object b) In direct contact with the rotating object c) At an angle to the rotating object, following the manufacturer's instructions d) Directly below the rotating object - Most efficiant method of damping is a)Air Friction damping b)Fluid friction damping c)Dynamic d)Eddy current damping - What unit of measurement does a lux meter display? a) Watts b) Volts c) Wb/m2 d) Amperes - Where should the lux meter be placed for accurate illumination measurement? a) Near the power source b) At the point where the light is being utilized c) At the center of the circuit d) Away from the light source - What is the most common use of a lux meter? a) Measuring the temperature of a light bulb b) Measuring the voltage of a circuit c) Measuring light intensity or illumination levels d) Measuring current flow in an electrical circuit - What is the first step in using an oscilloscope to measure frequency and voltage? a) Connect the probes to the circuit b) Turn on the oscilloscope and check calibration c) Adjust the time base setting d) Measure the illumination - It is used in Recording Instruments a)Pointer b)Pen c)A graph paper d)Boht B and C - What does the oscilloscope display when properly set for voltage measurement? a) A continuous tone b) A waveform of the electrical signal c) The current in amperes d) The resistance value - What is the correct procedure to connect an oscilloscope probe to a circuit? a) Connect the probe's signal lead to the positive terminal and the ground lead to the negative terminal b) Only connect the ground lead to the circuit c) Connect both leads of the probe to any two wires in the circuit d) Connect the signal lead to the power source - Why is it essential to calibrate the measuring instruments before use? a) To ensure the accuracy of the measurements b) To increase the lifespan of the instrument c) To reduce the need for additional equipment d) To speed up the process - What should be checked before using a multimeter for voltage measurement? a) The Multimeters battery charge b) The Multimeters resistance setting c) The circuit's current flow d) The circuit's insulation - Why is it important to document the measured parameters? a) For future reference and compliance purposes b) To ensure that the equipment is not damaged c) To increase the speed of installation d) To reduce the cost of measurement - When using a multimeter to measure voltage, how should the probes be connected? a) Series with the load b) Parallel with the load c) Across the power supply only d) Parallel to the ground - Which of the following is an absolute instrument a)Digital Volt meter b)Digitalampere meter c)Analogue ampere meter d)Tangent Galvanometer - What is the main purpose of using an oscilloscope to measure frequency? a) To measure the current in the circuit b) To observe the waveform and measure its frequency c) To measure the resistance of components d) To check the voltage levels in the circuit - Ampere Meter is connected a)Serice b)Perallel c)Both In Serice and Perallel d)None of these - How can you verify the accuracy of your measurements? a) Compare the readings with expected or standard values b) Use the lowest range setting for all measurements c) Measure only one parameter at a time d) Ensure the instruments are used in the same way each time - The material used for control spring in indicating instruments - Phosphor bronze - Hardroled silver - Silicon bronze - Al ofthese - This is an example of Analogue instrument a ) Multi-meter having pointer and scale b ) Electromechanical energy meter c ) Red type frequency meter d) Digital Multimeter - No observer is needed for these instruments during use - Recording instruments, - None of these - Indicating instruments - Digital instrument - Ohm meter is used for a ) Measurement of voltage b ) Measurement of resistance c) Measurement of power d) Al of these - Internal resistance of a volt meter is: - Very low - Very high - Medium - Almost zero - To get correct reading in indicating, instruments, it is essential - Deflecting force - Controling force - Damping force - All of these - This force bring the pointer from final position to zero position is - Deflecting torque - Controling torque - Damping torque - Both a & b - The value of damping in which pointer stop at last stage without oscillation in short time is called: - Oping - Under damping - Critical damping - None ofthese - These instruments are not indicated the measured value directly - Absolute instruments - Secondary instruments - Electrical resonance frequency meter - Frequency meter Answer Keys: Formative Assessment (Part A). 1; a; 16; a. 2; a; 17; b. 3; c; 18; d. 4; d; 19; b. 5; c; 20; a. 6; d; 21; a. 7; c; 22; a. 8; b; 23; a. 9; c; 24; a. 10; b; 25; b. 11; d; 26; b. 12; b; 27; d. 13; a; 28; b. 14; a; 29; b. 15; a; 30; a. | - What is the primary function of a Megger in electrical testing? a) To measure voltage b) To measure earth resistance c) To measure current d) To measure power factor - What must be ensured before using the Megger for earth resistance testing? a) The circuit is powered on b) The earth electrode is properly grounded c) The Megger is calibrated and in good working condition d) The multimeter is set to resistance mode - What is the correct connection method for measuring power factor with a Power Factor Meter? a) Connect the meter in series with the power supply b) Connect the meter across the load in parallel c) Connect the meter across the power source d) Connect the meter to the earth electrode - Which of the following is the primary purpose of measuring power factor? a) To determine the earth resistance b) To measure the total current flowing in the circuit c) To evaluate the efficiency of the electrical system d) To measure the resistance of a component - Which of the following is a key component of the Wheatstone Bridge? a) Capacitor b) Inductor c) Resistor d) Power source - How should the Wheatstone Bridge be balanced to measure medium resistance? a) Adjust the voltage b) Adjust the current c) Adjust the resistors until the galvanometer shows zero current d) Adjust the load connected to the bridge - What is the main limitation of a Wheatstone bridge? a) Cannot measure high resistance accurately b) Requires an external power source c) Difficult to balance d) Limited to AC measurements only - The Method use accurate measurement of less then one ohm - Weatstone bridge - Kelvin double bridge - Wane bridge - Maxwell bridge - Megger is combination of - Motor and Generator - Generator and ammeter - Generator and voltmeter - Generator and OHM meter - Why is it important to record the measurement results systematically? a) To analyze the data later b) To verify the accuracy of the instruments used c) For future reference and comparison with expected values d) All of the above - What is the purpose of using a 250V or 500V Megger for earth resistance testing? a) To ensure accurate measurement of high resistance b) To ensure that the earth electrode is not overloaded c) To test insulation resistance d) To prevent accidental short circuits during measurement - What is the main advantage of using a Wheatstone Bridge for resistance measurement? a) It provides high accuracy for low resistance measurements b) It is quicker than other resistance measurement techniques c) It provides a direct current measurement d) It is suitable for measuring inductance - In the Wheatstone Bridge when the galvanometer shows zero current the bridge is said to be: a) Unbalanced b) Balanced c) Incomplete d) Faulty - The power factor meter displays a value of 0.8 what does this indicate? a) The system is 80% efficient b) The system has a poor power factor c) The current is 80% of the total power d) The load is balanced - Techometer are such type of instrument which are used for a)Measurement of rotating speed b)Measurement of pressure c) Measurement of illumination d)Measurement of frequency - What is the first step in using the Wheatstone Bridge to measure medium resistance? a) Balance the bridge with a known resistance b) Set the bridge to its highest resistance setting c) Place the unknown resistor in the correct arm of the bridge d) Set the power supply to zero volts - In electronics voltmeter ,meter movement used is a)D Arsonval type b)Electronic type c)Digital type d)None of these - When using the Megger for earth resistance testing what type of test result would indicate a well-functioning earthing system? a) High resistance values b) Low resistance values c) Zero resistance d) Unstable resistance - This is advantage of digital instruments a)These can hold data b)Have simple circuit c)Repair is easy and cheaper d)Sampling rate is high - The Wheatstone bridge is used to measures - Current - Resistance - Voltage - power - Meggar is used to measure a)Current b) Voltage c)Insulation resistance d)Power - What is the primary purpose of an earth resistance tester? a) Measure soil resistivity b)Determine the resistance of earthing systems c) Test the insulation of wires d) Measure current flow - Which method is commonly used to measure earth resistance? a) Direct current (DC) method b) Two-point method c) Three-point (fall of potential) method d) Wheatstone bridge method - What is the acceptable range of earth resistance for a domestic earthing system? a) 0–1 ohms b) 1–10 ohms c) 10–50 ohms d) 50–100 ohms - What does a Megger measure? a) Current flow b) Voltage potential c) Insulation resistance d) Capacitance - What is the typical voltage range generated by a hand-operated Megger? a) 10–50V b)100–500V c) 500–1000V d) 1000–5000V - Why should equipment be de-energized before using a Megger? a) To prevent overheating of the Megger b) To avoid damage to the equipment being tested c)To ensure accurate readings and safety d)To avoid discharging the battery of the Megger - What does a power factor meter measure? a) Voltage only b) The ratio of real power to apparent power c) Current only d) The phase angle between voltage and current - What is the power factor of a purely resistive load? a) 0 b)0.5 c)1 d) Infinity - What condition indicates balance in a Wheatstone bridge? a) Voltage across the bridge is maximum b) Voltage across the bridge is zero c) Current through the galvanometer is maximum d) Resistance values are equal in all branches Answer Keys: Formative Assessment (Part B). 1; b; 16; c. 2; c; 17; a. 3; b; 18; b. 4; c; 19; a. 5; c; 20; b. 6; c; 21; c. 7; a; 22; b. 8; b; 23; c. 9; d; 24; b. 10; d; 25; c. 11; a; 26; c. 12; a; 27; c. 13; b; 28; b. 14; b; 29; c. 15; a; 30; d. | - For a load that operates on a single phase supply, which type of energy meter should be selected? a) Three-phase energy meter b) Single phase energy meter c) Digital energy meter d) None of the above - Why is it important to ensure that the energy meter is calibrated before installation? a) To check the brand of the meter b) To ensure accurate measurement of energy consumption c) To check the color of the meter d) To ensure the meter can handle the load - When installing a single-phase energy meter, which two wires must be connected to the meter terminals? a) Phase (P) and Neutral (N) b) Phase (P) and Earth (E) c) Neutral (N) and Ground (G) d) Phase (P) and Load - In a single-phase system, where should the Phase and Neutral wires be connected? a) To the load terminals b) To the meter’s designated terminals c) To the ground terminal d) To the output terminals - In a three phase system which terminals are connected to the energy meter? a) Only Phase 1 (L1) and Neutral (N) b) Phase 1 (L1), Phase 2 (L2), Phase 3 (L3), and Neutral (N) c) Phase 1 (L1) and Phase 2 (L2) only d) Neutral (N) and Ground (G) - How many wires are required to connect a three-phase energy meter? a) Two wires b) Three wires c) Four wires (three phases and neutral) d) Four wires (three phases and earth) - After connecting the energy meter where should the load be attached? a) To the neutral terminal of the meter b) To the output terminals of the energy meter c) To the earth terminal d) To the input terminals of the meter - What is the first thing to do when turning on the system after completing the installation? a) Immediately check the load b) Monitor the energy meter to ensure accurate measurement c) Increase the load gradually d) Ensure the meter display is blank - What should be done if the energy meter fails to measure energy consumption accurately? a) Test the meter with a known load b) Adjust the meter to a lower reading c) Increase the load and test again d) Wait for the meter to adjust automatically - What is a key factor to consider when selecting an energy meter for a system? a) The meter's aesthetic appearance b) The meter’s rated capacity should match the system's load requirements c) The energy meter's color d) The meter's manufacturer - What type of energy meter should be used for a heavy industrial load with a three phase supply? a) Single-phase digital meter b) Three-phase energy meter with higher rating c) General-purpose meter d) Smart energy meter with Wi-Fi - When connecting a load to a three phase energy meter what should be ensured? a) The load is properly balanced across all three phases b) The load is connected only to one phase c) The load is connected in series d) The load is minimal to avoid overloading - After installation, what should be done to verify that the energy meter is functioning properly? a) Test the meter with a standard known load b) Disconnect the load and check the meter readings c) Test the meter without any load d) Wait for the meter to warm up before checking - What is the correct approach if the energy meter is not showing expected values? a) Adjust the load b) Reset the meter to zero c) Check all connections, including load and wiring d) Replace the meter immediately - How can the accuracy of an energy meter be maintained? a) By cleaning the meter’s display regularly b) By ensuring regular calibration and inspections c) By turning off the meter every week d) By covering the meter with insulation - If an energy meter reads higher than expected consumption what could be a potential cause? a) Overloading of the system b) A low-quality meter c) Incorrect wiring or meter malfunction d) All of the above - If the energy meter shows no reading what should be checked first? a) The meter’s warranty b) The energy supply and connections c) The meter brand d) The physical appearance of the meter - Energy meter is also called - KWH meter - Watt hour meter - Watt meter - Both a & b - Cable from energy meter to main switch is - sub main - main - final - service line - Torque on the disc of energy meter is produce by - Current coil - Voltage coil - Permanent magnet - Both A and B - An energy meter counts one unit when a)1kw power is used in one mili Second b)1kw power is used in one hours c)1w power is used in one mint d)1w power is used in one second - This is not a part of Single phase energy meter a)Braking Magnit b)Current Coil c)Pressure Coil d)Controling Spring - Energy meter Measure the a)Power of a Circuit b)Power consumed in a specific time c)Boht A and B d)None of these - Current coil of energy meter is connected in series with a)Supply b)Load c)Between Supply and load d)Perallel of the Supply - Energy meter is an ______ Instrument. a)Recording b)Indicating c)Intergrating d)All of these - What function should be selected on the LCR meter to measure the inductance of a coil? a) Resistance b) Capacitance c) Inductance d) Frequency - What should you ensure when selecting the correct range on the LCR meter? a) Select the maximum range to avoid overloading b) Select the range that matches the expected measurement c) Select the lowest range to maximize accuracy d) Use the same range for all measurements - When measuring inductance, capacitance, or resistance with an LCR meter what safety precaution should be followed? a) Always measure with the circuit powered on b) Disconnect the load before testing c) Avoid grounding the test leads d) Only measure with the instrument in manual mode - How should the LCR meter be connected to the component under test? a) In series with the load b) Across the power supply c) Across the component terminals d) Parallel to the ground - The rotating disc of energy meter is usually made of: a)Copper b)Aluminium c)Iron d)Plastic Answer Keys: Formative Assessment (Part C). 1; b; 16; d. 2; b; 17; b. 3; a; 18; a. 4; b; 19; b. 5; b; 20; d. 6; c; 21; b. 7; b; 22; d. 8; b; 23; b. 9; a; 24; c. 10; b; 25; c. 11; b; 26; c. 12; a; 27; b. 13; a; 28; b. 14; c; 29; c. 15; b; 30; b. |

- Limited Response Questions

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) |
| --- | --- | --- |
| LU (1 to 9) | LU (10 to 16) | LU (15) |
| - How can absolute and secondary measuring instruments be differentiated? - List the steps to record values using a tangent galvanometer. - What are the key functions of a multimeter that must be identified before use? - How is the desired function selected on a multimeter? - Enlist types of tachometer - Why is calibration important before using any measuring instrument like an ammeter or voltmeter? - How do you connect a voltmeter correctly to measure voltage? - What are the units measured by an ohm meter, and how are they recorded? - How AC voltage is measured with CRO - Write the names of types of lux meter - What safety precautions should be taken while operating measuring instruments? - Define Indicating Instruments? - What is the diference between analog and digital instruments - Write properties of spring materials - Define critical damping - Write uses of multimeter - What is parallax error - Write two characteristics of secondary instruments - How would you recognanize a recording instrument - Enlist types of damping methorts - What is lux meter - Define sampling oscillator - Write at least 2 uses of CRO - Write three uses of tangent galvanometer - Define integrating instruments | - What is the primary purpose of a Megger in electrical measurements? - What is the procedure to measure earth resistance using a Megger. - What precautions should be taken when connecting test leads for earth resistance measurement? - How does a Power Factor Meter measure the power factor of a load? - What is the role of balancing in using a Wheatstone Bridge to measure medium resistance? - Enlist the steps involved in accurately balancing a Wheatstone Bridge. - Write a name of External Part of a Megger? - Write the Possible Errors of Wheatstone Bridge? - Write name of parts of earth tester - Define power factor meter - Write uses of megger - Define continuity test - How many types kinds of frequency meter used in power circuits - Write the working principle of wheatstone bridge - Define spark | - How do you determine whether to use a single-phase or three-phase energy meter for a specific load? - How is a single-phase energy meter connected?. - How are the three phases (L1, L2, L3) and neutral connected to a three-phase energy meter? - What are the key steps in attaching a load to an energy meter? - Why is it important to inspect wiring for correct polarity before powering on the system? - What indicators verify that an energy meter is measuring consumption accurately after installation? - What types of loads can be connected to the output terminals of an energy meter? - What safety guidelines should be followed during the installation of an energy meter? - What are the common issues that might arise during energy meter installation and how can they be resolved? - Write Errors of Energy Meter? - How do you select the correct function and range on an LCR meter for measuring inductance? - What parameters can be measured using an LCR meter and what units are they recorded in? - Define creeping error in energy meter - Enlist the parts of single phase energy meter - State which effect of current is used in single phase energy meter - Write four errors occurring in energy meter - What is the working principle of energy meter |

- Assessment Tasks

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) |
| --- | --- | --- |
| LU (1 to 9) | LU (10 to 16) | LU (15) |
| - Carry out measurment of the following parameters with the help of multimeter, techometer and lux meter. - Voltage - Ampere - Resistance - RPM - Ilumination Performance Criteria: - Follow all safety precautions to avoid electrical hazards during measurements. - Select the appropriate measuring instrument (multimeter, tachometer, or lux meter) for each parameter. - Ensure the instrument is in good working condition and properly calibrated. - Set the multimeter to the correct function and range for measuring voltage, current, or resistance. - Connect the multimeter probes securely to the circuit/component for accurate readings. - Use the tachometer to measure RPM by aligning it with the rotating object as per the manufacturer's instructions. - Measure illumination using the lux meter by positioning it at the required location and angle. - Record readings accurately and compare them with standard or expected values. - Document the measured parameters systematically for analysis and reporting. - Connect Oscilloscope across the Cisrcuit to veify Frequency and Voltage Performance Criteria: - Select a suitable oscilloscope for the circuit's frequency and voltage range. - Ensure the oscilloscope is properly calibrated before use. - Connect the oscilloscope probes to the circuit securely, matching the polarity (ground and signal). - Set the oscilloscope to the appropriate mode and range for frequency and voltage measurement. - Adjust time base and voltage settings to obtain a clear waveform display. - Verify the frequency and voltage values displayed on the oscilloscope against expected values. - Observe the waveform for stability, amplitude, and frequency characteristics. - Follow all safety precautions to prevent damage to the oscilloscope and circuit. - Record the measured data systematically for analysis and documentation. | Measurethe earth resistance value with the help of megger, power factor value with the help of power factor meter,medium resistance value with the help of wheatstone bridge Performance Criteria: - Select appropriate instruments for each measurement (Megger, Power Factor Meter, Wheatstone Bridge). - Ensure all instruments are calibrated and in proper working condition. - Use the Megger to measure earth resistance by connecting the test leads securely and following standard procedures. - Measure the power factor using the Power Factor Meter, ensuring correct connections and load operation. - Utilize the Wheatstone Bridge to measure medium resistance, balancing the bridge accurately for precise readings. - Record readings systematically for each parameter and verify them against expected values or standards. - Follow all safety precautions during testing, especially for live electrical components. - Document the test results clearly for analysis and future reference. | a)Connect Single Phase and 3 Pahse Energy Meter with Load Performance Criteria: - Select the appropriate energy meter (single phase or three phase) based on the required electrical load. - Ensure the energy meter is functional and has been calibrated properly before installation. - For a single phase system, connect the Phase (P) and neutral (N) wires to the designated terminals on the meter. - In a three phase system, connect all three phases (L1, L2, L3) to the respective terminals of the meter, along with the neutral (N). - Attach the load (such as appliances or machinery) to the output terminals of the energy meter, ensuring all connections are secure and correct. - Inspect the wiring to confirm correct polarity and ensure everything is safely connected before powering on the load. - Turn on the system and monitor the energy meter to verify it is measuring the consumption accurately. - Adhere to all safety guidelines to prevent electrical accidents during installation and testing. - Record the setup, including load specifications and meter readings, for documentation and future reference. b)Measure inductance, capicitance and resistance with the help of LCR meter Performance Criteria: - Ensure the LCR meter is properly calibrated and powered on. - Select the appropriate mode (L, C, or R) based on the parameter to be measured. - Connect the test leads securely to the LCR meter. - Attach the leads to the component under test, ensuring proper contact. - Choose the correct measurement frequency or test signal if adjustable. - Set the appropriate range or use auto-range for optimal accuracy. - Verify the LCR meter’s accuracy specifications to ensure measurements meet required tolerances. - Avoid external interferences, such as stray inductance or capacitance. - Conduct measurements in a stable environment, free from temperature fluctuations or electromagnetic noise. - Read and record the values displayed on the LCR meter. - Compare with expected values or datasheet specifications. - Cross-check measurements with a reference standard, if required, to confirm precision. - Handle the components carefully to prevent damage. - Ensure the LCR meter is disconnected from any live circuit during measurements. - Use the LCR meter to measure inductance, capacitance, and resistance, selecting the correct function and range. |