Course / Subject Name: NQ-ET 282 Digital Logic Design

- MCQs

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) |
| --- | --- | --- |
| LU (1-12) | LU (13-24) | LU (25-31) |
| - When the input to an inverter is Low (0), the output is - High or 0 - Low or 0 - HIGH or 1 - LOW or 1 - An inverter performs an operation known as - Complementation - Assertion - Inversion - both a and c - Logical expression for NOT gate is - A=0 - A=1 - A=A - A=A’ - The output of an AND gate with inputs A, B and C is 0 (LOW) when - A = 0, B = 0, C = 0 - A = 0, B = 1, C = 1 - both a and b - None - Logical expression for AND gate is - X=A+B - X=A.B - X=AB - None of these - The output of an OR gate with inputs A, B and C is 0 (LOW) when - A = 0, B = 0, C = 0 - A = 0, B = 1, C = 1 - both a and b - None - Logical expression for OR gate is - X=A+B - X=A.B - X=AB - None of these - Logical expression for NAND gate is - X = A.B - X = A+B - X = AB - Both a and c - Logical expression for NOR gate is - X = A.B - X = A+B - X = AB - Both a and c - Logical expression for exclusive OR gate is - X = A⨁B - X = A+B - X = AB - X = A⨁ B - Logical expression for exclusive NOR gate is - X = A⨁B - X = A+B - X = AB - X = A⨁ B - TTL stands for - Two Tables Logic - Two Transistor Logic - Transistor to Transistor Logic - None of these - A logic circuit which can add 2 bits is called - Half Adder - Full Adder - Half Subtractor - Full Subtractor - A half adder consists of - 2 inputs & 1 output - 2 inputs & 2 outputs - 3 inputs & 2 outputs - 3 inputs & 3 outputs - This adder cannot handle the input carry - Half Adder - Full Adder - Half Subtractor - Full Subtractor - The gate which performs the function of Sum in Half adder is - AND gate - OR Gate - X-OR Gate - NAND Gate - The gate which performs the function of CO in Half adder - AND gate - OR Gate - X-OR Gate - NAND Gate - Boolean expression for CO in Half Adder is - A + B - A - B - A . B - A + B - A logic circuit which can add 3 bits at a time is called - Half Adder - Full Adder - Half Subtractor - Full Subtractor - A Full adder consists of - 2 inputs & 1 output - 2 inputs & 2 outputs - 3 inputs & 2 outputs - 3 inputs & 3 outputs - This adder can handle the input carry - Half Adder - Full Adder - Half Subtractor - Full Subtractor - If all the inputs of Full Adder are HIGH (1), then its outputs are - Sum = 0, Co = 1 - Sum = 1, Co = 1 - Sum = 0, Co = 0 - Sum = 1, Co = 0 - A logic circuit used to subtract the 2 bits is called - Half Adder - Full Adder - Half Subtractor - Full Subtractor - A half subtractor consists of - 2 inputs & 1 output - 2 inputs & 2 outputs - 3 inputs & 2 outputs - 3 inputs & 3 outputs - The gate which performs the function of Di in Half subtractor - AND gate - OR Gate - X-OR Gate - NAND Gate - Boolean expression of Borrow out (Bo) in half subtractor is - A . B - A.B - A + B - A+B - A logic circuit used to subtract the 3 bits is called - Half Adder - Full Adder - Half Subtractor - Full Subtractor - A Full subtractor consists of - 2 inputs & 1 output - 2 inputs & 2 outputs - 3 inputs & 2 outputs - 3 inputs & 3 outputs - If all the inputs of Full Subtractor are HIGH (1), then its outputs are - Di = 0, Bo = 1 - Di = 1, Bo = 1 - Di = 0, Bo = 0 - Di = 1, Bo = 0 - The number of full adders in a 4-Bit parallel adder will be - Two - Three - Four - Eight - Full adder consists of - Two half adder - Two half adder & AND gate - Two half adder & OR gate - Two NAND gates Answers Keys: - c; - d; - d; - c; - b. - a; - a; - d; - b; - d. - a; - c; - a; - b; - a. - c; - a; - c; - b; - c. - b; - b; - c; - b; - c. - a; - d; - c; - b; - c. - c. | - Multiplexer is also called - Data Distributer - Data Selector - Decoder - Encoder - Multiplexer has - Many inputs & Many Outputs - One input & Many outputs - Many inputs & one outputs - one input & one output - The number of select lines in 8 to 1 line multiplexer are - Two - Three - Four - Five - De-Mux means - One into one - One into many - Many into one - Many into many - De-Mux is also called - Data Distributer - Data Selector - Decoder - Encoder - A logic circuit which converts binary into decimal - Multiplexer - Demultiplexer - Decoder - Encoder - 7-Segment display consists of - 6 LEDs - 7 LEDs - 8 LEDs - 9 LEDS - A single 7-Segment display displays the decimal digits from - 0 to 7 - 0 to 9 - 0 to 10 - 0 to 15 - Which IC is used to drive the 7-Segment display? - 7446 - 7447 - Both a & b - None of these - A logic circuit which converts decimal into binary: - Encoder - Decoder - Multiplexer - Demultiplexer - 74LS147 is an encoder - Octal to Binary - Binary to decimal - Decimal to BCD - BCD to decimal - Output of encoder is - Decimal - Binary - Octal - All of these - Input of decoder is - Decimal - Binary - Octal - All of these - When all seven segments of a display are energized, the number shown will be - 8 - 6 - 9 - 10 - Seven segment displays in which all the anodes of LEDs are common is called - Common Anode - Common Cathode - Both a & b - None of these - A digital element which can store one bit of binary data at a time - Multiplexer - Flip Flop - Encoder - Clock - It is the simplest type of flip flop - RS flip flop - D flip flop - JK flip flop - T flip flop - A basic RS latch consists of this gate - NAND Gate - NOR Gate - a or b - OR Gate - Clocked RS flip flop has - Two inputs & two outputs - Two inputs & one output - Three inputs & Two outputs - None of these - The number of Data input of D-flip flop is - 1 - 2 - 3 - 4 - This condition is not happened in D - Flip flop - Set - Reset - Race - None of these - When a digital circuit triggers on the level of clock pulse, it is called - Level triggering - Edge triggering - Clearing - Loading - Toggle means - Set - Reset - Change to opposite state - No change - T flip flop can be implemented by using this flip flop - RS flip flop - Clocked RS flip flop - JK flip flop - All of these - JK flip flop is a circuit - Single feedback - Double feedback - Without feedback - None of these - The 555 Timer IC got its name from the three _______ resistors that are used in its voltage divider network - 5Ω - 5KΩ - 5MΩ - 5TΩ - Astable multivibrator is - A square wave clock - Equivalent to a flip flop - A one shot multivibrator - None of these - Most popular IC used in timing circuits - 555 - 741 - 7400 - All of these - A mono stable multivibrator is also referred as - One shot - Two shot - Three shot - Four shot Answers Keys: - b; - c; - b; - b; - a. - c; - b; - b; - c; - a. - c; - b; - b; - a; - a. - b; - a; - c; - c; - a. - c; - a; - c; - c; - b. - b; - c; - a; - a. | - A shift register has the characteristics of - Memory - Shift only - Both memory and shift - None of these - Shift register is constructed by combining - Diodes - Transistors - Flip Flops - SCRs - The number of flip-flops in 4- bit register are - 8 - 6 - 4 - 16 - A shift register can accept data bits in - Serial fashion - Parallel fashion - Both a and b - None of these - SISO data shifting process is - Slow - Fast - Very fast - None of these - The shifting of one byte data through serial shift register requires - One clock pulse - One load pulse - Eight clock pulses - One clock pulse for each 1 in the data. - The full form of SIPO is - Parallel-in Serial-out - Serial-in Serial-out - Serial-In Peripheral-Out - Serial-in Parallel-out - In a parallel in/parallel out shift register, D0 = 1, D1 = 1, D2 = 1, and D3 = 0. After one clock pulse, the data outputs are - 1110 - 0001 - 1100 - 1000 - Counters are used as instrument for measuring - Time - Frequency - Speed - a and b - For a four bit counter how many FFs are needed? - 2 - 3 - 4 - 5 - Asynchronous counters are known as - Decade counters - Ripple counters - Modules counters - Ring Counters - A synchronous counter differs from a asynchronous counter in - No of states in its count - Method of clocking - Type of ffs used - Modulus number - Maximum count value of a n bit counter - 2n- 1 - 2n - 22n - 2n+ 1 - Which technology are using in synchronous and Asynchronous counter ICs - TTL - CMOS - Both - not of them - How many different state does a 3-bit asynchronous counter have - 2 - 4 - 8 - 16 - Value of a digital quantity has - two states - three states - one state - all of these - A circuit that convert analog quantity to digital quantity is called - A/D convertor - D/A convertor - Oscillator - a and b - A circuit that convert digital quantity to analog quantity is called - A/D convertor - D/A convertor - Oscillator - a and b - Temperature is a quantity - Digital - Analog - Universal - none of these - Digital numbers are also called - Decimal - Hexadecimal numbers - Octal numbers - Binary numbers - The information exist in real world are - Digital - Analog - Universal - none of these - Basic types of DAC are - two - three - four - all are correct - Continuous time varying signal is also called - Analog signal - Digital signal - Accurate signal - None of these - The analog input of most ADCs is - 10 V - 15 V - 20 V - 5 V Answers Keys: - c; - c; - c; - c; - a. - c; - d; - a; - d; - c. - b; - b; - a; - c; - c. - a; - a; - b; - b; - d. - b; - a; - a; - d. |

- Limited Response Questions

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) |
| --- | --- | --- |
| LU (1-12) | LU (13-24) | LU (25-31) |
| - Define NOT gate. - Define OR gate. - Define AND gate. - Define NAND gate. - Define NOR gate. - Define Ex-OR gate. - Define Ex-NOR gate. - Differentiate between NAND and AND gate. - Differentiate between NOR and OR gate. - Define Half adder. - Make truth table for Half adder. - Draw logic circuit of half adder. - State Boolean expression for Half Adder. - Which types of logic gates are used in Half adder? - Define Full Adder. - Draw block diagram of Full adder. - Make truth table for Full Adder. - Draw logic circuit of Full Adder. - State Boolean expressions for Full Adder. - Define Half Subtractor. - Draw block diagram and logic circuit of Half Subtractor. - Make truth table for Half Subtractor. - Define Full Subtractor. - Draw logic circuit of Full Subtractor. | - Define Multiplexer? - Define 4 to 1 line Mux. - Define 8 to 1 line multiplexer. - Define Demultiplexer. - Define decoder. - Define BCD to decimal decoder. - Define 7-Segment display. - Define common anode seven segment display. - Define common cathode seven segment display. - Define BCD to 7-segment decoder. - Define Encoder. - Define decimal to BCD encoder. - Define sequential circuit. - What is the difference between latch and flip flop? - Define triggering. - Enlist types of triggering. - Define level triggering. - Define edge triggering. - Enlist types of edge triggering. - Define RS flip flop. - What is Race condition? - Define clocked RS flip flop. - Define D-Flip flop. - Draw truth table for D flip flop. - Define Edge triggered D flip flop. - Define JK flip flop. - Why JK flip flop is called “Universal flip flop”? - Define Toggle condition of JK flip flop. - Define No change condition of JK flip flop. - Define T – flip flop and draw its diagram. - Define 555 timer. - List the operating modes of 555 timer IC. - Define astable multivibrator. - Define monostable multivibrator. - Define bistable multivibrator. | - What is shift register? - Define SISO. - Define SIPO? - What is PISO? - Define PIPO. - Which type of shift register is slow and why? - List types of shift registers. - Why D – Flip Flop is used in shift register? - What is bi directional shift register? - What is counter? - Differentiate between synchronous and asynchronous counter. - Why synchronous counter is faster than asynchronous. - What are the applications of asynchronous Counters? - Draw block diagram of synchronous counter. - What are the advantages of synchronous counter? - How many flip-flops are in 3 bit asynchronous counter? - Define analog quantity. - Define digital quantity. - Define DAC. - Define ADC. - What is OP Amp? - List two disadvantages of Binary weighted Resistor D/A convertor. - List two advantages of Binary weighted Resistor D/A convertor. - List the types of Analog to digital convertor. |

- Assessment Tasks

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) |
| --- | --- | --- |
| LU (1-12) | LU (13-24) | LU (25-31) |
| Construct Full Adder and Full Subtractor circuit and also verify truth table of both the logic circuits Performance Critaria: - Draw block Diagram of both Full Adder and Full Subtractor logic circuits. - Draw truth table of both Full Adder and Full Subtractor logic circuits. - Write Boolean Expression of Full Adder and Full Subtractor logic circuit. - Draw logic circuits of both Full Adder and Full Subtractor logic circuits. - Place Full Adder and Full Subtractor logic gate ICs on Breadboard. - Identify input, output, Vcc and ground pin for the both Full Adder and Full Subtractor logic gate ICs. - Connect LEDs to the output pins of both Full Adder and Full Subtractor logic gate ICs and apply different logics at input pins. - Record & verify output result against each given input for both Full Adder and Full Subtractor logic circuits. | Configure 555 IC as A-stable, Mono-stable and Bi-stable Multi-vibrators Performance Critaria: - Draw circuit diagram for A-stable, Mono-stable and Bi-stable Multi-Vibrators. - Place 555 timer IC on bread board/trainer for A-stable, Mono-stable and Bi-stable Multi-Vibrators. - Make connection as per diagram for A-stable, Mono-stable and Bi-stable Multi-Vibrators. - Apply voltage to circuits of A-stable, Mono-stable and Bi-stable Multi-Vibrators. - Verify output signal wave form of . the A-stable, Mono-stable and Bi-stable Multi-Vibrator’s with the help of oscilloscope | Construct SISO, PIPO, SIPO and PISO 4-bit shift registers using D-flip flop Performance Critaria: - Determine number of Stages for SISO, PIPO, SIPO and PISO 4-bit shift registers. - Select type of flip flop for SISO, PIPO, SIPO and PISO 4-bit shift registers. - Draw schematic Diagram of SISO, PIPO, SIPO and PISO 4-bit shift registers. - Make connection of D-Flip Flop as per diagram for SISO, PIPO, SIPO and PISO 4-bit shift registers. - Connect data inputs (D0, D1, D2, D3) to the D inputs of the flip-flops (FF0, FF1, FF2, FF3) respectively for SISO, PIPO, SIPO and PISO 4-bit shift registers. - Apply data at the input of SISO, PIPO, SIPO and PISO 4-bit shift registers and give clock pulse - Record output according to the input of SISO, PIPO, SIPO and PISO 4-bit shift registers - Draw output Truth table for SISO, PIPO, SIPO and PISO 4-bit shift registers |