Course / Subject Name: NQ-ET 326 AC Machines

- MCQs

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) |
| --- | --- | --- |
| LU (1 to 2) | LU (3 to 5) | LU (6 to 7) |
| - What is the primary function of a transformer? - To convert AC to DC - To step up or step down voltage - To increase current - To store electrical energy - The efficiency of a transformer is maximum when - It is operating at full load - It is operating at no load - It is operating at half load - It is under heavy load - In a single-phase transformer, what happens if the primary winding is open-circuited? - The transformer will continue to work - No current will flow in the secondary winding - The transformer will burn out - The secondary voltage will increase - What is the primary purpose of the laminated core in a transformer? - To increase the efficiency - To reduce eddy current losses - To increase the voltage - To improve insulation - In a three-phase transformer, the total power is the sum of - Powers from all three phases - The maximum power from one phase - The voltage and current of one phase - The average power in one phase - In a three-phase transformer, if one phase is disconnected, what will happen? - The transformer will continue to operate normally - The transformer will overheat - The transformer will lose its power output completely - The transformer will provide reduced power - In a three-phase transformer, if the windings are connected in a star configuration, what is the relationship between line and phase voltages? - Line voltage equals phase voltage - Line voltage is √3 times phase voltage - Phase voltage is √3 times line voltage - Phase voltage is equal to half of line voltage - The three-phase transformer with a delta-star connection is used to - Increase the voltage - Reduce harmonics - Provide a neutral point in the secondary - Decrease the power factor - In a three-phase transformer, what is the frequency of the induced EMF in the secondary windings? - Same as primary winding frequency - Double the primary frequency - Half the primary frequency - Zero - What is the effect of harmonics on a three-phase transformer? - It decreases the efficiency - It increases the voltage stability - It reduces the core losses - It improves the transformer performance - In a three-phase transformer bank, the most commonly used (domestic) connection for the secondary side is - Delta - Star - Zigzag - Open Delta - A delta-delta connected transformer is used to supply - Low voltage loads - High voltage loads - Power factor correction - Three-phase balanced loads - In a three-phase transformer, the neutral point is used for: - Connecting phase-to-phase - Providing a reference point for phase voltages - Reducing the overall voltage - Balancing the transformer windings - What is the power factor of a balanced load in a three-phase transformer? - 0 - 1 - 0.5 - It depends on the load type. - Which of the following is NOT a characteristic of a three-phase transformer? - More efficient than single-phase transformers - Requires only one set of windings - Can supply three-phase loads - Provides a higher power output Answer Keys: 1. b; 2. a; 3. b. 4. b; 5. a; 6. d. 7. b; 8. c; 9. a. 10. a; 11. b; 12. d. 13. b; 14. b; 15. b. | - Which of the following is true for a single-phase induction motor? - It runs only on a single-phase supply. - It cannot start by itself. - It is self-starting. - It has two windings. - What is the function of a capacitor in a capacitor-start single-phase induction motor? - To reduce the starting current - To create a phase shift for starting torque - To increase the efficiency - To provide continuous speed regulation - What type of motor is commonly used in household fans? - Universal motor - Split-phase induction motor - Squirrel cage induction motor - Shaded pole motor - In a single-phase motor, the direction of rotation can be reversed by: - Reversing the direction of the supply - Changing the polarity of the capacitor - Reversing the start winding connections - Reversing the rotor connections - A split-phase induction motor requires - Two windings with a capacitor - A shaded pole winding - A capacitor and a start winding - Two windings of equal impedance - In a three-phase induction motor, the rotor speed is - The same as the synchronous speed - Higher than the synchronous speed - Lower than the synchronous speed - Constant at all loads - The slip of a three-phase induction motor is defined as - The ratio of synchronous speed to rotor speed - The difference between synchronous speed and rotor speed - The difference between line and phase voltages - The ratio of rotor resistance to stator resistance - The synchronous speed of a three-phase motor depends on - The rotor resistance - The number of poles and supply frequency - The rotor speed - The current flowing through the motor - In a three-phase induction motor, the rotor current is induced by - The stator magnetic field - The rotor itself - A separate excitation system - External capacitors - Which of the following methods is used to reduce the starting current of a three-phase induction motor? - Star-delta starter - Three point starter - Four point starter - DOL starter - Which of the following is the most common type of single-phase induction motor? - Squirrel cage motor - Wound rotor motor - Permanent split capacitor motor - Synchronous motor - Which of the following factors does not affect the speed of a three-phase induction motor? - Supply voltage - Number of poles - Frequency of supply - Slip - The power factor of an induction motor is - Always unity - Always less than unity - Equal to the ratio of mechanical power to electrical power - Constant across all loads - The efficiency of a three-phase induction motor is maximum when - Slip is zero - Slip is at its maximum - The motor runs at synchronous speed - The motor operates at the rated load - In a three-phase induction motor, the slip at no-load condition is - Zero - Very high - Low - Equal to the full-load slip Answer Keys: 1. b; 2. b; 3. d. 4. c; 5. d; 6. c. 7. b; 8. b; 9. a. 10. a; 11. a; 12. a. 13. b; 14. d; 15. c. | - Which of the following is true about a synchronous motor? - It always runs at synchronous speed. - It operates at variable speed. - It can never operate at synchronous speed. - It has a slip like an induction motor. - The synchronous speed of a 4-pole motor operating on a 50 Hz supply is - 1000 rpm - 1500 rpm - 2000 rpm - 3000 rpm - In a synchronous motor, the rotor rotates at - A speed that is less than synchronous speed. - Synchronous speed. - A speed that is greater than synchronous speed. - Zero speed. - To start a synchronous motor, which method is commonly used? - Direct-on-line start - Autotransformer start - Starting by an induction motor - Star-delta start - What is the primary function of an alternator? - To convert electrical energy to mechanical energy - To convert mechanical energy to electrical energy - To regulate voltage - To store electrical energy - In an alternator, the induced EMF is generated due to - Change in magnetic flux - Change in electric field - Change in speed - Rotation of the rotor - In a 3-phase alternator, the phases are separated by - 90 degrees - 120 degrees - 180 degrees - 360 degrees - Which of the following is true for an alternator operating in parallel with another? - Both must have the same voltage and frequency. - Only the voltage needs to match. - Only the frequency needs to match. - There is no need to match either voltage or frequency. - Which of the following methods is used to control the output voltage of an alternator? - Varying the rotor speed - Varying the stator resistance - Varying the field excitation - Changing the load on the alternator - The efficiency of a synchronous motor is primarily influenced by - Field excitation - Stator resistance - Power factor - Rotor speed - Synchronous motors are typically used in applications requiring - High starting torque - Constant speed operation - Low torque - High efficiency at varying speeds - The synchronous speed of an alternator depends on - The rotor current - The supply frequency and the number of poles - The load on the alternator - The voltage - In a salient pole alternator, the poles are - Cylindrical in shape - Projecting and large - Flat and small - Non-salient - The alternator‘s output voltage can be controlled by adjusting: - Stator voltage - Rotor speed - Field excitation - Load current - Which of the following is used to measure the speed of a synchronous motor or alternator? - Tachometer - Ammeter - Voltmeter - Wattmeter Answer Keys: 1. a; 2. b; 3. b. 4. c; 5. b; 6. a. 7. b; 8. a; 9. c. 10. c; 11. b; 12. b. 13. b; 14. c; 15. a. |

- Limited Response Questions

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) |
| --- | --- | --- |
| LU (1 to 2) | LU (3 to 5) | LU (6 to 7) |
| Q1. What is the primary function of single-phase transformer? Q2. Define copper losses in single-phase transformer? Q3. What is the role of tap changer in single-phase transformer? Q4. Write the formula for transformation ratio in single-phase transformer? Q5. How many primary and secondary windings does a three-phase transformer have? Q6. In how many configurations three phase transformer can be connected? Q7.What is the main advantage of three-phase transformer over single-phase transformer? Q8. Why transformer is rated in KVA and not in KW? Q9. Define delta (Δ) connection in three-phase transformer? Q10. What will be the effect of load on the voltage in single-phase transformer? | Q1. What is the role of auxiallary windings? Q2. What is function of centrifugal switch in single phase motor? Q3. What is the typical application of single-phase induction motor? Q4. Is single phase motor self starting? Q5. How to change direction of rotation of three phase induction motor. Q6. Define "star-delta" starter for three-phase induction motor? Q7. Which gas is used for cooling of induction motor? Q8. Define "squirrel-cage" rotor in three-phase induction motor? Q9. Define slip in three phase induction motor? Q10. Enlist power stages in induction motor. | Q1. Write down the formula for synchronous speed of alternator? Q2. Define synchronous generator? Q3. What happens if synchronous motor is not in synchronism? Q4. Define the function of damper windings in synchronous motor? Q5. What is the condition in which synchrnous motor power factor is leading? Q6. Deferentiate between salient and non-salient pole rotors? Q7. Define exciation of rotor? Q8. How is the output voltage of synchronous generator controlled? Q9. Differentiate between synchronous and induction motors? Q10.Define “synchronous reactance” in synchronous motor? |

- Assessment Tasks

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) |
| --- | --- | --- |
| LU (1 to 2) | LU (3 to 5) | LU (6 to 7) |
| Connect three phase transformer in star- star configuration and measure phase voltages. Performance Criteria: - Draw the circuit diagram of three phase star-star transformer. - Make connections according to the circuit diagram. - Check connections for safety measures. - Verify the expected phase voltage values. - Verify correct connection of neutral. - Measure the voltage between each phase and neutral. - Verify all voltages are nearly the same. - Connect balance load across all phases. - Record the measured values. - Check that phases are in the correct order. - Disconnect transformer from the supply. | Demonstrate operation of a Direct-On-Line (DOL) three phase induction motor starter. Performance Criteria: - Draw circuit diagram of the DOL starter. - Make connections of control circuit. - Make connections of Power circuit. - Check connections for safety measures. - Start the motor. - Measure Line Voltages of induction motor. - Calculate phase voltages of induction motor. - Stop the Motor. - Disconnect all the connections from motor. | Connect alternator with three phase unity power facotor load and measure output power. Performance Criteria: - Draw circuit diagram of alternator with three phase star connected load. - Connect dc supply to the rotor. - Connect the load to alternator. - Connect voltmeter to measure the load voltage. - Connect ammeter to measure the load current. - Check connections for safety measures. - Turn on the rotor dc supply. - Monitor and adjust excitation - Measure phase voltage and current of the load. - Using three phase power formula to find output power supplied to the load. - Disconnect all the connections. |