Course / Subject Name: NQ-ET-345 Transmission, Distribution & Protection of Electrical Power Systems

- MCQs

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) |
| --- | --- | --- |
| LU (1 to 5) | LU (6 to 8) | LU (9 to 11) |
| - The main benefit of improving the power factor in industries? a) Reduces power loss and demand charges b) Increases equipment lifespan c) Improves voltage regulation d) All of the above - Which of the following is a major cause of low power factor. - Excessive use of resistive loads - b) Operation of synchronous motor - Excessive use of Inductive loads. - High-efficiency lighting - Disadvantages of Low Power Factor a) Decreased current in the system b) Higher energy efficiency c) Increased power losses in cables d) Reduced need for power factor correction devices - Low power factor increases operating costs because: a) It reduces load on the transformer b) It causes higher line losses and requires larger equipment c) It minimizes the energy billing charges d) It reduces harmonics in the system - In industrial tariff systems, low power factor is penalized because it: a) Reduces overall power demand b) Requires additional generation capacity c) Improves the voltage profile of the system d) Encourages the use of resistive loads - Which voltage level is typically classified as Extra High Voltage (EHV)? - Below 1 kV - 1–33 kV - 33–220 kV - Above 220 kV - Medium voltage transmission lines usually operate in the range of: - 1–33 kV - 33–132 kV - 132–220 kV - Above 220 kV - Underground transmission systems are preferred over overhead lines in: - Remote rural areas - Urban and densely populated regions - Coastal areas with high salt content in the air - Low-load industrial areas - One major advantage of DC transmission systems over AC systems is: - Lower initial cost - Simpler voltage transformation - Reduced transmission losses - Better suitability for short distances - Which of the following is an advantage of AC transmission over DC transmission? - Easier voltage step-up and step-down - Lower skin effect - Lesser requirement for insulation - More economical for long-distance transmission - What are the primary transmission line constants? - Resistance, Inductance, Capacitance, and Susceptance - Voltage, Current, Power, and Frequency - Reactance, Impedance, Voltage, and Power Factor - Conduction, Insulation, Frequency, and Reactance - Which transmission line constant causes power losses? - Inductance - Capacitance - Resistance - Susceptance - What is the main effect of inductance in a transmission line? - Causes capacitive charging currents - Reduces the line voltage drop - Generates a lagging reactive power - Minimizes power losses - Which transmission line constant causes voltage rise at the receiving end during light/No load conditions? - Resistance - Inductance - Capacitance - Susceptance - The voltage drop across a transmission line depends on: - Only the load current - Load current and the impedance of the line - Load voltage and the capacitance of the line - Only the resistance of the line - What is the primary purpose of strain insulators? a) To support conductors on horizontal sections of the line b) To provide insulation at tension points or sharp bends c) To reduce vibration on the conductor d) To protect insulators from lightning strikes - Which conductor material is widely used for overhead lines due to its high strength-to-weight ratio? a) Copper b) Aluminium c) ACSR (Aluminium Conductor Steel Reinforced) d) Steel - Which type of line support is best suited for high-voltage transmission lines? a) Wooden poles b) Concrete poles c) Steel lattice towers d) Guyed poles - What is the primary function of vibration dampers in overhead lines? a) To increase the current-carrying capacity of conductors b) To reduce conductor sag c) To minimize conductor vibrations caused by wind d) To enhance conductor insulation - Line sag is primarily influenced by: a) The diameter of the conductor b) The weight of the conductor and span length c) The color of the conductor d) The type of insulator used - Which of the following materials is commonly used as the conductor in underground power cables? - Copper - Steel - Aluminum - Lead - What is the function of the outer sheath in underground power cables? - To reduce electrical resistance - To protect the insulation from mechanical damage and environmental factors - To conduct the electricity - To provide flexibility to the cable. Answer Keys: 1. d; 2. c; 3. c; 4. b; 5. b. 6. d; 7. a; 8. b; 9. c; 10. a. 11. a; 12. c; 13. c; 14. c; 15. b. 16. b; 17. c; 18. c; 19. c; 20. b. 21. a; 22. b. | - What does HVDC stand for in power transmission? a) High Voltage Direct Circuit b) High Voltage Direct Current c) High Voltage Distribution Current d) High Voltage Diode Current - Which type of HVDC transmission line is commonly used for connecting distant offshore wind farms? a) Overhead lines b) Submarine cables c) Underground cables d) Hybrid cables - What is the primary function of a three-phase bridge converter in an HVDC system? a) To convert AC to DC and DC to AC b) To regulate voltage levels c) To filter harmonics d) To store electrical energy - What is the primary limitation of HVDC systems for smaller power networks? - High power losses - High cost of converters and limited scalability - Lack of insulation requirements - Inefficiency in power transfer - In a DC distributor, voltage drop depends on: - Resistance and length of the line - Current and distance of the load from the feeder - Voltage of the system - Both A and B - In an AC distributor, power factor affects: - Resistance only - Current only - Both voltage drop and current - Neither voltage drop nor current - Which type of distributor is preferred for a uniformly distributed load in DC systems? - Radial distributor - Ring main distributor - Interconnected distributor - Parallel distributor - In an AC distributor, the voltage drop along the line is affected by: - Resistance only - Resistance and reactance - Load power factor - Both B and C - The impedance of an AC line is calculated as: - Z = R + XL - Z = √(R² + (XL)²) - Z = R - XL - Z = R × XL - In a balanced three-phase AC distributor, the voltage drop per phase is: - The same for all phases - Different for each phase - Zero for the neutral phase - None of the above - regulation at remote ends - In a DC distributor fed from both ends, the point of minimum potential is: - At the midpoint of the line - B) Near the end with the larger load - C) At the load center - D) Near the end with the smaller load - What is the primary purpose of a booster in a DC distribution system? - To increase line voltage - To balance load currents - To reduce line resistance - To provide reactive power compensation - AC balancers are primarily used for: - Voltage regulation - Reducing harmonic distortion - Balancing unbalanced loads - Frequency stabilization - Which type of booster is commonly used in AC transmission lines to control voltage? - Static booster - Synchronous booster - Series booster - Shunt booster - A DC balancer set is mainly composed of: - Two motors connected in series - B) Two generators connected in parallel - C) A motor and a generator coupled mechanically - D) A transformer and a rectifier - Mechanically - What is the function of a shunt booster in a power system? - Compensating for reactive power - Increasing current flow in the main line - Supporting voltage at the far end of the line - Reducing system harmonics Answer Keys: 1. b; 2. b; 3. a; 4. b; 5. d. 6. c; 7. b; 8.d; 9. b; 10. a. 11.c; 12.b; 13. d; 14. b; 15. a. 16. c. | - Which type of substation is used to increase voltage for long-distance transmission? - Distribution substation - Transmission substation - Collector substation - Switching substation - What is the purpose of grounding in substations? - To enhance power generation - To prevent electrical shocks and protect equipment from surges - To improve voltage regulation - To reduce transmission losses - In a substation, the 'busbar' is used for: - Power generation - Voltage regulation - Interconnecting incoming and outgoing circuits - Protection against faults - Which of the following is a common grounding method used in substations? - Metal-to-metal grounding - inductive grounding - solid grounding - Capacitor grounding - In a substation, the 'isolator' is used to: - Interrupt the flow of current during fault conditions - Protect transformers from overload - Isolate the circuit for maintenance/Repair purposes - Measure power consumption - What is the primary unction of a protective system in electrical power system. - To increase the voltage - To monitor the power quality - To detect faults and isolate faulted sections - To control the frequency of the system - What is the purpose of protective zones in power systems? - To reduce the power loss - To segment the system for easier control and protection - To regulate the frequency of transmission - To monitor the load demand - Which of the following is true about primary protection?0 - It operates only when the backup protection fails - It is designed to isolate the faulted section of the system immediately - It works for non-critical sections of the system - It is used only in high-voltage networks - What is the function of a protective relay in a power system? - To control the voltage levels - To connect and disconnect the power supply - To detect abnormal conditions and operate the circuit breaker - To monitor system efficiency - Which of the following is a desirable quality of protective relaying? - High cost - Selectivity and reliability - Slow response time - Unpredictable operation - What is the primary cause of an electric arc in a circuit breaker during interruption? - High voltage - High current - Ionization of air between contacts - Mechanical wear - Which circuit breaker type uses an arc chute for arc extinction? - Oil circuit breaker - Air circuit breaker - Vacuum circuit breaker - SF6 circuit breaker - Which gas is used in SF6 circuit breakers for arc extinction? - Sulfur dioxide - Nitrogen - Sulfur hexafluoride - Hydrogen - What is the main purpose of an isolator in an electrical system? - Arc extinction - Fault detection - To provide visible isolation for maintenance - Overvoltage protection - What is the main function of a lightning arrester? - To extinguish arcs - To isolate faulty sections - To protect equipment from lightning surges - To absorb overcurrent - Which phenomenon occurs when lightning strikes a power line directly? - Recovery voltage - Direct stroke - Indirect stroke - Restriking voltage - What is "current chopping" in circuit breakers? - A deliberate reduction in current during fault conditions - Interruption of current before natural zero crossing - An increase in current during the arc extinction process - A condition caused by insufficient breaker contacts Answer Keys: 1. b; 2. b; 3. c; 4. c; 5. c. 6. c; 7. b; 8. b; 9. c; 10.b. 11. c; 12. b; 13. a; 14. c; 15. c. - b; - b. |

- Limited Response Questions

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) |
| --- | --- | --- |
| LU (1 to 5) | LU (6 to 8) | LU (9 to 11) |
| - Enlist advantages of power factor improvement. - State the causes of low power factor. - What are the key economic benefits of power factor correction? - Define power factor. - Define reactive power. - What is the voltage range of low, medium, and extra-high transmission lines? - Differentiate between overhead and underground transmission lines. - Why are extra-high voltage (E.H.T) lines preferred for long-distance power transmission? - Enlist advantages of AC over DC in power transmission. - Why is DC preferred for long-distance transmission in modern power systems? - What are the primary constants of a transmission line? - What factors affect the resistance of a transmission line conductor? - What causes voltage drops in transmission lines? - Define voltage regulation of power transmission lines. - What is the Ferranti effect? - Enlist different types of insulators. - Enlist different types of line supports. - Enlist different types of vibration dampers. - Enlist different types of transmission line conductors - State the function fo vibration damper. - State requirement of line insulator’s. - Define sag of transmission line. - What are the main components of an underground power cable? - List the classifications of underground cables based on voltage ratings. - What is the purpose of armoring in underground power cables | - What is an HVDC transmission system, and how does it differ from HVAC? - List two major advantages of HVDC over HVAC transmission systems. - Define the concept of inversion in HVDC systems. - What is a multi-bridge (B-bridge) converter station? - What is the function of a three-phase bridge converter in HVDC systems? - State the difference between a radial and a ring main distributor. - What are the advantages of a ring main distributor over a radial distributor? - Define radial distributor. - Define ring main distributor. - State primary function of a DC booster in a power distribution system. - Describe the role of an AC balancer in three-phase systems. - What are the main types of boosters used in AC systems? - State the shunt booster differ from a series booster? - What is the purpose of using a DC balancer set? | - State primary function of a substation in a power distribution system? - Enlist different equipment’s typically used in a substation. - Enlist different types of substations. - State the importance of substaion grounding. - State the purpose of a busbar in a substation. - State the function of a protection system in a power system. - Define protective zones in a power system. - Differentiate between primary and backup protection. - Enlist the desirable qualities of a good protective relaying system. - List the common protection relays used in power system. - What is Arc phenomenon in circuit breakers? - Define recovery voltage in a circuit breaker. - Define current chopping phenomenon in circuit breaker. - Define restriping voltage. - State advantages of SF6 circuit breaker. - Enlist different components of oil circuit breaker. - enlist physical properties of SF6 gas. - State the function of isolator in power system. - Differentiate between single-break and double-break isolators. - Define line surges in power systems? |

C.Assessment Tasks

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) |
| --- | --- | --- |
| LU (1 to 5) | LU (6 to 8) | LU (9 to 11) |
| Connect R-L loads in series and improve its power factor by using Capacitor. Performance criteria: - Adop safety precautions - Draw circuit diagram - Identify required equipment - Connect all equipment using circuit diagram and proper wiring techniques. - Turn on the ac supply and take initial readings - Connect a capacitor in parallel with the RL load. - Ensure secure and proper connections. - Turn on the power supply and record new readings (current, voltage, power, and power factor). - Observe improvement in power factor. - Calculate reactive power, apparent power, and compare the before-and-after power factor. - Aintain an organized and clean workspace to ensure efficiency and minimize hazards. | Connect DC Balancer in DC circuit for Voltage balancing Performance criteria: - Follow all safety procedures. - Draw the circuit diagram - identify all required tools and components (e.g., DC balancer, DC source, resistive load, connecting wires). - connect the DC balancer into the DC circuit as per the schematic diagram - Apply the DC voltage to the circuit and ensure the system operates as intended. - Monitor and record the voltage levels before and after balancing to confirm functionality. - Atain an organized and clean workspace to ensure efficiency and minimize hazards. | Construct and operate trip circuit and perform continuity test on it to ensure all components (relay, breaker coil, and auxiliary contacts) are properly operational. Performance criteria: - Follow all safety procedures - Draw the circuit diagram - Identify all required components and tools - Arrange components systematically and ensure proper labeling for clarity and accuracy. - Connect all the required equipmetn as per circuit diagram - Ensure all connections are secure and free of loose ends or improper terminations. - Energize the circuit and monitor its operation to ensure functionality. - Perform a continuity test using a multimeter to verify the proper connection of all components - Record observations, continuity test results, and any adjustments made during troubleshooting. |