## Course / Subject Name: NQ-ET 373 Industrial Electronics

- MCQs

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) |
| --- | --- | --- |
| LU (1-3) | LU (4-6) | LU (7-8) |
| - What is the basic function of a power diode in a circuit? a) Amplifying current b) Allowing current to flow in one direction c) Storing energy d) Switching between AC and DC - In series configuration, the current through each power diode is: a) Different b) Zero c) Same d) Unpredictable - In a series configuration of IGBTs, what must be matched to ensure balanced performance? a) Gate threshold voltages b) Gate capacitance c) Forward voltage drops d) All of the above - What happens when two power diodes are connected in series and one diode fails? a) Both diodes will continue to work. b) Current will continue to flow through the circuit. c) The circuit will be open and no current will flow. d) Only one diode will stop conducting. - For safe operation of SCRs in series, what is a critical parameter to monitor? a) Voltage across each SCR b) Temperature of the SCRs c) Gate pulse timing d) Reverse recovery time - What is a typical application of IGBTs in a series configuration? a) Audio amplification b) Power supplies c) Motor drives d) Lighting control - What is a key advantage of using IGBTs in a series configuration over SCRs? a) Higher switching speed b) Lower forward voltage drop c) Better temperature stability d) Lower cost - What is basic function of a diode in a circuit a) Amplification b) Switching c) Rectification d) Filtering - Which component is most suitable for high-speed switching applications in series configurations? A) Power diode B) SCR C) IGBT D) Transformer - Which of the following is a characteristic of power diodes? A) Low current handling B) High voltage handling C) High frequency operation D) Low power dissipation - What is the basic function of an IGBT? a) Amplification b) Switching c) Rectification d) Filtering - Which characteristic makes IGBTs suitable for high-power applications? a) High current handling b) High voltage handling c) High frequency operation d) All of the above - What is the purpose of a datasheet when working with IGBTs? a) Circuit diagrams b) Operating parameters c) Manufacturing process d) Safety precautions - What is the primary purpose of connecting power diodes in parallel? a) Increase voltage handling b) Increase current handling c) Improve efficiency d) Reduce cost - What should be considered when selecting power diodes for parallel connection? a) Forward voltage drop b) Reverse recovery time c) Current handling capacity d) All of the above - What is the purpose of testing a parallel SCR connection? a) Verify polarity b) Check for shorts c) Measure voltage drop d) Ensure proper functioning - Which tool is commonly used to test parallel SCR connections? a) Multimeter b) Oscilloscope c) Signal generator d) Logic analyzer - How should SCRs be connected to prevent reverse polarity? a) Using a fuse b) Using a diode c) Using a resistor d) Using a rectifier - Which component helps ensure equal current sharing in parallel SCRs? a) Resistors b) Inductors c) Capacitors d) Current-sharing devices - What is the primary purpose of forced commutation in rectifier circuits? a) Improve efficiency b) Increase voltage handling c) Enable controlled switching d) Reduce cost - Which component is crucial for forced commutation? a) Diode b) Thyristor c) IGBT d) Capacitor - What is the primary purpose of natural commutation in rectifier circuits? a) Improve efficiency b) Increase voltage handling c) Enable controlled switching d) Reduce cost - Which component is crucial for natural commutation? a) Diode b) Thyristor c) IGBT d) Transformer - Which type of rectifier circuit uses natural commutation? a) Half-wave b) Full-wave c) Bridge d) All of the above - How should components be connected in forced commutated rectifier circuits? a) Series connection b) Parallel connection c) Bridge connection d) Hybrid connection Which component helps ensure proper current sharing in forced commutated rectifier circuits? a) Resistors b) Inductors c) Capacitors d) Current-sharing devices - How should the gate trigger be connected in forced commutated rectifier circuits? a) Directly to the thyristor b) Through a resistor c) Through a capacitor d) Through a transformer - What is the importance of proper grounding in forced commutated rectifier circuits? a) Reduces noise b) Improves efficiency c) Increases reliability d) All of the above - Connect Components in Naturally Commutated Rectifier Circuits 1. How should components be connected in naturally commutated rectifier circuits? a) Series connection b) Parallel connection c) Bridge connection d) Hybrid connection - What is the purpose of a filter capacitor in naturally commutated rectifier circuits? a) Regulate output voltage b) Control switching frequency c) Improve efficiency d) Reduce harmonics - How should the diodes be connected in naturally commutated rectifier circuits? a) In series b) In parallel c) In a bridge configuration d) In a hybrid configuration Answer Keys: 1. b; 2. c; 3. d. 4. c; 5. a; 6. c. 7. a; 8. c; 9. c. 10. b; 11. b; 12. d. 13. b; 14. b; 15. d. 16. d; 17. a; 18. b. 19. d; 20. c; 21. b. 22. a; 23. a; 24. d. 25. c; 26. b; 27. d. 28. c; 29. a; 30. c. | - What is the basic function of a 1-phase full-wave inverter? a) Convert DC to AC b) Convert AC to DC c) Regulate voltage d) Filter harmonics - Which topology is commonly used for 1-phase full-wave inverters? a) Half-bridge b) Full-bridge c) Push-pull d) Flyback - What is the advantage of using PWM (Pulse Width Modulation) in 1-phase full-wave inverters? a) Improved efficiency b) Reduced harmonics c) Increased reliability d) All of the above - How is the output voltage regulated in 1-phase full-wave inverters? a) Using a transformer b) Using a voltage regulator c) Adjusting PWM frequency d) Adjusting PWM duty cycle - What is the purpose of a filter capacitor in 1-phase full-wave inverters? a) Regulate output voltage b) Filter harmonics c) Improve efficiency d) Reduce noise - What is the basic purpose of four-quadrant control in 1-phase inverters? a) Improve efficiency b) Increase reliability c) Enable bidirectional power flow d) Reduce harmonics - Which control strategy is commonly used for four-quadrant control in 1-phase inverters? a) PWM (Pulse Width Modulation) b) PFM (Pulse Frequency Modulation) c) SVM (Space Vector Modulation) d) DTC (Direct Torque Control) - What is the advantage of using four-quadrant control in 1-phase inverters? a) Improved efficiency b) Increased reliability c) Enhanced flexibility d) All of the above - What is the basic function of a 3-phase full-wave inverter? a) Convert DC to AC b) Convert AC to DC c) Regulate voltage d) Filter harmonics - Which topology is commonly used for 3-phase full-wave inverters? a) Half-bridge b) Full-bridge c) Six-switch inverter d) Twelve-switch inverter - What is the advantage of using PWM (Pulse Width Modulation) in 3-phase full-wave inverters? a) Improved efficiency b) Reduced harmonics c) Increased reliability d) All of the above - How is the output voltage regulated in 3-phase full-wave inverters? a) Using a transformer b) Using a voltage regulator c) Adjusting PWM frequency d) Adjusting PWM duty cycle - What is the purpose of a filter capacitor in 3-phase full-wave inverters? a) Regulate output voltage b) Filter harmonics c) Improve efficiency d) Reduce noise - What is the primary purpose of four-quadrant control in 3-phase inverters? a) Improve efficiency b) Increase reliability c) Enable bidirectional power flow d) Reduce harmonics - Which control strategy is commonly used for four-quadrant control in 3-phase inverters? a) PWM (Pulse Width Modulation) b) PFM (Pulse Frequency Modulation) c) SVM (Space Vector Modulation) d) DTC (Direct Torque Control) - What is the basic function of a rectifier in a DC motor control system? a) Convert AC to DC b) Convert DC to AC c) Regulate voltage d) Filter harmonics - Which type of rectifier is commonly used for DC motor control? a) Half-wave rectifier b) Full-wave rectifier c) Bridge rectifier d) PWM rectifier - What is the purpose of a filter capacitor in a DC motor control system? a) Regulate output voltage b) Filter harmonics c) Improve efficiency d) Reduce noise - How is the motor speed controlled in a DC motor control system? a) Adjusting PWM frequency b) Adjusting PWM duty cycle c) Adjusting voltage d) Adjusting current - What is the advantage of using a rectifier in a DC motor control system? a) Improved efficiency b) Increased reliability c) Simplified design d) Reduced cost - What is the basic function of a rectifier in an AC motor control system? a) Convert AC to DC b) Convert DC to AC c) Regulate voltage d) Filter harmonics - Which type of rectifier is commonly used for AC motor control? a) Half-wave rectifier b) Full-wave rectifier c) Bridge rectifier d) PWM rectifier - What is the purpose of an inverter in an AC motor control system? a) Convert DC to AC b) Convert AC to DC c) Regulate voltage d) Filter harmonics - How is the motor speed controlled in an AC motor control system? a) Adjusting PWM frequency b) Adjusting PWM duty cycle c) Adjusting voltage d) Adjusting frequency - What is the advantage of using a rectifier in an AC motor control system? a) Improved efficiency b) Increased reliability c) Simplified design d) Reduced cost - What is the basic function of an SCR cyclo-converter in a DC motor control system? a) Convert AC to DC b) Convert DC to AC c) Regulate voltage d) Control motor speed - Which type of SCR is commonly used for DC motor control? a) Thyristor b) Triac c) IGBT d) MOSFET - What is the purpose of a gate trigger in an SCR cyclo-converter? a) Regulate output voltage b) Control motor speed c) Initialize commutation d) Protect against overcurrent - How is the motor speed controlled in an SCR cyclo-converter-based DC motor control system? a) Adjusting PWM frequency b) Adjusting PWM duty cycle c) Adjusting voltage d) Adjusting firing angle - What is the advantage of using an SCR cyclo-converter in a DC motor control system? a) Improved efficiency b) Increased reliability c) Simplified design d) Reduced cost - What is the basic function of an SCR cyclo-converter in an AC motor control system? a) Convert AC to DC b) Convert DC to AC c) Regulate voltage d) Control motor speed - Which type of SCR is commonly used for AC motor control? a) Thyristor b) Triac c) IGBT d) MOSFET - What is the purpose of a gate trigger in an SCR cyclo-converter? a) Regulate output voltage b) Control motor speed c) Initialize commutation d) Protect against overcurrent Answer Keys: 1. a; 2. b; 3. d. 4. d; 5. b; 6. c. 7. c; 8. d; 9. a. 10. c; 11. d; 12. d. 13. b; 14. c; 15. c. 16. a; 17. c; 18. b. 19. b; 20. a; 21. a. 22. c; 23. a; 24. d. 25. a; 26. d; 27. a. 28. c; 29. d; 30. a. 31. d; 32. a; 33. c. | - What is the basic function of a synchro-generator? a) Convert mechanical energy to electrical energy b) Convert electrical energy to mechanical energy c) Regulate voltage d) Filter harmonics - Which type of synchro-generator is commonly used? a) Alternator b) Generator c) Transformer d) Motor - What is the purpose of the rotor in a synchro-generator? a) Produce magnetic field b) Generate voltage c) Regulate frequency d) Provide mechanical support - How is the output voltage of a synchro-generator controlled? a) Adjusting excitation current b) Adjusting armature current c) Adjusting frequency d) Adjusting voltage - What is the advantage of using a synchro-generator? a) Improved efficiency b) Increased reliability c) Simplified design d) Reduced cost - What is the basic function of a synchro-motor? a) Convert electrical energy to mechanical energy b) Convert mechanical energy to electrical energy c) Regulate voltage d) Filter harmonics - Which type of synchro-motor is commonly used? a) Induction motor b) Synchronous motor c) DC motor d) Stepper motor - What is the purpose of the stator in a synchro-motor? a) Produce magnetic field b) Generate torque c) Regulate frequency d) Provide mechanical support - How is the speed of a synchro-motor controlled? a) Adjusting voltage b) Adjusting current c) Adjusting frequency d) Adjusting load - What is the advantage of using a synchro-motor? a) Improved efficiency b) Increased reliability c) Simplified design d) Reduced cost - Which component is used for position feedback in AC servo-mechanisms? a) Encoder b) Amplifier c) Controller d) Motor - What is the purpose of the controller in an AC servo-mechanism? a) Regulate voltage b) Control current c) Monitor position d) Adjust speed - What is the basic function of a DC servo-mechanism? a) Convert electrical energy to mechanical energy b) Convert mechanical energy to electrical energy c) Regulate voltage d) Filter harmonics - Which component is used for position feedback in DC servo-mechanisms? a) Encoder b) Amplifier c) Controller d) Motor - What is the purpose of the amplifier in a DC servo-mechanism? a) Regulate voltage b) Control current c) Monitor position d) Adjust speed - What is the basic function of a PLC? a) To control mechanical processes b) To regulate electrical systems c) To monitor industrial processes d) To automate manufacturing - Which component is responsible for storing PLC programs? a) CPU b) Memory c) Input/Output modules d) Power supply - What is the advantage of using a PLC? a) Improved efficiency b) Increased reliability c) Simplified design d) Reduced cost - What is the name of the software used for programming Allen-Bradley PLCs? a) RSLogix b) TIA Portal c) GX Developer d) CX-Programmer - What is the purpose of PLC programming software? a) To monitor PLC performance b) To troubleshoot PLC issues c) To create and edit PLC programs d) To configure PLC hardware - What is the purpose of memory mapping in PLCs? a) To allocate memory for programs b) To assign addresses to data c) To configure input/output modules d) To monitor PLC performance - What is the purpose of addressing techniques in PLCs? a) To access data in memory b) To configure input/output modules c) To monitor PLC performance d) To troubleshoot PLC issues - What is the advantage of using symbolic addressing? a) Improved efficiency b) Increased reliability c) Simplified programming d) Reduced memory usage - What is the scan cycle in a PLC? a) The time it takes to execute a program b) The time it takes to update inputs/outputs c) The sequence of program execution d) The mode of PLC operation - Which PLC mode is commonly used for normal operation? a) Run mode b) Monitor mode c) Test mode d) Debug mode - What happens when a PLC is in Run mode? a) Programs are executed continuously b) Inputs/outputs are updated continuously c) PLC performance is monitored d) PLC programs are edited Answer Keys: 1. a; 2. a; 3. a. 4. a; 5. a; 6. a. 7. b; 8. a; 9. c. 10. a; 11.a; 12. c. 13. a; 14. a; 15. b. 16. d; 17. b; 18. a. 19. a; 20. c; 21. b. 22. a; 23. c; 24. c. 25. a; 26. a. |

- Limited Response Questions

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) |
| --- | --- | --- |
| LU (1-3) | LU (4-6) | LU (7-8) |
| - Enlist Series operation of power diodes - Define SCR - Define Construction rating of SCR - What is operation and application of SCR? - Define Firing /triggering technique of SCR - How to measure SCR parameters - Draw the VI characteristics of SCR - Describe Series operation of SCR - Define Construction biasing of IGBT. - State working and application of IGBT - How to find rating of IGBT - differentiate between Series and parallel operation of IGBT. - Define Parallel operation of power diodes - Define Parallel operation of SCR - Enlist the Types of commutation - Draw Single phase half & full wave central tape rectification. - Draw Three phase full wave fully bridge rectification. | - Enlist the types and uses of inverter - Define the single phase full-wave inverter circuit - What is single phase rectification? - State the Single-phase combined operation of rectifier - Define inverter as four quadrant control of a converter. - Draw 3-phase full-wave bridge inverter circuit - Draw 3 phase Combined operation of rectifier - Define inverter as four quadrant control of a converter - What is Speed control of DC motors using 3-phase full-wave controlled rectifier circuits. - What is Speed control of DC motors using single phase fully controlled rectifier circuit, with the help of circuit and waveform - What is 3-phase fully phases controlled, 4-quadrant speed control of DC motors. - Draw the diagram Single phase central tape & bridge | - Define the synchro generator and working principle - Define types of synchro generator types - Explain the Applications of synchro-generator - Define the synchro- motor, working principle - Define working principle synchro- motor, - Draw the AC servo-mechanism, block diagram - Define Applications of AC-servo mechanism - Define DC-servo mechanism - Draw the Block diagram of PLC, - what is the working principle of PLC - Define key components of PLC |

- Assessment Tasks

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) |
| --- | --- | --- |
| LU (1-3) | LU (4-6) | LU (7-8) |
| - Connect power diodes and IGBT in series & parallel Performance Criteria: - Operate power diodes in circuits. - Operate IGBTs in multiple applications. - Connect power diodes in series configurations. - Connect SCRs in series configurations. - Connect IGBTs in series configurations. - Operate power diode in circuit. - Operate IGBTs in multiple applications. - Connect power diodes in parallel configurations. - Connect SCRs in parallel configurations. - Connect IGBTs in parallel configurations. - Assemble naturally commutated full wave semi controlled rectifier circuits. Performance Criteria: - Assemble forced commutated rectifier circuits. - Assemble naturally commutated rectifier circuits. - Connect components in forced commutated rectifier circuits. - Connect components in naturally commutated rectifier circuits. - Adjust settings for forced commutated rectifier circuits. - Adjust settings for naturally commutated rectifier circuits. | - Construct three phase full wave inverter Performance Criteria: - Operate 1-phase full-wave inverters - Implement four-quadrant control for 1-phase inverters. - Operate 3-phase full-wave inverters. - Implement four-quadrant control for 3-phase inverter - Implement four-quadrant control for 3- phase inverter Performance Criteria: - Connect DC motor control systems using rectifiers. - Connect AC motor control systems using rectifiers. - Configure DC motor control systems using SCR cyclo-converters. - Configure AC motor control systems using SCR cyclo-converters. | - Operate AC servo-mechanisms and show is results Performance Criteria: - Operate synchro-generators. - Operate synchro-motors. - Operate AC servo-mechanisms. - Operate DC servo-mechanisms. - Perform ON & OFF operation of indicator lamp by using only one normally open push button using PLC Performance Criteria: - Configure input and output modules for PLCs. - Debug ladder logic programs for PLCs. - Test motor circuits controlled by PLCs. - Calibrate sensors and actuators interfaced with PLCs. - Modify existing ladder logic programs to meet new requirements. |