Course / Subject Name:  NQ-ET 125 Principles of Electrical Engineering

Assessment Task (Summative Assessment)

| a) Set up a simple capacitive voltage divider circuit using two capacitors in series, and connect it to an AC power supply (with a frequency of 50Hz). Measure the voltage across each capacitor, and calculate the total impedance of the circuit. Also measure breakdown down voltage of both capacitor. |  |  |  |
| --- | --- | --- | --- |
| Critical / Non Critical | Performance Criteria | Yes | No |
| Non Critical | - Select appropriate PPEs to ensure health and safety during the task. |  |  |
| Critical | - Identify and gather the required tools, components, and measuring instruments. |  |  |
| Non Critical | - Draw a connection diagram for the capacitive voltage divider circuit. |  |  |
| Critical | - Set up the circuit using two capacitors connected in series with an AC power supply (50 Hz). |  |  |
| Critical | - Verify the connections and ensure the circuit is properly assembled |  |  |
| Critical | - Measure the voltage across each capacitor using a voltmeter. |  |  |
| Critical | - Measure the total current |  |  |
| Critical | - Calculate the total impedance of the circuit based on the measured values |  |  |
| Critical | - Increase the applied voltage gradually. |  |  |
| Critical | - Test and measure the breakdown voltage of both capacitors |  |  |
| Critical | - Record all measurements and calculations in a structured table. |  |  |
| Non Critical | - Analyze the results to confirm the circuit's functionality |  |  |
| Non Critical | - Disassemble the circuit safely after completing the measurements |  |  |
| Non Critical | - Store all the tools, equipments and instruments in a specified storage area |  |  |

| b) Construct two coils, A and B, with different numbers of turns. Connect an AC variable power supply to coil A and measure the induced voltage in coil B. Observe and record the effect of inserting an iron core between the two coils. |  |  |  |
| --- | --- | --- | --- |
| Critical / Non Critical | Performance Criteria | Yes | No |
| Non Critical | - Select appropriate PPEs to ensure health and safety during the experiment. |  |  |
| Non Critical | - Identify and gather the necessary tools, equipment, and components for the task. |  |  |
| Critical | - Construct two copper coils, A and B, with different numbers of turns. |  |  |
| Non Critical | - Draw a connection diagram for the setup, including the AC variable power supply and the two coils. |  |  |
| Non Critical | - Connect the AC variable power supply to coil A and verify the connections |  |  |
| Critical | - Measure the induced voltage in coil B using a voltmeter |  |  |
| Critical | - Insert an iron core between the two coils and observe the changes in the induced voltage. |  |  |
| Non Critical | - Record the observations and measurements in a systematic tabular format |  |  |
| Non Critical | - Analyze and record the effect of the iron core on the induced voltage |  |  |
| Non Critical | - Disassemble the setup safely after completing the experiment. - Store all the tools, equipments and instruments in a specified storage area |  |  |

| c) Measure and record the line voltage and phase sequence of the three-phase supply. Connect a resistive load (three 100W lamps) with a voltmeter, ammeter, and power factor meter. Draw a single-phase supply from the three-phase source and measure the total voltage, current, power, and power factor. |  |  |  |
| --- | --- | --- | --- |
| Critical / Non Critical | Performance Criteria | Yes | No |
| Non Critical | - Select appropriate PPEs to ensure health and safety during the experiment. |  |  |
| Critical | - Identify and gather the necessary tools, equipment, and instruments for the task. |  |  |
| Critical | - Connect a volt meter to Measure and record the line voltage of the three-phase supply |  |  |
| Critical | - Connect the phase sequence meter and record the phase sequence of the supply. |  |  |
| Critical | - Remove the phase sequence meter |  |  |
| Critical | - Construct the circuit with three 100W lamps as a resistive load. |  |  |
| Critical | - Draw a single-phase supply from the three-phase source. |  |  |
| Critical | - Install the circuit with single phase supply and connect volt meter and ammeter in the circuit. |  |  |
| Critical | - Measure the current and voltage drop across each lamp |  |  |
| Critical | - Calculate the total power by using the power formulas (P=V×I) |  |  |
| Critical | - Calculate power factor by using the formula cosø= P/V×I |  |  |
| Non Critical | - Document all readings and observations in a tabular format. |  |  |
| NonCritical | - Disassemble the circuit safely after completing the measurements |  |  |
| NonCritical | - Store all the tools, equipments and instruments in a specified storage area |  |  |