Course / Subject Name NQ-ET 252 - Electrical Installation, Planning & Estimating

Assessment Tasks (Summative Assessment)

| Design, estimate and install domestic wiring with PVC casing capping including earthing connection to the MDB as per the desired residential or commercial plan. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Critical/Noncritical | Performance Criteria | Yes |  | No |
| Critical | - Collect Requirements of electrical installation. |  |  |  |
| Critical | - Draw a simple wiring layout for installation. |  |  |  |
| Non Critical | - Plan routes for conduits and wiring. |  |  |  |
| Non Critical | - Mark circuit breakers and safety devices Points. |  |  |  |
| Non Critical | - Mark an earthing points in the design |  |  |  |
| Critical | - Measure the total length of wiring required. |  |  |  |
| Critical | - Count the number of junction boxes needed. |  |  |  |
| Critical | - Calculate the number and type of bends and fittings. |  |  |  |
| Non Critical | - List the required size and type of cables. |  |  |  |
| Critical | - Collect tools like screwdrivers, pliers, drills, and cutting tools. |  |  |  |
| Critical | - Collect required materials like wires, PVC casing, capping, screws, and earthing rods. |  |  |  |
| Non Critical | - Mark locations for switches, sockets, and other fixtures on walls. |  |  |  |
| Critical | - Cut PVC casing to the required length. |  |  |  |
| Critical | - Fix the casing on the marked routes using screws. |  |  |  |
| Critical | - Pull wires through the casing as per the plan. |  |  |  |
| Critical | - Close the capping on the casing after placing the wires. |  |  |  |
| Critical | - Connect wires to switches, sockets, and fixtures securely. |  |  |  |
| Critical | - Connect the earthing wire to the earthing terminal of the MDB. |  |  |  |
| Critical | - Connect the incoming supply, outgoing circuits, and earthing to the MDB. |  |  |  |
| Non Critical | - Check all connections for continuity by multimeter. |  |  |  |
| Non Critical | - Verify earthing resistance and safety. |  |  |  |
| Non Critical | - Identify and fix any loose or incorrect connections. |  |  |  |
| Critical | - Ensure all wiring complies with safety standards. |  |  |  |
| Non Critical | - Remove debris, unused materials, and tools from the site. |  |  |  |
| Non Critical | - Clean the site for client. |  |  |  |