Course / Subject Name: NQ-ET 283 Basic Electonics

Assessment Task (Summative Assessment)

| - Construction of Full-Wave Bridge Rectifier |  |  |  |
| --- | --- | --- | --- |
| #### Critical / Non Critical | #### Performance Criteria | #### Yes | #### No |
| Non-Critical | - Gather the required components: four diodes, step down transformer (220V/12V), load resistor Rs, oscilloscope, Capacitor, Zener Diode, connecting wires and AC power supply. |  |  |
| Non-Critical | - Connect the primary side of the transformer to the 220V AC power supply. |  |  |
| Non-Critical | - Connect bridge configuration with the secondary side of the transformer. |  |  |
| Non-Critical | - Connect the load resistor across the diodes' output. |  |  |
| Critical | - Complete the circuit connections |  |  |
| Critical | - Apply AC input voltage. |  |  |
| Critical | - Use the oscilloscope to observe and measure the input and output wave forms of the rectifier. |  |  |
| Non-Critical | - Verify the rectified output is DC with ripple. |  |  |

| - Assembly of Alarm System with Photo-resistor |  |  |  |
| --- | --- | --- | --- |
| #### Critical / Non Critical | #### Performance Criteria | #### Yes | #### No |
| Non-Critical | - Gather the necessary components: photoresistor (LDR), transistor, alarm (buzzer), resistor, power supply, and breadboard. |  |  |
| Non-Critical | - Connect the photoresistor (LDR) in a voltage divider configuration with a resistor. |  |  |
| Non-Critical | - Connect the output of the LDR network to the base of a transistor. |  |  |
| Non-Critical | - Connect the collector of the transistor to the buzzer, and the emitter to ground. |  |  |
| Critical | - Complete the circuit connections |  |  |
| Critical | - Power the circuit with a DC power supply. |  |  |
| Critical | - Shine light on the LDR to change its resistance. |  |  |
| Critical | - Observed if the buzzer turns on or off based on the light intensity. |  |  |

| - Measurement of DC operating voltages of a JFET amplifier |  |  |  |
| --- | --- | --- | --- |
| #### Critical / Non Critical | #### Performance Criteria | #### Yes | #### No |
| Non-Critical | - Get the JFET transistor, resistors, capacitors, DC power supply, and multimeter. |  |  |
| Non-Critical | - Connect the JFET in a common-source configuration. |  |  |
| Non-Critical | - Used resistors for biasing and capacitors for coupling at the input and output. |  |  |
| Critical | - Complete the circuit connection |  |  |
| Critical | - Connect the DC power supply to the appropriate points for the drain and source. |  |  |
| Critical | - Used the multimeter to measure the DC voltages at the gate, drain, and source of the JFET. |  |  |
| Critical | - Ensure the gate-source voltage is within the recommended range for proper operation. |  |  |

| #### Demonstration of bidirectional conduction in a gated TRIAC and DIAC functional use of op-amps in various circuits to observe their operation and outputs. |  |  |  |
| --- | --- | --- | --- |
| #### Critical / Non Critical | #### Performance Criteria | #### Yes | #### No |
| Non-Critical | - Collect TRIAC, DIAC, resistor, capacitor, AC power supply, and oscilloscope |  |  |
| Non-Critical | - Connect the DIAC in series with the gate of the TRIAC. |  |  |
| Non-Critical | - Connect the TRIAC to the AC supply, with the load in series. |  |  |
| Critical | - Use a capacitor to trigger the DIAC and control the TRIAC's firing. |  |  |
| Critical | - Apply a gate pulse to the TRIAC through the DIAC and observe its bidirectional conduction. |  |  |
| Critical | - Use the oscilloscope to monitor the voltage and current waveforms across the TRIAC. |  |  |

| #### Functional use of op-amps in various circuits to observe their operation and outputs. |  |  |  |
| --- | --- | --- | --- |
| #### Critical / Non Critical | #### Performance Criteria | #### Yes | #### No |
| Non-Critical | - Collect Op-Amps (e.g., 741), resistors, capacitors, DC power supply, oscilloscope, and breadboard. |  |  |
| Non-Critical | - Set up different Op-Amp configurations: |  |  |
| Critical | - Inverting Amplifier: Connect the input signal through a resistor to the inverting terminal, and the non-inverting terminal to ground. |  |  |
| Critical | - Non-Inverting Amplifier: Connect the input signal to the non-inverting terminal and feedback to the inverting terminal. |  |  |
| Critical | - Comparator: Connect the two input terminals to different reference voltages. |  |  |
| Non-Critical | - Connect appropriate power supplies (typically +15V and -15V for op-amps). |  |  |
| Critical | - Apply different input signals and observe the output on the oscilloscope. |  |  |
| Non-Critical | - For the inverting amplifier, verify the phase shift. |  |  |
| Critical | - For the comparator, observe the threshold behavior and output switching. |  |  |