Course / Subject Name:  NQ-ET 203 DC MACHINES & BATTERIES

Assessment Task (Summative Assessment)

| a) Measure resistance of field winding, armature winding and determine terminal polarity of a DC Geneartor. |  |  |  |
| --- | --- | --- | --- |
| Critical / Non Critical | Performance Criteria | Yes | No |
| Non Critical | - Wear PPEs for ensuring health and safety. |  |  |
| Critical | - Select required Tools / Equipmensts / Materials and a DC Machine. |  |  |
| Critical | - Place DC machine on workbench. |  |  |
| Critical | - Open terminal box. |  |  |
| Critical | - Connect Ohm meter with the terminal of both windings. |  |  |
| Critical | - Record the value of Ohm's meter reading. |  |  |
| Critical | - Connect Galvanometer with armature winding. |  |  |
| Critical | - Rotate Armature slowly by hand in clockwise direction. |  |  |
| Critical | - Identify armature polarity with the deflection of Galvanometer pointer. |  |  |
| Non Critical | - Place all the tools and equipments in specified area. |  |  |

| b) Measure Speed, Current and Torque of DC Shunt Motor and plot load characteristics. |  |  |  |
| --- | --- | --- | --- |
| Critical/ Non Critical | Performance Criteria | Yes | No |
| Non Critical | - Wear PPEs for ensuring health and safety. |  |  |
| Critical | - Select required Tools / Equipments / Materials and a DC shunt Motor. |  |  |
| Critical | - Connect shunt motor according to the diagram. |  |  |
| Critical | - Carry out Coupling of electro dynamo meter to the DC motor with belt , Set dynamo meter control knob in such position to produce a minimum starting load. |  |  |
| Critical | - Turn ON power supply. |  |  |
| Critical | - Run motor at rated speed. |  |  |
| Critical | - Apply load to DC motor by varying the dynamo meter control knob. |  |  |
| Critical | - Measure current, speed and torque. |  |  |
| Critical | - Draw load characteristics on DC shunt motor. |  |  |
| Non Critical | - Place all the tools and equipments and place in specified area. |  |  |

| C) Construct series parallel Combination of Batteries, measure the voltage across the batteries and operate DC Series motor with the help of the batteries |  |  |  |
| --- | --- | --- | --- |
| Critical/ Non Critical | Performance Criteria | Yes | No |
| Non Critical | - Wear PPEs for ensuring health and safety. |  |  |
| Non Critical | - Select required Tools / Equipments / materials. |  |  |
| Critical | - Select 04 No’s of Batteries. |  |  |
| Critical | - Construct Series-Parallel Combnation of Batteries and record the voltage. |  |  |
| Critical | - Connect the two combination of Series batteries in parallel. |  |  |
| Non Critical | - Place a DC Series Motor on workstation. |  |  |
| Critical | - Identify the terminals of DC series Motor. |  |  |
| Critical | - Connect series winding and armature winding in series connection. |  |  |
| Critical | - Connect the DC Series motor with combnation of batteries. |  |  |
| Critical | - Connect a DC circuit breaker between the combinatin of batteries and DC series motor for switching and surges protection. |  |  |
| Critical | - Switch on the DC breaker and operate the DC series motor. |  |  |
| Non Critical | - Switch off the DC breaker and Disassemble the circuit |  |  |
| Non Critical | - Place all the tools and equipments and place in specified area. |  |  |