### Course / Subject Name: NQ-ET 273 Electrical Instruments and Measurements

Assessment Tasks (Summative Assessment)

| - Perform measurements of electrical parameters using different measuring instruments(Ammeter, Volt Meter Multimeter,Tachometer,Lux meter,single-phase Energy meter ,three-phase Energy meter, megger and Wheatstone bridge) |  |  |  |
| --- | --- | --- | --- |
| Critical/NonCritical | Performance Criteria | Yes | No |
| Non Critical | Select the required PPE to ensure health and safety. |  |  |
| Non Critical | Select the Required tool, equipement and measuring instruments. |  |  |
| Non Critical | Calibrate the measuring instruments. |  |  |
| Critical | Connect the ammeter in series with the load for measurement of current. |  |  |
| Non Critical | Ensure the current does not exceed the meter’s rated capacity. |  |  |
| Non Critical | Record the current reading accurately. |  |  |
| Critical | Connect the voltmeter in parallel with the component or circuit for measurement of voltage. |  |  |
| Non Critical | Read and note the voltage value displayed. |  |  |
| Critical | Select the correct function (voltage, current, resistance) on the multimeter. |  |  |
| Critical | Set multimeter on the appropriate range setting to avoid dammage |  |  |
| Critical | Ensure proper probe connections. |  |  |
| Critical | Measure the electrical quantitiy(Current, Voltage and Resistance) with multimeter. |  |  |
| Critical | Position the tachometer sensor correctly on the rotating object. |  |  |
| Non Critical | Record revolutions per minute (RPM) accurately. |  |  |
| Non Critical | Place the lux meter sensor under the light source. |  |  |
| Critical | Ensure there are no obstructions affecting the light intensity. |  |  |
| Critical | Record the light intensity in lux. |  |  |
| Critical | Connect the energy meter with the load and ensure proper wiring (Phase and neutral). |  |  |
| Non Critical | Verify the load operation and record energy consumption over a specified duration. |  |  |
| Critical | Connect the three phase meter according to the manufacturer’s wiring diagram (3-phase 4-wire or 3 Phase 3-wire). |  |  |
| Non Critical | Ensure proper load balancing and record energy usage. |  |  |
| Critical | Disconnect the Megger from the power supply before testing. |  |  |
| Critical | Connect the megger leads to the earth electrode and reference point. |  |  |
| Critical | Apply the test voltage and record the resistance value. |  |  |
| Critical | Connect the unknown resistor in the bridge circuit. |  |  |
| Non Critical | Adjust the variable resistor to achieve a balanced condition. |  |  |
| Critical | Calculate the unkown medium resistance. |  |  |

| - Perform Measurment of frequency and voltage with the help of oscilloscope, power factor with a power factor meter, inductance, capacitance, and resistance with an LCR meter. |  |  |  |
| --- | --- | --- | --- |
| Critical/NonCritical | Performance Criteria: | Yes | No |
| Critical | Connect the oscilloscope probes to the circuit under test. |  |  |
| Non Critical | Adjust time and voltage settings for a stable waveform. |  |  |
| Critical | Measure peak-to-peak voltage and frequency from the displayed waveform. |  |  |
| Critical | Connect the power factor meter in the circuit, ensuring it matches the system voltage and current ratings. |  |  |
| Non Critical | Record the power factor reading. |  |  |
| Critical | Select the desired parameter (L, C, or R) on the LCR meter. |  |  |
| Critical | Connect the test leads to the component and choose an appropriate frequency. |  |  |
| Non Critical | Record the displayed values of inductance, capicitance and resistance. |  |  |
| Non Critical | Verify measurements with reference values or alternative instruments. |  |  |
| Non Critical | Document all readings and observations systematically. |  |  |