Course / Subject Name  NQ. Mech-127. Workshop Practice-I (C). Perform Welding and Forging

- MCQs

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) |
| --- | --- | --- |
| LU 01-03 | LU 04-06 | LU 07-08 |
| - Which of the following types of fuel gas is commonly used in gas welding? a) Biogas b) Coal gas c) Acetylene d) Methane - Oxy-acetylene process is most suited for which of the following process of joining? a) Metal wires b) Metal sheets c) Metal tubes d) Metal bars - For what thickness of a material, in an oxy-acetylene is a welding rod used? a) 10 mm b) 15 mm c) 20 mm d) 25 mm - How many types of flames are there in gass welding? a) 1 b) 2 c) 3 d) 4 - In which of the following type of flame, oxygen is in excess proportion with acetylene? a) Neutral flame b) Oxidizing flame c) Carburizing flame d) Both oxidizing flame and carburizing flame - Which of the following flame is harmful to steel? a) Neutral flame b) Oxidizing flame c) Carburizing flame d) Both oxidizing flame and carburizing flame - The chemical formula of acetylene is? a) C2H4 b) C2H6 c) C2H5OH d) C2H2 - The oxy acetylene gas welding is a type of? a) Endothermic reaction b) Exothermic reaction c) Neutral reaction d) Both endothermic reaction and exothermic reaction - 25.4mm is equal ? a) 2inch b) 1inch c) 1.5inch d) .5inch - Grinder is tool? a) marking tool b) cutting tool c) holding tool d) none of a,b,c 11. What is the primary heat source in oxy-acetylene welding? a) Electric arc b) Gas flame c) Laser beam d) Plasma torch 12. What is the purpose of flux in welding? a) To cool the weld b) To remove impurities and protect the weld c) To increase heat generation d) To prevent oxidation of the base metal - 2.54mm is equal ? a) 2cm b) 2.54cm c) 1.5cm d) 1cm - 6inch is equal ? a) 13cm b) 12cm c) 10cm d) 15cm | 1. What is the main purpose of welding? a) To create a strong bond between materials b) To cut materials c) To polish surfaces d) To paint materials 2. Which type of welding uses a consumable electrode? a) TIG welding b) MIG welding c) Oxy-acetylene welding d) Resistance welding 3. What does TIG stand for in welding? a) Tungsten Inert Gas b) Titanium Inert Gas c) Tungsten Integrated Gas d) Titanium Integrated Gas 4. Which gas is commonly used as a shielding gas in MIG welding? a) Hydrogen b) Argon c) Nitrogen d) Oxygen 5. What is the primary heat source in oxy-acetylene welding? a) Electric arc b) Gas flame c) Laser beam d) Plasma torch 6. In which welding process is slag produced? a) TIG welding b) MIG welding c) Shielded Metal Arc Welding (SMAW) d) Oxy-acetylene welding 7. What is the purpose of flux in welding? a) To cool the weld b) To remove impurities and protect the weld c) To increase heat generation d) To prevent oxidation of the base metal 8. Which welding technique is best suited for aluminum? a) TIG welding b) MIG welding c) Arc welding d) Spot welding 9. What is porosity in a weld? a) A smooth weld surface b) Presence of gas pockets in the weld c) Excess metal on the weld d) A strong weld joint 10. What does the term "weld pool" refer to? a) The area prepared for welding b) A molten area formed during welding c) A defect in welding d) A cooling technique in welding 11. Which of the following is a common welding defect? a) Undercut b) Overlap c) Porosity d) All of the above 12. What is the role of a welding helmet? a) To protect the eyes and face from sparks and UV radiation b) To provide extra grip during welding c) To reduce noise during welding d) To maintain a steady arc 13. Which welding process is also known as "stick welding"? a) MIG welding b) TIG welding c) Shielded Metal Arc Welding (SMAW) d) Gas Metal Arc Welding (GMAW) 14. What is a common filler material in welding? a) Nickel b) Iron c) Steel wire d) All of the above 15. What type of welding uses a laser as the heat source? a) MIG welding b) TIG welding c) Laser Beam Welding (LBW) d) Plasma Arc Welding (PAW) 16. What is the advantage of robotic welding? a) Increased precision b) Reduced production time c) Consistent quality d) All of the above 17. Which welding method is commonly used for joining thin materials? a) TIG welding b) Resistance Spot Welding c) MIG welding d) Submerged Arc Welding 18. What does the term "preheating" mean in welding? a) Heating the electrode before welding b) Heating the base metal before welding c) Heating the filler material before welding d) None of the above 19. Which metal is most commonly welded using Shielded Metal Arc Welding (SMAW)? a) Aluminum b) Stainless steel c) Mild steel d) Copper 20. What is the purpose of post-weld heat treatment? a) To increase welding speed b) To remove slag c) To reduce residual stresses and improve toughness d) To clean the weld | 1. What is the main difference between soldering and brazing? a) The type of joint created b) The materials used c) The temperature of the filler metal d) The equipment used 2. Which temperature range is typically used in soldering? a) Below 200°C b) Below 450°C c) 450°C to 700°C d) Above 700°C 3. What is the purpose of flux in soldering and brazing? a) To strengthen the joint b) To clean and prevent oxidation c) To increase heat conductivity d) To cool the joint quickly 4. Which material is commonly used as a filler metal in soldering? a) Copper b) Tin-lead alloy c) Aluminum d) Stainless steel 5. What is the melting point of brazing filler metals? a) Below 200°C b) Below 450°C c) Above 450°C d) Above 1000°C 6. Which of the following is a common application of soldering? a) Plumbing b) Joining electrical components c) Automotive parts d) Heavy machinery assembly 7. Which gas is commonly used in the brazing process? a) Oxygen b) Acetylene c) Argon d) Carbon dioxide 8. What type of joint is commonly formed during soldering and brazing? a) Butt joint b) Lap joint c) Corner joint d) All of the above 9. What is capillary action in the context of brazing? a) The process of heat transfer b) The flow of molten filler metal into the joint c) The removal of impurities d) The cooling of the joint after brazing 10. What is a disadvantage of soldering compared to welding? a) It creates weaker joints b) It is more expensive c) It requires more complex equipment d) It produces more heat 11. What is a common brazing filler metal for joining copper? a) silver alloy b) Tin alloy c) Aluminum alloy d) Steel alloy 12. Which safety precaution is essential during soldering? a) Wearing insulated gloves b) Using a fume extractor c) Wearing safety goggles d) All of the above 13. What is a key property of a good solder joint? a) A shiny and smooth surface b) Excess solder around the joint c) Minimal use of flux d) uneven filler metal distribution 14. Which tool is commonly used to heat the filler metal in soldering? a) Soldering iron b) Blowtorch c) Heat gun d) Plasma cutter 15. What happens if flux is not used during soldering or brazing? a) The joint becomes stronger b) Oxidation can prevent proper bonding c) The filler metal will melt faster d) The joint will be easier to clean 16. Which soldering process is commonly used in electronics? a) soft soldering b) Hard soldering c) Brazing d) Fusion soldering 17. In brazing, what determines the strength of the joint? a) The thickness of the filler metal b) The melting temperature of the filler metal c) The overlap of the base metals d) The speed of cooling 18. What is a disadvantage of brazing compared to welding? a) It requires higher temperatures b) It is more time-consuming c) It provides lower joint strength d) It cannot join dissimilar metals 19. What does lead-free solder primarily consist of? a) Copper and silver b) Tin and copper c) Lead and zinc d) Aluminum and nickel |

- Limited Response Questions

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) |
| --- | --- | --- |
| Learning unit 01-03 | Learning unit 04-06 | Learning unit 07-08 |
| - Types of welding joints? - What is engineering drawing? - What is the overlap joint? - What is a power tool? - What are the sharp tools? - What is acetylene cylinder color? - How to make a neutral flame? - What is oxidizing flame? - What means correct size of the nozzle? - What means by inner cone in flame? | - What is 1F? - What is 2F? - What is 3F? - Types of gas welding torch? - What means by welding operations? - What mean by soldering? - What mean by brazing? - Types of flux in brazing? - How to start gas welding torch? - What mean by mild steel? | - What is visual welding defect? - What mean by AC and DC supply? - What is polarity? - What is corner joint? - What is permanent and temporary welding? - What mean by welding procedure specification (WPS)? - What is difference between electrode and filler wire? - What mean by FCAW? - How many welding defects? - What mean by PPE? |

- Assessment Tasks

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) |
| --- | --- | --- |
| Learning unit o1-03 | Learning unit 04-06 | Learning unite 07-08 |
| Interpret and Take Steps before start welding. Performance Criteria Performance Criteria - Interpret Engineering drawings - Interpret Welding symbols - Identify Hand and power tools - Identify Sizes electrods - Arrange Power tools as per sequence of operations - Adjust pressure of both the gas cylinders - Use ppe - Set welding torch - Set welding flame | Perform Oxy-Acetylene Welding, Gas Welding on Mild Steel Plates (1F, 2F, 3F), and Soldering & Brazing Operations Performance Criteria Performance Criteria - Set welding torch and nozzle - Make gas welding flame - Perform welding in 1F position - Perform welding in 2F position - Perform welding in 3F position - Perform Soldring and brazing - Make lape joint - Make a tee joint | Perform Forged Welding (FCAW) Performance Criteria Performance Criteria - Cut metal sheet - File workpiece before welding - Clean weld with Steel Wire brush - Set FCAW welding machine - Performe welding root pass,filing pass, caping pass - Set furance temperature - Use hamer and make shape |