Course / Subject Name: NQ-Mech-246. Workshop Practice-II (C) Perform Advance Welding

- MCQs

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) |
| --- | --- | --- |
| LU 1-4 | LU 5-6 | LU 7-8 |
| - ______ before setting up the oxy-acetylene torch for manual cutting. - Set the torch to a neutral flame - Check the oxygen and acetylene gas pressures - Neutral flame - Clean the work piece - _______ is the correct distance to maintain between the oxy-acetylene torch nozzle and the workpiece during cutting. - 5–10 cm - 0.5–1 cm - 15–20 cm - 1–3 cm - ______ type of oxy-acetylene flame is used for cutting - Reducing flame - Neutral flame - Oxidizing flame - Carburizing flame - ______ before preparing to cut a metal workpiece using an oxy-acetylene torch - Carburizing flame - Set the flame to neutral - Set up the automatic cutting machine - Mark the cutting line on the work piece - ______ factor is crucial when setting the feed rate of an auto-feed cutting machine. A) The thickness of the metal B) The type of cutting gas used C) The angle of the torch nozzle D) The material of the cutting disc - When setting up the automatic disk grinder, ______ must be adjusted to ensure proper cutting. - The guide blade and the feed rate - The work piece size - The oxygen pressure - The flame temperature - ______ is true about the metal cutting techniques using oxy-acetylene and grinding. - Both techniques are primarily used for welding, not cutting - Oxy-acetylene cutting uses heat and oxidation, while grinding uses friction to remove metal - Grinding is used for cutting thick metals, while oxy-acetylene is used for thin metals - Oxy-acetylene is used for shaping metals, while grinding is used for heating - ______ should be identified first when preparing for GMAW welding. - The welding machine’s brand - The welding torch type - The length of the electrode - The welding requirements from technical drawings - When setting up a GMAW machine, ______ should be done according to Standard Operating Procedures (SOP). - Set the polarity as per welding procedure specifications - Set the wire feed speed to the maximum - Adjust the machine without checking the procedure specifications - Use any type of welding wire available - ______ when setting up a welding machine for GMAW. - Ensure that no safety equipment is worn during setup - Adjust the settings based on personal preference, not WPS - Use any gas flow settings without checking the procedure - Connect the machine to an independent power supply - ______ before depositing filling passes in GMAW. - Use the same parameters as the root pass to ensure consistency - Use higher voltage for filling passes to increase heat - Adjust parameters to avoid excessive spatter and control penetration - Avoid adjusting parameters to save time - ______ should be done to the GMAW machine after completing the weld in the overhead (4F) position. - Discard the used consumables and clean the machine immediately - Change the welding electrode to a different type for the next weld - Leave the machine powered on until the next session - Store all equipment and consumables in accordance with the organization’s guidelines - ______ is the first step in preparing an SMAW machine before starting welding. - Check the type of shielding gas to be used - Set the welding machine accessories as per job requirements - Adjust the current and voltage settings - Connect the machine to an independent power supply - Using too high of a welding current . - The weld will have too little penetration - The weld pool will become too large and cause spatter - The electrode will melt too slowly, leading to poor fusion - The arc will be too short, causing lack of fusion - Purpose of the root pass in SMAW is ______. - To complete the weld in a single pass - To create a smooth, aesthetic surface finish - To reduce the heat input to the base metal - To ensure proper penetration at the joint's root - when performing SMAW in confined spaces ______ should be followed. - Ensure a proper ventilation to prevent the buildup of hazardous fumes - Wear only welding gloves to avoid burns - Perform welding without a helmet to improve visibility - Work without protective clothing for better comfort - ______ is an advantage of Gas Tungsten Arc Welding (GTAW). - Higher deposition rate compared to GMAW - Suitable for welding thick sections of material - Clean welds with minimal spatter - Can be used without shielding gas - A key hazard associated with GTAW is ______. - Electric shock from the welding machine - Exposure to harmful UV radiation from the welding arc - Fume inhalation from the welding process - All of the above - The role of the shielding gas in GTAW ______. - To increase the welding speed - To protect the weld pool and electrode from contamination - To reduce the heat input during welding - To increase the penetration of the weld | - SAW stand for ______ . - Submerged Arc Welding - Shielded Arc Welding - Structural Arc Welding - Standard Arc Welding - The recommended welding position for performing Submerged Arc Welding on a flat plate ______ . - Overhead position (4F) - Vertical position (3F) - Flat position (1F) - Horizontal position (2F) - ______ is a major advantage of Submerged Arc Welding (SAW). - High electrode consumption - Limited use for thick materials - Use of manual electrode feeding - High deposition rates - ______ position is not typically used for Submerged Arc Welding. - 1F (Flat) - 2F (Horizontal) - 3F (Vertical) - 4F (Overhead) - The purpose of adjusting polarity in welding is ______ . - To control welding temperature - To match the welding machine to the power source - To align the current flow with specifications - To increase arc stability - ______ is a hazard related to SAW. - High UV radiation exposure - Noise pollution - Overheating of equipment - Exposure to harmful fumes - ______ is adjusted to control wire feed speed. - Voltage knob - Current regulator - Wire drive system - Polarity switch - In the 6G position, the pipe is oriented ______ . - Vertically - Horizontally - Inclined at a 45° angle - Flat - ______ is the primary principle behind friction welding. - Melting of materials at the joint - Heat generation through friction between materials - Use of an electric arc to fuse materials - Adhesive bonding of materials - In Inertia Friction Welding, one workpiece is mounted on a ______. - Stationary chuck - Rotor-driven chuck - Oscillating chuck - Both A and B - In Linear Friction Welding, the lateral motion chuck provides ______. - Circular motion - Vertical motion - Oscillating motion - Rotational motion - ______ determines the r.p.m. setting in Inertia Friction Welding. - The material of the workpieces - The size of the chuck - The operator’s experience - The safety protocol - ______ is a key safety protocol during friction welding. - Using cooling fluids during welding - Ensuring proper clamping of workpieces - Reducing machine speed to minimum - Stopping the process before plastic forming temperature - ______ generates heat during Linear Friction Welding. - Oscillating motion and pressure - Electric current - Rotational motion and low pressure - Plastic deformation of materials - ______ is the main advantage of friction welding. - High material waste - Low equipment cost - No surface preparation is needed - No external filler material is required - ______ ensures proper weld alignment in friction welding. - Using a jig or fixture - Applying excessive pressure - Increasing the rotational speed - Minimizing surface preparation | - The primary purpose of a visual inspection of a weld is ______ . - To check internal weld defects - To determine the type of filler material used - To measure the thickness of the weld - To check external weld defects like cracks and pores - ______ is commonly used to identify undercuts during a visual inspection. - Ultrasonic probe - Magnifying glass - Vernier caliper - Micrometer - In an X-ray test, a dark spot on the negative film represent ______ . - An area of high density - The filler material - A welding defect - The edge of the weld - The key purpose of producing a negative film during X-ray testing is to ______ . - Highlight surface defects - Enhance the weld's durability - Improve the weld's aesthetic appearance - Detect subsurface or internal defects - The Magna flux test is specifically used to identify ______. - Internal voids - Surface and near-surface defects - Pits and pores - Missed joints - ______ is the primary advantage of non-destructive testing (NDT). - It ensures weld quality without damaging the weld - It increases the welding speed - It reduces the cost of the welding process - It eliminates the need for skilled welders - Weld joint preparation standards are critical for ______ . - Ensuring ease of X-ray testing - Enhancing the speed of visual inspection - Avoiding the need for testing altogether - Minimizing the risk of defects during welding - It important to document defects found during visual inspection ______ . - To provide feedback for corrective actions - To reduce the cost of welding - To simplify Magna flux testing - To avoid conducting X-ray tests - What is the main purpose of a tensile test on a welded joint? - To measure the impact strength - To determine the weld's ultimate tensile strength - To check the weld’s magnetic properties - To assess the surface finish - What is recorded during a tensile test? - Magnetic flux and thermal conductivity - Impact strength and yield point - Weld hardness and porosity - Ultimate tensile strength and Poisson’s ratio - ______ is crucial during destructive welding tests. - Lead aprons - Radiation shields - Protective goggles and gloves - Heat-resistant boots - ______ is measured in Poisson’s ratio. - Axial and lateral strain - Shear strain - Thermal strain - Volumetric strain - ______ is the primary purpose of destructive welding tests. - To improve weld aesthetics - To assess mechanical properties of the weld - To clean the weld joint - To identify subsurface defects - The Izod test measures ______ . - Resistance to bending - Resistance to impact forces - Resistance to heat - Resistance to elongation - Difference between tensile and impact testing is ______ . - The type of machine used - The size of the test specimen - The properties being measured - The materials being tested |

- Limited Response Questions

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) |
| --- | --- | --- |
| LU 1-4 | LU 5-6 | LU 7-8 |
| - What is the purpose of oxidizing flame in oxyacetylene cutting?. - What should be considered when setting the r.p.m. of the disk grinder? - How does an Auto Feed machine improve the cutting process compared to manual cutting? - Why is it necessary to maintain safety precautions when performing oxyacetylene cutting and grinding? - What parameters must be adjusted for GMAW welding to produce a proper fillet weld? - What is the difference between a fillet weld and a groove weld in GMAW? - What is the function of the gas shielding in GMAW? - What parameters need to be adjusted in SMAW to create a proper fillet weld? - Why is maintaining the correct gap between the electrode and base metal important in SMAW? - Why is visual inspection crucial after performing SMAW? - What should be done after completing a weld in SMAW? - What is the importance of setting up welding accessories and consumables correctly? - What positions are typically used in GTAW? | - What is Submerged Arc Welding (SAW)? - List two advantages of SAW? - What are two safety precautions for SAW? - What does "6G" indicate in welding positions? - What is friction welding? - Name the two types of friction welding which are discussed? - Why is pressure increased during the final stage of friction welding? - What safety protocols are critical during friction welding? - How does Linear Friction Welding differ from Inertia Friction Welding? - What materials can be welded using friction welding? - What is inspected during the quality control of friction welds? | - What is the purpose of visual inspection in welding? - What type of defects can be detected by visual inspection? - What is the primary purpose of X-ray testing in welding? - What safety equipment is essential when performing X-ray testing? - Why is it important to prepare the weld joint according to standards before testing? - What is the main objective of tensile testing? - What is Poisson’s ratio? - What safety measures should be taken during destructive tests? - Why are destructive tests performed on welds? - What is the purpose of proper joint preparation before destructive tests? - What happens when the weld exceeds its tensile strength? |

- Assessment Tasks

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) |
| --- | --- | --- |
| LU 1-4 | LU 5-6 | LU 7-8 |
| Cut 2 M.S plates size (1/8x1x4 inches) with Gas cutting manual and auto. Perform GMAW, GTAW and GMAW welding in Flat(1F,1G) Horizontal (2F,2G) Vertical (3F,3G) positions Performance criteria: - Take Work piece according as per drawing for cutting metal by Gas manually and automatically - Set flame and start cutting. Complete cut as per standard for manual cut - Set cutting torch in Auto Feed machine - In Auto Feed Machine Set Feed Rate of machine - Cut on marked line with disk grinder by hand for manual Cutting - Set guide blade in grinding machine for automatic cutting set feed of disk grinding machine and cut on marked line. - Set up welding machine accessories as per Standard Operating Procedure (SOP) for GMAW, SMAW and GTAW welding - Set polarity indicated in the welding procedure specifications, Adjust welding parameters. - Weld the Plates As (GMAW), (SMAW) and (GTAW) makes Grooves in late (1F,1G) Horizontal (2F,2G) Vertical (3F,3G) positions - Adopt manufacturing codes and standards for acceptance criteria of visual welding defects | Task 1: Weld two MS rod by Submerge welding Task 2: Weld two MS rod by Friction Welding Performance criteria: Task:1 - Set SAW welding machine accordance with welding Procedures, specifications and Parameters. - Set polarity indicated in the welding procedure specifications - Deposit and check root passes, filling passes and capping passes for visual inspection as per welding procedure specifications/job requirements follow PPEs - Weld specific MS Plate in 1F and 1G position Task 2 Method A: - Prepare both work pieces smooth and square - Mount one piece in stationary chuck And the other on driven chuck to rotate. - Apply Low and high pressure to generate plasticity and high-pressure force to the stationary work piece to generate high friction force for high temperature. - Apply pressure until the plastic forming temperature is achieved. - Stop rotor and increase pressure force until the whole weld is formed. Method B: - One piece on lateral/ oscillating motion chuck and other stationary - Apply high pressure force to the stationary work piece to produce friction for high temperature, plasticity to perform weld. | Task 1: Perform Non- Destructive Welding test Task 2: Perform Destructive Welding Tests . Performance criteria: Task 1: - Prepare weld joint for Magna flux Test as per standard for X-Ray test - Prepare and Place work piece in x-ray machine - Expose welded area to x-ray beam - Produce negative film identifying defects as Check for cracks, pits, pores, under cut and missed joint - Sprinkle very small iron particles on welded joint - Slide magnet underneath the welded joint Mark area where iron particles are gathered Task 2: Destructive welding - Prepare weld joint as per standard for Tensile Test - Place work piece on (UTM) - Mount and start UTM operation. - Note ultimate tensile strength of welded joint - Note Poisson’s ratio of welded joint V= Lateral strain/Linear Strain Test 2: - Prepare weld joint as per standard for Impact Test - Set machine for Izod test method as per standard - Prepare and Mount work piece for Izod test method as per standard - Strike test piece with striker note test readings |