Course / Subject Name NQ-Mech-333 Machine Design and Analysis

- MCQs

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) |
| --- | --- | --- |
| Learning Units 01-04 | Learning Units 05-08 | Learning Units 09-11 |
| - What is the primary purpose of machine design? - Enhance aesthetics - Ensure safety and functionality - Reduce manufacturing cost - Improve sales - Which factor is not a design consideration in machine design? - Material properties - Manufacturing methods - Weather forecast - Load requirements - What is the first step in the general procedure of machine design? - Selection of materials - Problem identification - Testing the prototype - Finalizing dimensions - In the SI system, which is a derived unit? - Meter - Kilogram - Pascal - Second - What is tensile stress? - Stress due to twisting forces - Stress causing elongation - Stress causing compression - Stress due to heat - Which property is represented by the slope of the elastic region in a stress-strain diagram? - Poisson's Ratio - Modulus of Rigidity - Factor of Safety - Young’s Modulus - What is Poisson’s Ratio? - Ratio of stress to strain - Ratio of lateral strain to axial strain - Ratio of compressive to tensile strength - Ratio of shear to volumetric strain - What is the Factor of Safety (FoS) used for? - Ensuring safety under applied loads - Increasing material strength - Calculating strain percentage - Reducing stress levels - What is the primary application of pressure vessels? - Storing solids - Generating electricity - Storing fluids under pressure - Measuring temperature - In a thin cylindrical shell under internal pressure, hoop stress acts: - Along the length of the cylinder - Perpendicular to the length - Diagonally across the cylinder - Along the radius - Which formula is used to calculate the thickness of a thin cylindrical shell under internal pressure? - Lame's Equation - Thickness = (Pressure × Diameter) / (2 × Stress) - Thickness = (Diameter × Stress) / (2 × Pressure) - Thickness = Pressure × Radius × Stress - What type of stress is equal in all directions for a thin spherical shell under internal pressure? - Hoop stress - Longitudinal stress - Shear stress - Volumetric stress - Which of the following is an advantage of welding joints? - Reduced strength - Permanent joint - Increased material cost - Limited applications - What type of welded joint is used when two plates are joined edge-to-edge? - Lap joint - Butt joint - T-joint - Corner joint - What stress acts parallel to the weld in a parallel fillet weld under static loading? - Shear stress - Tensile stress - Compressive stress - Bending stress - In a transverse fillet weld under static loading, the strength of the weld depends on: - Weld length, plate width, and tensile stress - Plate thickness, weld length, and shear stress - Weld material only - Plate thickness and bending stress | - What is a primary application of screw joints? - Temporary joining of components - Decorative purposes - Insulating materials - Reducing weight - Which term describes the distance a screw advances axially in one complete turn? - Pitch - Lead - Root diameter - Crest diameter - What type of stress develops in a screw due to external forces under static loading? - Bending stress - Shear stress - Tensile stress - Compressive stress - What is the purpose of a sunk key? - Reduce weight - Transfer torque from shaft to hub - Insulate electrical connections - Prevent axial movement of the shaft - What is a major disadvantage of screw joints? - Permanent assembly - Prone to loosening under vibrations - High initial cost - Difficult to manufacture - Which term refers to the angle between the flanks of a thread? - Pitch angle - Helix angle - Thread angle - Lead angle - What type of key is commonly used for light loads? - Saddle key - Sunk key - Woodruff key - Round key - In a screw joint, initial tightening stress is caused by: - External forces - Torque applied during screwing - Material deformation - Impact loads - What is the primary purpose of a shaft in machinery? - To transmit power and torque - To reduce vibration - To support structural loads - To insulate components - Which material property is most important for a shaft? - High thermal conductivity - High tensile strength - Low density - High electrical resistance - Which type of shaft is generally lighter but can handle similar loads as a solid shaft? - Solid shaft - Hollow shaft - Stepped shaft - Flexible shaft - What is a flange coupling used for? - To transmit torque between shafts - To absorb shock loads - To align misaligned shafts - To reduce rotational speed - What is the primary purpose of belt and pulley drives? - Transmitting power between shafts - Storing mechanical energy - Reducing machine weight - Increasing electrical efficiency - Which type of belt is suitable for high-speed applications with minimal slip? - Flat belt - V-belt - Round belt - Timing belt - What happens when there is slip in a belt drive? - Reduced power transmission efficiency - Increased velocity ratio - Decreased belt tension - Enhanced torque | - The deflection of a helical spring depends on: - Material density and wire length - Applied load, spring material, and coil dimensions - Coil shape and color - Wire temperature and surface finish - In a helical spring, shear stress is highest at: - The center of the wire - The midpoint of the spring - The ends of the spring - The surface of the wire - What material is most commonly used for helical springs? - Aluminum - Brass - Carbon steel - Copper - Which type of spring is commonly used in shock absorbers? - Leaf spring - Helical spring - Torsion spring - Disc spring - What is the primary function of a spring in machinery? - To absorb energy and provide cushioning - To increase rotational speed - To reduce friction between components - To transmit power - Which type of spring is commonly used in shock absorbers? - Leaf spring - Helical spring - Torsion spring - Disc spring - What material is most commonly used for helical springs? - Carbon steel - Aluminum - Brass - Copper - In a helical spring, shear stress is highest at: - The center of the wire - The midpoint of the spring - The ends of the spring - The surface of the wire - The deflection of a helical spring depends on: - Material density and wire length - Coil shape and color - Wire temperature and surface finish - Applied load, spring material, and coil dimensions - What is the primary function of a bearing in machinery? - To reduce friction between moving parts - To transmit torque between shafts - To store mechanical energy - To increase power efficiency - Which type of bearing is used to support axial loads? - Ball bearing - Thrust bearing - Roller bearing - Journal bearing - Bearings are classified based on the nature of contact as: - Rolling and sliding contact bearings - Elastic and inelastic contact bearings - Axial and radial contact bearings - Load and speed-based bearings - What term describes the layer of lubricant between a bearing and shaft? - Bearing modulus - Lubrication film - Friction layer - Pressure layer - What factor is essential when selecting a journal bearing? - Color of the bearing - Type of shaft material - Weight of the machine - Speed of the shaft and viscosity of the lubricant - How is the velocity ratio in a belt drive calculated? - By dividing the diameters of the pulleys - By multiplying belt length with pulley diameter - By dividing the speeds of the driver and driven pulleys - By adding slip and creep values |

- Limited Response Questions

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) |
| --- | --- | --- |
| Learning Units 01-04 | Learning Units 05-08 | Learning Units 09-11 |
| - What is the first step in the design process? - Why is it important to identify design constraints? - What type of drawings are typically created for a design? - What is included in a final design document? - How do you calculate tensile stress in a material? - What is compressive stress and how is it calculated? - How do you calculate shear stress in a material? - What is strain in the context of material design? - What is the significance of modulus of elasticity? - How do you calculate the percentage elongation of a material? - What does the factor of safety indicate in design? - What are the different types of pressure vessels? - How do you calculate hoop stress in a cylindrical shell? - What is the cause of transverse failure in pressure vessels? - How do you calculate the shear stress in a parallel fillet weld? | - What is the stress area of a screw? - Why is it important to fix screw tightening torques? - How does an external load affect bolts and screws under static conditions? - What are the different types of keys used in mechanical design? - How do you determine the optimal key size for a shaft? - How do you calculate the width and thickness of a key based on shaft and pulley size? - How do you calculate the torque transmitted by rectangular and square keys? - What is the most important factor when selecting material for shafts and couplings? - How do you design the optimal length and diameter of a shaft for an application? - What factors do you consider when designing a shaft profile? - How do you interpret tolerances and fits for assembly and operation? - What is needed to ensure efficient torque transmission between shafts? - How do you calculate the required power transmission capacity of a belt drive? - How do you determine the speed ratio between driver and driven shafts in a belt drive? - What factors do you consider when selecting a belt type and its material? | - How do you calculate the maximum load a spring can bear? - What formula do you use to calculate the stress in a spring? - How is the stiffness of a spring determined? - What factors help in selecting the suitable wire diameter for a spring? - How do you choose the material for a spring wire? - What formula is used to calculate the number of coils in a spring? - How do you determine the end type of a spring? - What is the fatigue limit, and how do you calculate it? - How do you calculate the endurance limit for a spring? - How is the magnitude of loads on a bearing calculated? - What factors are considered when selecting the type, material, size, and geometry of a bearing? - How do you choose the appropriate lubricant for a bearing? - How do you calculate the bearing modulus? - What is the method to calculate the coefficient of friction for journal bearings? - How do you calculate the optimal cam profile, size, and shape for a cam mechanism? |

- Assessment Tasks

| Formative Assessment (Part A) | Formative Assessment (Part B) | Formative Assessment (Part C) |
| --- | --- | --- |
| Learning Units 01-04 | Learning Units 05-08 | Learning Units 09-11 |
| Illustrate the design methodology, calculate simple stresses in machine parts, design pressure vessels and design welded joints. Performance Certiria: - Classification of machine design - Design consideration factors - General procedure in design - Fundamental Units and derived units - Visit to machine shop and explain/ identify basic principles construction of any machine, and make brief dialogue/debates between the groups - Calculate (tensile, compressive and shear), stress and strain, modulus of elasticity, %age elongation, %age reduction in area, factor of safety for simple machine parts - Calculate thickness and diameter of thin cylinders for hoop and longitudinal stresses - Make butt joint by using various thickness (<5mm,) of plates/pieces | Design screwed joints, keys, shafts, and couplings, and perform the selection of belt drives. Performance Certiria: - Calculate stresses setup due to initial tightening and external load on screws. - Check dimensions of square and rectangular keys due to failure in shearing and crushing. - Check dimensions of solid and hollow shaft. - Design solid and hallow shaft subjected to twisting moment only. - Visit to any machine shop and practically identify/show belt and pulley drive in machine to every student and make brief debates - Solve problem to find out the speed of shaft considering the diameters of flat pulleys and slip between belt and flat pulleys | Design springs, perform the selection of bearings, and design cams and followers. Performance Certiria: - Visit to any machine shop and practically identify/show spring application in machine to every student and gives completes guidance on springs - Suggest suitable journal bearing, considering the load on shaft, speed, viscosity of lubricant, bearing pressure, coefficient of friction and bearing modulus. - Divide students into pairs or small groups - Show examples of different types of CAM and Followers in workshop - Challenge each group to design and build a simple CAM and Follower from card paper/cardboard |