National Competency Standard of Level 5 Diploma of Associate Engineer

In

Mechanical Technology

2024

National Vocational and Technical Training Commission (NAVTTC),

Government of Pakistan

Table of Contents

1st Year DAE Mechanical Technology	3

Mech-127. Workshop Practice-I (A). Perform General Metal Work	4

Mech-127. Workshop Practice-I (B). Perform Wood Work & Make Pattern	8

Mech-127. Workshop Practice-I (C). Perform Welding and Forging	13

Mech-127. Workshop Practice-I (D) Perform Basic Machining-I	21

Mech-151. Occupational Health, Safety and Environment	28

Comp-152.  Computer Applications	31

Mech-173 Engineering Drawing & Graphics	34

2nd Year DAE Mechanical Technology	39

Elect-212. Applied Electricity and Electronics	40

Mech-233. Computer Aided Designing (CAD)	43

Mech-246. Workshop Practice-II (A). Perform Basic Machining-II	48

Mech-246. Workshop Practice-II (B). Perform Foundry & Casting Works	55

Mech-246. Workshop Practice-II (C). Perform Advance Welding	58

Mech-262. Metallurgy	67

Mech-272. Metrology	71

3rd Year DAE Mechanical Technology	76

Mech-302. Fluid Mechanics & Hydraulics Machines	77

Mech-313. Applied Thermodynamics	80

Mech-321. Industrial Planning and Adopt Production Methods	84

Mech-332. Material Testing and Heat Treatment	93

Mech-333. Machine Design & Analysis	100

Mech-352. CAD/CAM	104

Mech-354. Workshop Practice-III (Manufacture Mechanical Projects)	110

Mech-363. Tool and Mould Design	118

Mech-372. CNC Machines	122

# 1st Year DAE Mechanical Technology

## Mech-127. Workshop Practice-I (A). Perform General Metal Work

Overview

This competency standard covers the skills and knowledge required to Develop Name Plate manually, Prepare Dovetail Joint, Prepare Bottle Opener, Cut Threads on Work Piece with tap and die, Cut Pipe Threads, Prepare spanner (small size) and Prepare Funnel.

| Competency Units | Performance Criteria |
| --- | --- |
| - Develop Name Plate manually | - Arrange marking tools as per job requirement - Clamp sheet in vice. - Cut sheet as per drawing - Perform surface finishing with file - Level surface with tri-square - Mark plate as per name requirements - Punch marked area - Perform finishing with sand paper |
| - Prepare Dovetail Joint | P1. Arrange marking tools as per job requirement P2. Cut sheet as per drawing P3. Perform surface finishing with file P4. Level surface of both work pieces with tri-square P5. Mark both work pieces according to drawing P6. Create outer notch on work piece using flat file and hacksaw P7. Create inner notch using hacksaw and chisel P8. Compare both pieces by inserting outer notch into inner notch P9. Perform finishing with sand paper |
| - Prepare Bottle Opener | P1. Arrange marking tools as per job requirement P2. Cut sheet as per drawing P3. Perform surface finishing with file P4. Level surface with tri-square P5. Mark the center of the radius as per drawing P6. Draw radius as per drawing P7. Make notch with round file P8. Perform finishing with sand paper |
| - Cut Threads on Work Piece with tap and die | P1. Identify taps & die according to requirement P2. Identify work piece clamping method P3. Align tap and die P4. Apply lubricants while threading P5. Take safety measure to avoid unwanted engraving and slips P6. Cut internal threads with Tap. P7. Cut external threads with Die |
| - Cut Pipe Threads | P1. Arrange marking tools as per job requirement P2. Cut pipe as per drawing P3. Select die as per pipe size P5. Set die into die holder P6. Select correct vice for pipe clamping P7. Perform pipe threading and follow safety precautions P8. Perform finishing with sand paper |
| - Prepare spanner (small size) | P1. Arrange marking tools as per job requirement P2. Cut sheet as per drawing P3. Perform surface finishing with file P4. Level the surface with tri-square P5. Mark center of the radius as per drawing P6. Draw radius as per drawing P7. Make notch with round file P8. Perform finishing with sand paper |
| - Prepare Funnel | P1. Arrange marking tools as per job requirement P2. Cut sheet as per drawing P3. Perform surface finishing with file P5. Mark sheet according to drawing P6. Cut sheet with hand shear P7. Create radius of funnel using correct tools P8. Perform flat lock seam bend using bench vice P9. Perform finishing with sand paper. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Define basic measurement

- Describe basic measuring /Marking /cutting tools

- Describe clamping/holding methods and devices.

- Describe types of chisels

- Understanding of chiseling process

- Understanding of types of fits.

- Knowledge of radius gauge

- Knowledge and use of files.

- Knowledge of manual threading.

- Knowledge of different kinds of taps & dies.

- Knowledge of calculation for drill size for internal threading.

- Knowledge of standard bolts.

- Understanding proper use of hand shear.

- Knowledge of surface development of cone.

- knowledge of flat lock seam end in metal sheet working

Critical Evidence:

- Identify required tools for a specific task.

- Interpret drawings.

- Cut internal and external threads with Tap & Die

- Cut metal according to the drawing.

Tools & Equipment

- Work bench

- Bench vices

- Hammer

- Tri-square

- Hand hacksaw

- Scriber

- Vernier caliper

- Flat File

- Number/alphabet punch

- Round file

- Metal working chisel

- Punching tools

- Tap set

- Die set

- Pipe vice

## Mech-127. Workshop Practice-I (B). Perform Wood Work & Make Pattern

Overview:

This competency standard covers the skills and knowledge required to identify type of timber, estimate timber quantity, identify tools used in carpentry, sharpen carpentry cutting tools, make butt joints, Perform Biscuit joinery, Make Miter joints, Make Edge joint, Make Dovetail joint, Make Mortise and tenon joint.

| Competency Units | Performance Criteria |
| --- | --- |
| - Identify Type of timber | - Recognize color and odor of locally produced timber. - Prepare list of locally produced timber in accordance with color and odor. - Identify size and numbers of knots in timber |
| - Calculate timber quantity | - Prepare drawing of the product - Identify components of the product - Recognize timber requirement for specific components of product. - Calculate volume of the timber |
| - Identify tools used in carpentry | - Identify measuring, marking and cutting tools - Identify hand, power and pneumatic tools - Identify PPEs used in wood work shop |
| - Sharp carpentry Cutting tools | - Identify blunt tools to be sharpen - Perform plane iron sharpening - Perform chisel sharpening - Perform scraper sharpening - Perform router bit sharpening - Perform craving tool sharpening |
| - Make butt joints | - Identify butt joint - Cut timber according to requirement of butt joint - Join cut pieces according to requirement |
| - Perform Biscuit joinery | - Identify biscuit joint - Cut timber according to requirement of biscuit joint - Join cut pieces according to requirement |
| - Make Miter joints | - Identify Miter joints - Cut timber according to requirement of Miter joints - Join cut pieces according to requirement |
| - Make Edge joint | - Identify Edge joint - Cut timber according to requirement of Edge joint - Join cut pieces according to requirement |
| - Make Dovetail joint | - Identify Dovetail joint - Cut timber according to requirement of Dovetail joint - Join cut pieces according to requirement |
| - Make Mortise and tenon joint | - Identify Mortise and tenon joint - Cut timber according to requirement of Mortise and tenon joint - Join cut pieces according to requirement |
| - Make Dado joint | - Identify Dado joint - Cut timber according to requirement of Dado joint - Join cut pieces according to requirement |
| - Drill holes in wood job | - Observe personal and work place safety. - Set up drilling machine for producing holes according to job requirement. - Operate machine tool controls safely and correctly in line with operational procedures. - Produce components to the required quality and within the specified dimensional accuracy. - Shut down the machine safely after the completion of the machining activity. |
| - Perform fastenings | - Select correct fastener - Select correct tools for fastenings - Drill holes on required product - Apply fastener to the product |
| - Perform skin in job surface | - Make surface smooth by using sand paper. - Perform skinning in rubber and apply polish for job color and clarity - Perform skin in surface covering small areas at a time, using correct amount of pressure and suitable rubber movements - Apply pumice powder as necessary in accordance with workplace procedure. - Inspect skinned in surface |
| - Perform body up on skinned in surface | - Mix polish, lubricating oil and pumice powder for body up the skinned in surface, - Apply correct amount of pressure and suitable rubber movements - Monitor rubber and job surface continuously - Recharge body up rubber and lubricating oil - Apply pumice as necessary so that surface faults and blemishes do not develop |
| - Perform polishing | - Prepare surface for polishing - Apply spirit on surface using correct amount of pressure and suitable rubber movements - Observe blemishes or rubber marks - Apply polish on job surface - Apply Spirit on surface |
| - Make Patterns for Foundry works | - Prepare layout by adding allowances (draft, machining, shrinkage, shaking) - Arrange 60 mm square deodar wood piece with 160 mm length. - Face outer end by holding wooden piece chuck. - Turn outer diameter 50mm with shrinkage allowance. - Turn outer diameter 25mm and into length 25mm for core print. - Repeat procedure by changing direction of pattern. - Prepare another core print diameter. - Remove pattern from lathe machine and mark parting line. - Cut pattern and fit dowel pin holes. - Assemble pattern symmetrically. - Make 3-piece Pattern |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Define wood.

- Explain constructional wood (Timber) with sketch.

- Describe procedure for selection of timber.

- Explain planning for product construction.

- Explain procedure for calculation of timber quantity.

- Knowledge and safe use of hand, power and pneumatic carpentry tools.

- Explain procedure for sharpening wood cutting tools.

- Describe procedure for cutting of timber

- Explain types of wood joints.

- Explain procedure of making butt joint, biscuit joint, miter joint, edge joint, dove tail joint and tenon and mortise joint.

- Knowledge of drill bits used in wood work.

- Explain procedure for drilling hole in wood.

- Knowledge of different types of fasteners used in wood work.

- Knowledge of fastening tools.

- Explain procedure of different fastening processes.

- Define wood polishing.

- Explain procedure for polishing.

Critical Evidences Required:

The candidate needs to produce the following critical evidences in order to be competent.

- Identify tools used in carpentry.

- Identify type of timber.

- Estimate timber quantity.

- Drill hole in wood.

- Make Dado, Butt and Dove Tail Joints.

- Identify fastenings.

- Polish wood surface.

- Make pattern for foundry work.

List of tools and Equipment:

1. Hand Tools

  - Saws. Hand Saw, Back Saw, Jigsaw

- Planes. Jack Plane, Block Plane, Bench Plane

- Chisels. Bevel-Edge Chisel, Mortise Chisel

- Hammers and Mallets. Claw Hammer, Wood Mallet

- Screwdrivers. Flathead and Phillips Screwdrivers

- Measuring and Marking Tools. Tape Measure, Square, Calipers, Marking Gauge, Scriber

- Files and Rasps. Half-Round File, Rasp

2. Power Tools

Circular Saw, Table Saw, Band Saw, Jigsaw, Router, Drill and Drill Press, Belt Sander, Planer, Joiner, Dust Collector

3. Workshop Equipment

Workbench, Vise, Clamps, Tool Storage

4. Safety Equipment

Safety Glasses, Hearing Protection, Dust Mask or Respirator, Work Gloves, First Aid Kit

5. Miscellaneous Tools

Clamping Devices, Saw Blades, Router Bits

## Mech-127. Workshop Practice-I (C). Perform Welding and Forging

Overview:

This competency standard covers the skills and knowledge required to Interpret Drawing and Execute to Fabricate Parts, Apply Hand and Power Tools, Perform Pre-Welding Operations, Perform Oxy Acetylene Welding, Perform Gas Welding on Mild Steel Plates(1F,2F,3F), Perform Soldering and Brazing Operation, Perform Flux Cored Arc Welding (FCAW) and Perform Forged Welding

| Competency Units | Performance Criteria |
| --- | --- |
| - Interpret Drawing and Execute to Fabricate Parts | - Recognize basic lines used in engineering drawings - Distinguish Orthographic and Isometric views of a drawing - Identify manufacturing requirements according to drawings - Prepare job layout according to manufacturing requirements - Draw basic and supplementary welding symbols used in manufacturing drawings - Draw welded joints - Identify welding requirements according to welding symbols given in the manufacturing drawings - Identify material specifications according to drawing - Identify bill of material (BOM) according to drawing - Interpret dimensional tolerances according to drawing - Assemble and tag weld parts according to drawing |
| - Apply Hand and Power Tools | - Select correct hand and power tools as per task requirements. - Check specifications which may include finish, size or shape. - Adopt safety requirements before, during and after use. - Mark unsafe or faulty tools for repair according to designated procedures before, during and after use. - Sharpe Tools according to standard operational procedures. - Store hand tools according to standard operational procedures - Arrange power tools as per sequence of operations like clamping, alignment and adjustment |
| - Take steps before Gas Welding | - Adjust pressure of both the gas cylinders with the help of regulator - Open acetylene gas knob of welding torch Make carburizing flame by increasing acetylene gas quantity - Make neutral flame by adjusting both gases at same quantity - Make oxidizing flame by increasing oxygen gas quantity - Select correct size of the nozzle - Set both gas flame of welding torch as per standard - Obtain required material/s as per job requirements - Select marking tools as per job requirements - Mark area to be cut as per drawing/job requirements |
| - Carryout Oxy Acetylene Welding | - Take Work piece as per drawing for T Joint, Lap joint - Straiten it with the help of hammer and anvil - Grind work pieces on grinding machine to prepare the edges flat and parallel to each other - Place Bottom piece on work table and place the Top plate at 900 to each other as per drawing for T joint - Grind work pieces on grinding machine to prepare the edges flat and parallel to each other for lap joint - Set flame of welding torch as per standard - Complete bead as per standard Take Work piece as per drawing - Place Bottom piece on work table and place the Top plate along the marked line - Set flame of welding torch as per standard - Complete bead as per standard - Take Work piece as per drawing for Butt Joint - Place work pieces parallel to each other - Set flame of welding torch as per standard - Complete bead as per standard |
| - Carryout Gas Welding on Mild Steel Plates(1F,2F,3F) | - Open gas cylinder with the help of cylinder key - Adjust pressure of both gas cylinders with the help of regulator - Open acetylene gas knob of welding torch - Ignite acetylene gas with help of spark lighter - Open oxygen gas knob of welding torch - Set work piece as per standard - Perform fore hand welding method(1F) - Perform welding in right hand welding method(2F) - Ignite acetylene gas with help of spark lighter - Open oxygen gas knob of welding torch - Perform welding in downward welding method(3F) - Perform post welding operations |
| - Perform Soldering and Brazing Operation | - Perform marking as per drawing for Soldering - Cut metal sheet according to drawing using shearing machine - Straighten material with help of hammer - File the workpiece before soldering and brazing - Perform soldering operation as per standard - Perform marking as per drawing for brazing - Cut metal sheet according to drawing using shearing machine - Straighten material with help of hammer - File the joints after soldering and brazing - Open gas cylinder with the help of cylinder key - Adjust pressure of both gas cylinders with the help of regulator - Select correct size of the nozzle - Set flame to carburizing flame as per standard - Apply copper filler rod as filler metal - Perform brazing as per standard |
| - Perform Flux Cored Arc Welding (FCAW) | - Identify welding requirements from the technical drawings - Prepare FCAW welding machine in accordance with welding procedure specifications - Set up welding machine accessories and consumables as per job requirements, welding procedure specifications and/or manufacturer instructions - Connect welding machine to an independent power supply - Set polarity indicated in the welding procedure specifications - Adjust welding parameters (current, voltage, wire feed speed etc.) as per welding procedure specifications/job requirements to make fillet weld on steel plate 1F,1G,2F,2G Maintain gap between electrode and base metal as per standard practices - Carry out welding in in Flat (1F) and Horizontal (2F) Positions following standard procedures - Follow applicable manufacturing codes and standards for acceptance criteria of visual welding defects - Adjust welding parameters (current, voltage, wire feed speed etc.) as per welding procedure specifications/job requirements to make Groove weld on Steel Plate - Maintain gap between electrode and base metal as per standard practices - Carry out welding in Flat (1G) and Horizontal (2G) Positions following standard procedures - Deposit root pass as per welding procedure specifications - Deposit filling passes as per welding procedure specifications - Deposit capping pass as per welding procedure specifications - Check root, filling and capping passes for any visual discontinuities as per acceptance standards - Follow applicable manufacturing codes and standards for acceptance criteria of visual welding defects - Adjust welding parameters (current, voltage, wire feed speed etc.) as per welding procedure specifications/job requirements to make fillet weld on Carbon steel plate (3F,3G,4F,4G) - Adjust welding parameters (current, voltage, wire feed speed etc.) as per welding procedure specifications/job requirements to make Groove weld on steel plate - Carry out welding in Vertical (3G) and Overhead (4G) positions following standard procedures - Clean work area in accordance with workplace safety practices |
| - Perform Forged Welding | - Identify welding requirements as per technical drawings - Bevel edges of both work pieces to overlap on each other - Remove any oil or grease if necessary - Select coal or gas to fire hearth - Set temperature of hearth as per job material - Perform heating of work piece until it became red hot - Put both work piece on anvil - Strike sledge hammer on work piece as per standard - Put work piece in water tank when both pieces join each other - Carry out finishing work of welds following standard procedures - Inspect weld visually and mark any visual defects, as required - Carry out repair work in accordance with approved procedures, as required - Clean work area in accordance with workplace safety practices - Maintain and store tools/equipment/consumable materials in accordance with organization guidelines |

Knowledge & Understanding

The candidate must possess underpinning knowledge and understanding required to carry out tasks covered in this competency standard. Therefore he/she must be able to:

- Basic elements of engineering drawing

- Drawing symbols

- Dimensioning techniques

- General tolerance

- Applications of different hand and power tools

- Describe about welding torch

- Gas pressure regulators

- Explain temperature and its units

- Describe pre heating

- Describe the filler rod

- Describe electrode baking oven

- Describe purpose of flux

- Explain positions of electrode according to work

- Knowledge of setting the current on welding machine

- Describe motion of electrode in ARC welding

- Explain importance of gap between electrode and base metal

- Describe importance of cleanliness of surface to be welded

- Various flames

- Preparation of oxidizing flame

- Types of torch nozzle

- Manual cutting of metal with oxidizing flame

- Marking of work piece

- Use of regulator to adjust gas pressure

- Forming neutral flame for gas welding

- Fore hand welding method

- Right hand welding method

- Post welding operations

- Downward welding method

- Explain various types of welding processes

- Explain advantages of GMAW

- Explain various welding positions

- Demonstrate the method to correctly wear PPE

- Explain Specifications/ classification of electrode/s required for the job

- Explain Welding techniques as per WPS/instruction sheet

- Describe Welding procedure specifications (WPS)

- Describe Fillet lap joint

- Describe Tee-fillet joint

- Describe Corner joint

- Describe Butt joint

- Describe Double J joint

- Explain Polarity setting according to standard specifications

- Define Visual welding defects

- Describe Welding codes and standards

- Identify various gases/combination of gases for shielding

- Electrical supply AC and DC

- Electrical parameters like (voltage, current etc.) and their effects on weld

- Describe soldering

- Explain disadvantages of soldering

- Describe brazing

- Define carburizing flame

- Define neutral flame

- Define oxidizing flame

Critical Evidence(s) Required.

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Identify welding requirements according to welding symbols given in the manufacturing drawings

- Identify material specifications according to manufacturing drawing

- Adjust pressure of both gas cylinders with the help of regulator

- Make oxidizing flame by increasing oxygen gas quantity

- Set flame of welding torch as per standard

- Make Tee Joint.

- Make Lap Joint

- Make Butt Joint

Tools and Equipment

- Layout tools.

- Measuring devices.

- Hand held calculator.

- Fabrication and welding equipment

- Work bench

- Bench vice

- Hammer

- Tri-square

- Hand hacksaw

- Scriber

- Vernier caliper

- Flat File

- Oxygen cylinder

- Acetylene gas cylinder

- Pressure regulators

- Cylinder key

- Welding torch

- Rubber house pipe

- Back fire arrester

- Flash back arrester

- Spark lighter

- Steel wire brush

- Welding electrodes

- Metal Filler rod

- Welding flux

- Magnifying glass

- Iron particles

## Mech-127. Workshop Practice-I (D) Perform Basic Machining-I

Overview:

This competency standard covers the skills and knowledge required to Perform cutting on Metal Circular/Power Heck Saw, Perform Drilling Machine Operations, Perform general machining, Perform Basic Lathe Machine Operations, Perform Shaper, Planar and Slotter Machining, Perform broaching operation

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Cut Metal with Heck Saw or Circular Saw or Power Saw | - Mark job according to given drawing - Select correct blade according to job requirement - Set blade in frame of hacksaw as per procedure - Clamp work piece firmly as per set standards - Check blade tightness of Power Saw. - Follow marked line during sawing to ensure accuracy. - Carry out Sawing at different angles |
| CU2. Perform Drilling Machine Operations | - Observe personal and work place safety - Set up drilling machine for producing holes according to job requirement. - Manipulate machine tool controls safely and correctly in line with operational procedures. - Produce components within specified dimensional accuracy. - Shut down machine safely after completion of machining activity. - Select relevant tools according to information given in drawings and job specifications. - Check tooling is correct in terms of size, shape, type, and grade for the work. - Position work-piece in the drill that it is aligned, secured and stable during drilling. - Adjust speeds and feeds of drill in accordance with size, type and hardness of work-piece material, so that drill performs optimum cutting without damage to work-piece. - Perform Counter Boring and Counter sinking - Perform Machine reaming - Clamp work-piece in the vice properly. - Select reamer according to hole size and drawing requirements - Set reamer in the drill chuck according to procedure. - Provide lubricants during reaming for smooth cutting. |
| CU.3 Perform Basic Lathe Machine Operations | - Select tools according to job requirement for centering, facing, Turning, Drilling, Step Turning, Knurling, internal and external threading operation - Mount and set the required work-holding devices, work piece and cutting tools. - Follow correct specifications for the part or component to be produced. - Select safe procedures and tools to accomplish the work. - Adjust operating parameters (speed and feed, rpm) of machine tool for centering the job. - Check all safety mechanisms are in place. - Perform Taper turning by tail stock - Select Tools for Taper turning by tail stock off-set method - Loosen tailstock clamp out. - Offset tailstock required amount. - Centre cutting tool. - Setup cutting tool for parallel turning. - Starting at small diameter take excessive cuts until the taper is .05 to .06 in oversize. - Check taper for accuracy using a taper ring gauge. - Finish turns the taper to size and fit required. - Set degree by swiveling the compound rest - Perform Taper turning by swiveling the compound rest. - Remove binding screw that cross slide to cross feed screw nut. - Tighten lock screw and set cutting tool on center. - Set work piece in lathe and mark length of taper. - Connect sliding block to side of taper attachment by binding screw. - Select depth of feed cut by compound rest feed handle. - Take light cut and recheck taper fit. - Perform Taper turning by telescopic taper turning attachment - Select Tools for Taper turning by telescopic taper turning attachment - Clean and oil the guide bar. - Loose lock screws and offset end of guide bar, - Set bar to required taper in degrees. - Tighten lock screw and set cutting tool on center. - Set workpiece in lathe and mark length of taper and tighten connecting screw on sliding block. - Move carriage until center of attachment is opposite to length of taper. - Lock anchor bracket to lathe bed. - Take cut and select depth of cut. |
| CU4. Perform Shaper, Planar and Slotter Machining | - Produce squared shape work piece, V shaped work piece, Machining a Rack Gear, T-slot Machining, Machining Irregular Surfaces, Machining External Keyways, Machining internal Keyways - Identify safety hazards related with shaping operations and take steps to avoid any injury or accident. - Dial machine vice according to job requirement. - Select point cutting tool and set machine as per requirements. - Mount cutting tool and work piece in the machine. - Shut down machine at safe position after finishing the work |
| CU5. Perform broaching operation | - Perform Machining of Irregular Surfaces, make keyways, Make internal gears - Identify safety hazards related with shaping operations and take steps to avoid any injury or accident. - Dial machine vice according to job requirement. - Select point cutting tool for machining of irregular surfaces Broaching tool for keyways and internal gears. - Set machine according to job requirements. - Mount cutting tool and work piece in the machine. - Adjust feed and speed for cutting at different points according to given drawing - Shut down machine in safe position after finishing the work |

Knowledge & Understanding

The candidate must possess underpinning knowledge and understanding required to carry out tasks covered in this competency standard. Therefore he/she must be able to:

- Define methods and techniques of sawing

- Describe basic measurement

- Describe basic measurement

- Describe types of hacksaw frames

- Describe basic measuring /Marking /cutting tools

- Describe clamping/holding methods

- Methods and techniques of mounting and setting of work-piece.

- Methods and techniques of adjusting operating parameters of machine tool.

- Procedure of adjusting speed and feed.

- Calculation of speed and feed.

- Use of holding and cutting tools

- Reading and interpreting work specifications, drawings and sketches.

- Method and technique of setting up and adjusting the machine.

- Techniques to check quality of component produced.

- Procedure of shutting down of machine and equipment after closure of activities.

- Safety precautions and procedures need to be observed during work.

- Types of drilling or boring tools and their function.

- Procedure of mounting and setting up of work-holding devices, work pieces and cutting tools.

- Method and technique of adjusting RPM of lathe machine.

- Safe boring procedures.

- Techniques of checking quality of components.

- Calculation of RPM.

- Safety precautions and procedures.

- Kinds of tapers.

- Types of taper-turning methods.

- Calculation of tapers.

- Methods and techniques of adjusting speeds and feeds of turning machine.

- Method of checking quality of components produced.

- Specific safety guidelines and precautions.

- Types of knurling tools.

- Types of knurling.

- Procedure of setting tools and work piece in the machine.

- Methods of knurling.

- Procedure of adjusting speeds and feeds of lathe machine. Importance of using coolants during knurling.

- Safety precautions and guidelines.

- Knowledge of lathe operations

- Use of dial indicator

- Calculations for taper turnings

- Knowledge of lathe operations

- Types of threading tool.

- Types of threading.

- Procedure of setting tools and work piece in the machine.

- Methods of threading.

- Procedure of setting up of drilling machine.

- Safe procedure for operating drilling machines.

- Types of drilling machines.

- Selecting and adjusting speed and feed of drilling machine.

- Importance of coolants in drilling operations.

- Methods and techniques of quality checks.

- Different types of drilling tools and their implications.

- Importance of selecting right drilling tool for the job specifications.

- Methods and techniques for positioning the work-piece in the drill to ensure proper alignment and stability during drilling.

- Using speeds and feeds chart for different types of materials and their hardness.

- Specific safety precautions during boring and sinking operations.

- Selecting reamer according to hole size.

- Types of reamers (straight teeth or helical teeth).

- Method of setting reamer in the drill chuck.

- Importance of using lubricants during reaming.

- Importance of alignment of the reamer during operations

- List safety hazards related with the shaper machine operations.

- Use of Dial indicator

- Method of mounting of cutting tool

- Checking of right angle with the tri square.

- Safety guidelines and procedures.

- Safety checks for operating shaper machine.

- Interpreting information given in the engineering drawings and job specifications

- Checking of angle with the bevel protector.

- V-Shape cutting procedure.

- Use of Dial indicator

- Method of mounting of cutting tool

- Checking of angle with the bevel protector.

- V-Shape cutting procedure.

- Safety guidelines and procedures.

- Safety checks for operating Broaching machine.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Identify Machining requirements according to given in the manufacturing drawings

- Identify material specifications according to manufacturing drawing

- Identify bill of material (BOM) according to manufacturing drawing

- Interpret dimensional tolerances according to manufacturing drawing

- Identify and mark unsafe or faulty tools for repair according to designated procedures before, during and after use.

- Maintain tools, including hand sharpening according to standard operational procedures, principles and techniques.

- Identify and mark unsafe or faulty tools for repair according to designated procedures before, during and after use.

- Store power tools safely in appropriate location according to standard operational procedures.

- Perform cutting operation with power hacksaw

- Perform Drilling Operations

- Perform General machining Operations

- Cut material as per job specifications and dimensions provided in the drawing

- Perform Different lathe Operations

- Perform Shaper, Planer, and Slotter Machining

- Perform Broaching Operations

Tools & Equipment

- Bench vice

- Tri-square

- Hand hacksaw with blade

- Scriber

- Lathe Machine

- Drilling Machines

- Drill chuck with Key

- Machine Vice

- Marking Tools

- Measuring Tools

- Drill Sleeve and Socket

- Personal Protective Equipment

- Counter drill

- Cutting oil

- Shaper, Planar or Slotter

- Machine Vice

- Vernier Calliper

- Dial indicator with magnet stand

- Point cutting tools

- Personal Protective Equipment

- Bevel protector

- Cutting Tools

- Measuring Tools

- Personal Protective Equipment

- Files

- Checking gauges

- Threading Tools

- Flat File

- Broaching Machine

## Mech-151. Occupational Health, Safety and Environment

Overview

This competency standard covers the skills and knowledge required to Ensure occupational health, safety and environment by illustrating importance of safety, prevention of accidents and electric shocks, prevention of fire, adopting safety at work place, maintain personal protective equipment (PPE), follow environmental safety and reduce pollution, provide first aid to injured and analyse the causes of accidents, promote safety culture and adopt international safety standards.

| Competency Units | Performance Criteria |
| --- | --- |
| - Illustrate the importance of safety | - Share personal experiences affected others by safety - Develop images, posters to demonstrate safety hazards - Share economic benefits to adopt safety in workshop |
| - Create Awareness of Accidents in Mechanical Industry | - Create flow chart to show sequence of accidents - Make images to recreate accident scenes - Create posters to highlight hazards in industry |
| - Prevent electric shocks, | - Inspect cord and cables for damage and wear - Wear insulated gloves and shoes - Locate power switch in the workshop - Perform mock exercise for first aid treatment against electric shock |
| - Maintain fire extinguisher | - Check pressure gauge of fire extinguisher - Check expiry of fire extinguisher - Operate fire extinguisher - Replace fire extinguisher |
| - Adopt safety in plant layout | - Keep clear pathways for emergency evacuation - Check electrical layout to avoid trip over - Check chemicals storage at separate location - Check proper drainage and ventilation |
| - Maintain Personal Protective Equipment | - Arrange PPEs as per requirement - Wear PPE as per nature of job - Store PPEs at designated place after use |
| - Follow environmental safety | - Identify recycling plan - Turn off lights and other electric equipment when not in use - Track environmental hazards and report them to take corrective action |
| - Take actions to minimise pollution | - Minimize scrap and material waste during machining operations - Follow procedures to reuse or recycle materials wherever possible - Wear ear muffs to reduce noise pollution |
| - Practice mock exercise to provide first aid to the injured person | - Take emergency response training - Practice emergency response exercises - Perform mock exercise for first aid and Cardio Pulmonary Resuscitation (CPR) |
| - Analyse causes of accidents | - Follow failure mode analyses sequence - Make fish-bone diagram to identify causes of accidents and categorize into respective heads - Assess and determine corrective course of action |
| - Promote safety culture | - Follow emergency plan - Communicate safety instructions in the workshop - Assess risk and determine mitigation actions - Operate emergency equipment and supplies |
| - Follow safety regulations and adherence to international safety standards | - Adhere to environmental laws and update environmental safety practices - Update local safety practices in line with international standards - Adopt sustainable practices, reduce carbon footprint and support renewable energy resources - Plan Green waste management practices |

Knowledge & Understanding

The candidate must possess underpinning knowledge and understanding required to carry out tasks covered in this competency standard. Therefore he/she must be able to comprehend:

- Different types of hazards associated with Mechanical Technology

- Prevention and treatment of various hazards

- Personal Protective Equipment (PPE) and its uses

- International Occupational Health & Safety Regulations

- Knowledge and understanding of OHS policies in an organization

- Use of emergency equipment

- First-Aid-Box and its uses

- Knowledge of first-aid-box items.

- Know about the techniques for provision of first aid treatment

- Fire Extinguisher its types and uses

- Types of fire extinguishing materials

- Basic knowledge of air pollution (toxic and GHGs emission)

- Basic knowledge of air pollution control processes (Combustion, Conversion and Collection)

- Basic knowledge of noise control techniques (Insulation, Absorption, Damping and Isolation)

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Prepare list of applicable PPEs

- Demonstrate use of at least one of the PPEs as per requirement

- Perform first aid treatment against electric shock/minor injury.

- Demonstrate safety procedures at workplace are as per international standards

- Operate firefighting equipment as per the type of fire

- Demonstrate economical use of resources

Tools and Equipment

- Steel-toed footwear

- Safety gloves

- Appropriate safety glasses

- High visibility/reflecting tape vest

- Breathing apparatus

- Site emergency evacuation plan

- Fire extinguishers

- Fire blankets

- Respirators and masks

- Fire hoses

- First aid kit and stretchers

## Comp-152.  Computer Applications

Overview:

This competency standard covers the skills and knowledge required to Perform Electronic Data Processing (E.D.P.), Operate MS-Windows -2010, Operate MS-Office (MS-Word 2016), Operate MS-Office (MS-Excel-2016), Operate MS. Office (MS-Power Point- 2016), Browse on Internet & Practice of e-mails.

| Competency Units | Performance Criteria |
| --- | --- |
| - Perform Electronic Data Processing (E.D.P.) | - Enter Data in Computer - Process Data of Computer - Store Data in Computer - Secure Data of Computer - Create Data Backup |
| - Operate MS-Windows -2010 | - Install MS Windows - Install MS Office Word - Install MS office Excel. - Install MS Power Point - Install and update software correctly - Manage files and folders |
| - Operate MS-Office (MS-Word 2016) | - Operate Microsoft basic commands in MS word - Open File in MS word - Format file - Font (Type/size/bold/Italic) - Header Footer - Page number - Insert pics / table/hyperlink - Save File - Save folder of MS word |
| - Operate MS-Office (MS-Excel-2016) | - Operate basic commands in Microsoft MS Excel - Open a worksheet - Apply Sum, division, multiplication, subtract, filter functions - Apply If functions - Perform Basic calculations - Insert Table and graphs - Save a worksheet/folder |
| - Operate MS. Office (MS-Power Point- 2016) | - Prepare Microsoft power point presentation with basic commands - Make power point file - Insert pics/table/hyperlink - Design theme for slides - Save power point file |
| - Browse on Internet & write e-mails | - Perform browsing on the internet as per needs - Perform search online on new trends in market - Perform emailing |

Knowledge & Understanding

The candidate must possess underpinning knowledge and understanding required to carry out tasks covered in this competency standard. Therefore he/she must be able to:

- Basic parts of computers

- Importance and uses of MS Word

- Importance and uses of MS Excel

- Importance and uses of MS Power Point

- Importance of Internet.

- Use of various search engines like Google, YouTube etc.

- Printing Procedure.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Operate MS word

- Operate MS Excel

- Operate MS Power Point

- Browse Internet

- Perform Online searching

- Perform Emailing

Tools and Equipment

- Computers

- Multimedia

- Computer Tables

- Chairs

- Printer

- Internet facility

## Mech-173 Engineering Drawing & Graphics

Overview:

This competency standard covers the skills and knowledge required to Illustrate Technical Drawing/Drafting, Apply Drafting Instruments and Accessories Draw Lines and Symbols, Draw English Alphabets (Lettering), Draw Geometrical figures and Curves, Draw Freehand Sketches of Machine Parts, Draw Projection Lines, Draw Dimensioning lines Draw Pictorial drawings and Perform Development and Intersection.

| Competency Units | Performance Criteria |
| --- | --- |
| - Illustrate Technical Drawing/Drafting | - Demonstrate Engineering drawing & Importance of Technical Drawing - Enlist Application of Technical Drawing - Categorise Drawings Used in Engineering |
| - Apply Drafting Instruments and Accessories | - Identify drawing instruments - Draw angles of 30, 45, 60 and 90 with the help of set square - Draw angles of 30, 45, 60 and 90 with the help of compass |
| - Draw Lines and Symbols | - Prepare Drawing Sheet - Select drafting Tools - Draw Tittle Bar on drawing sheet. - Draw Horizontal Vertical & Inclined Lines by Using Lead Pencil T-Square & Set Square. - Draw object outline Lines, Centre lines, Hidden lines, Dimension lines - Draw Quadrilaterals & Circles Elements - Construct Parallel Lines, Perpendicular Lines, Bisect Lines, Angles & Equal Division of Lines - Draw Plumbing & Piping Symbols - Draw Welding Symbols - Draw Material Symbols & Conventional Breaks |
| - Draw English Alphabets (Lettering) | - Prepare Drawing Sheet. - Arrange Instruments to draw Lettering. - Select correct Pencil for Lettering with Holding Techniques. - Draw Boundaries Lines as Per Standards. - Start Writing Vertical Lettering with Style Like Gothic, Roman and Free Hand Lettering etc - Practice Single Stroke Lettering - Practice Capital Lettering & Digits - Practice Double Stroke Lettering by Using Graph - Make Vertical & Inclined Lettering |
| - Draw Geometrical figures and Curves | - Prepare Drawing Sheet. - Arrange Instruments to draw Geometrical figures and Curves. - Draw Boundaries Lines as Per Standards. - Divide The Sheets in Different Equal Parts. - Draw Rectangle - Draw Angles - Draw pentagon with inscribed circle - Draw pentagon with circumscribed circle - Draw Polygons (Pentagon, Hexagon, and Octagon) By using inscribed in circle, circumscribed about a circle. - Draw scalene, equilateral and isosceles triangle - Draw Involute, Spiral, Parabola & Hyperbola |
| - Draw Freehand Sketches of Machine Parts | - Prepare Drawing Sheet - Arrange Instruments for Freehand Sketches - Draw Boundaries Lines as Per Standards. - Divide The Sheets in Different Equal Parts. - Draw Keys & Cotters - Draw V Threads, Square Threads - Draw Bench Vice - Draw Revit Heads - Draw Square & Hexagonal Nuts |
| - Draw Projection Lines | - Prepare Drawing Sheet - Arrange Drawing Instruments - Draw Boundaries As per Standards - Draw First Angle Projection - Draw Third Angle Projection - Draw Orthographic Projection of Elements/Machine Parts |
| - Draw Dimensioning lines | - Prepare Drawing Sheet - Arrange Drawing Instruments - Practice To Draw Dimension Lines of Different Machine Parts |
| - Draw Pictorial drawings | - Prepare Drawing Sheet - Draw V Block - Draw A Knuckle Joint - Draw Muff Couplings |
| - Perform Development and Intersection | - Prepare Drawing Sheet - Draw Surface Development of Cone, Prism, Pyramid |
| - Draw Section Lines | - Prepare drawing sheet - Draw boundaries - Draw Sections of Machine Parts shows by Section lines - Draw section of step Pulley - Draw section of Valve |
| - Draw Mechanical Fasteners | - Prepare Drawing Sheet - Draw Sketch of Screw - Draw Sketch of Nut & Bolts - Draw Sketch of Revit & Revit Head - Draw keys, cotters and pins - Draw Couplings |
| - Draw production Lines | - Prepare Drawing Sheet - Draw Production Line of Assembling Screw Jack - Draw Production Lines/Assembly of Tail Stock |
| - Categorize drawing Standards with related sheet examples | - Prepare Drawing Sheet - Practice Division of Drawing Sheet - Write First & Third Angle Projection - Write names of front view, side view and top view - Practice Bill of Materials |

Knowledge & Understanding

The candidate must possess underpinning knowledge and understanding required to carry out tasks covered in this competency standard. Therefore he/she must be able to:

- Illustrate Importance of Technical drawing

- Explain Uses/Application of Technical Drawing

- Explain types of Drawings Used in Engineering

- Illustrate Introduction & Importance of Drafting Instruments

- Explain List of Drawing Instruments

- Define Lettering & its types

- Explain Construction, Uses & Care of All Instruments & Accessories

- Techniques of sketching straight lines in different directions

- Define Triangles, Quadrilateral and Polygons

- Describe circular arc using different line method

- Describe circular arc

- Types of Geometric Shape

- Two-dimensional shapes

- Three-dimensional shapes

- Types of Geometric Shape

- Regular Polyhedrons

- Methods of drawing Tangents & Normal

- Describe ellipse

- Describe different methods of sketching ellipse

- Describe parabola

- Describe different methods of parabola

- Describe hyperbola curve

- Describe different methods of hyperbola curve.

- Describe spiral curve

- Describe involute curve

- Describe cycloid

- Describe epicycloids

- Describe hypocycloid

- Explain Orthographic projection 1st angle.

- Explain Orthographic projection 3rd angle.

- Explain Oblique Drawing.

- Explain Multi view drawing of Simple Bearing.

- Explain Prism, Cone and pyramid

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Draw Horizontal Vertical & Inclined Lines by Using Lead Pencil T-Square & Set Square.

- Draw Lines, Centres, Curves & Crossing of Lines

- Draw Quadrilaterals, Type of Polygons (Pentagon, Hexagon, And Octagon) By Using Different Methods.

- Draw Square & Hexagonal Nuts

- Draw Muff Couplings

- Draw Surface Development of Cone, Prism, Pyramid

- Draw section of a step Pulley

Tools and Equipment

- Graph and drawing sheet

- Drawing Board/Table

- T-Square

- Set Square

- Templates

- Geometry Box

- First aid kits, stretchers,

# 2nd Year DAE Mechanical Technology

2024

## Elect-212. Applied Electricity and Electronics

Overview:

This competency standard covers the skills and knowledge required to Distinguish Fundamentals of Electricity, Install Protection Devices and Apply Electrical Safety, Make connection to Motors, Generators and Transformers, Categorize Electrical Instruments and carryout Wiring, Distinguish Fundamentals of Electronics, Analyse Transistors and Special Diodes, Illustrate Programable Logic Controller (PLC) and Logic Gates

| Competency Units | Performance Criteria |
| --- | --- |
| - Apply Fundamentals of Electricity | - Identify basic electrical components and their functions - Measure electric current - Measure voltage and resistance - Draw Series circuit diagrams - Draw Parallel circuit diagrams - Make series circuit and measure voltage - Make parallel circuit and measure current - Adopt safety precautions related to electrical circuits |
| - Install Protection Devices and Apply Electrical Safety | - Install circuit breakers and fuses - Ground electrical systems Correctly - Inspect and test protection devices - Wear personal protective equipment (PPE) |
| - Make Electrical Connections | - Identify and match connection terminals - Practice wiring diagrams precisely - Make connection to Motors, Generators and Transformers - Check connections to prevent looseness - Verify polarity and phase alignment - Test equipment for proper operation and safety |
| - Apply Electrical Instruments and carryout Wiring | - Identify and classify electrical instruments - Perform calibration of Analog and digital instruments - Select wire types and sizes as per requirement - Draw Singal Phase wiring diagrams - Carry out Singal Phase wiring - Test wiring continuity and functionality - Follow Safety codes and regulations |
| - Apply Fundamentals of Electronics | - Identify Basic electronic components - Interpret flow of electric current - Draw Simple electronic circuits - Make Simple electronic circuit - Follow Safe handling of electronic components |
| - Identify Transistors and Diodes | - Identify Transistor types and configurations - Draw transistor circuits - Draw diode circuits - Test transistors and diodes for functionality |
| - Operate Programmable Logic Controller (PLC) and Logic Gates | - Identify PLC components - Identify logic gates components - Perform PLC programing - Troubleshoot digital logic circuits - Integrate PLCs with logic gates |

Knowledge & Understanding

The candidate must possess underpinning knowledge and understanding required to carry out tasks covered in this competency standard. Therefore he/she must be able to:

- Understand Ohm’s Law and Kirchhoff’s Laws

- Reorganization and uses electrical components

- Characteristics of series circuit

- Uses of volt meter

- Characteristics of parallel circuit

- Uses of ampere meter

- Colour codes and its uses.

- Purpose of ohm meter

- Current, voltage and resistor interact with one another

- Analyzing AC and DC circuits

- Interpretation and creation of circuit diagrams

- Measuring electrical quantities accurately

- Distinguish Function of semiconductor devices

- Illustrate the operation of transistors

- Uses and programing microcontrollers/PLCs

- Importance and implementation of electrical safety standards and procedures

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Check details of electrical and electronic circuits.

- Record voltage, current, and resistance measurements, as well as signal waveforms observed using tools like multimeters and oscilloscopes.

- Set Working models or breadboard for demonstrating the successful operation of designed circuits and systems.

- Maintain records of identified issues, diagnostic steps taken, and solutions implemented during the testing and debugging of circuits.

- Adopt safety standards and procedures, including proper grounding, insulation, and use of personal protective equipment (PPE).

Tools and Equipment

- Multi meter

- Oscilloscope

- Solder

- Breadboard

- Power Supply

- Function Generator

- Logic Analyzer

- Wire Strippers

- Pliers and Cutters

- Test Leads and Probes

- Voltage Regulator

- Heat Shrink Tubing

- Digital Calliper

## Mech-233. Computer Aided Designing (CAD)

Overview:

This competency standard covers the skills and knowledge required to install software and create new file and create basic drawing.

| Competency Units | Performance Criteria |
| --- | --- |
| - Install AutoCAD Software and Follow User Interface | - Install Latest Software Version - Follow Interface - Create New Template - Select Drawing Limits - Select Units as Per Requirements - Set Units - Perform Dimension Setting - Set Layer - Save File in designated folder |
| - Create and Modify 2D Drawings | - Draw centre lines and hidden lines - Draw Rectangles - Draw Arc, Circle, Polylines, Multiline, Ellipse, Polygon, Triangle. - Apply Commands: Copy, Paste, Move, Trim, Fillet, Chamfer, Break, Join, Hatch Mirror, Rotate, Stretch, Match, Array, Offset, Scale and Explode Etc. |
| - Adjust Drawing Setting & Coordinate Systems | - Set Drawing Sheet - Make all drafting Settings - Apply Ortho Command - Apply OSNAP Command - Practice GRID ON / OFF - Apply Polar Tracking - Give Dynamic Input - Apply Absolute Co-Ordinates - Apply Relative Co-Ordinates - Apply Polar Co-Ordinates |
| - Create Layers | - Create Layers - Modify Layers (Name, Colour, Layer On/Off, Lock/Unlock) - Apply Layers on Objects in drawing - Create at least 05 Layers for required drawing. |
| - Modify existing Dimensions and Symbols | - Create Dimension Style & Name It - Change Arrow Style & Size - Change Text Size, Style, Color - Apply Linear Dimension on Figures - Apply Align Dimension on Figures - Apply Angular Dimension on Figures - Apply Radius Dimension on Figures - Apply Diameter Dimension on Figures - Apply Continued Dimension on Figures |
| - Draw Layouts | - Create Drawing Frame - Create Contents & Template - Create Tittle Block - Save Layouts in Pdf, Jpg Formats - Apply Print Command |
| - Draw Mechanical components in AutoCAD | - Identify Mechanical Symbols - Create Springs in AutoCAD - Create Belts in AutoCAD - Create Shaft in AutoCAD |
| - Prepare Bill of Materials, Parts Lists | - Create Bill of Materials - Make a List of Parts |
| - Develop 3D Solid Model | - Apply Boundary Commands - Apply Extrude Command - Apply Revolve Command - Apply Sweep Command - Apply Loft Command - Apply Press Pull Command - Create Box, Cone, Wedge, Sphere, Pyramid, Torus - Apply 3D Orbit Command - Apply Subtract Command - Apply Intersect - Apply Union Command - Apply Fillet Edges - Create Spline - Create 3D Polyline - Create Poly solid |
| - Render 3D Models | - Select 3D Modelling Environment from Task Bar - Interact With 3D Modelling Interface - Apply Materials on 3D Solid - Apply Materials on a 3D Solid Surface - Apply Lights on 3D model - Apply Sunshade Effect - Apply Render Command & save the rendered model in Picture format |
| - Create Mechanical Models | - Create Step Block In 3D - Create V-Block In 3D - Create Bench Vice In 3D - Create Butterfly Volve In 3D - Create Screw Thread, Square Thread - Create Hexagonal Nut & Bolt |

Knowledge & Understanding

The candidate must possess underpinning knowledge and understanding required to carry out tasks covered in this competency standard. Therefore he/she must be able to:

- Basic Drawing Settings

- Unit setting

- Limit’s setting

- User coordinate system Workspace setting

- Object Snap Settings

- Basic Commands and Concepts Angles and lines in AutoCAD.

- Differentiate between absolute, relative and polar system

- DIMSTYLE and MTEXT commands

- HATCHING concepts in AutoCAD

- Differentiate between CHAMFER and FILLET command

- Types of Array

- OFFSET, CIRCLE and ROTATE short commands

- Zooming options

- Tools palettes window

- Design center

- Scale and paper sizes

- Modify dimension style and text size according to paper size

- Backup file

- 3D modelling in AutoCAD

- 3D solids,

- surfaces,

- meshes, and

- Wireframe objects.

- Differentiate between Surface Modelling and Solid Modelling.

- 3D face and Edges

- Boolean operation concepts

- Subtraction

- Intersection

- Union

- 3D Navigate control

- Functions of different camera settings.

- Importance of scene creation

- Pre-set views such as isometric, top, bottom, front, left, etc.

- Perspective projection and parallel projection

- Walk

- Constrained Orbit

- Material and light control

- Planner mapping

- Texture map

- Opacity control

- Render context

- Render sampling

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

      - Saved Drawing Sheets,

      - Render Pictures

      - Printed Layouts

Tools and Equipment

- Computer with all accessories

- AutoCAD software

- Models

## Mech-246. Workshop Practice-II (A). Perform Basic Machining-II

Overview:

This competency standard covers the skills and knowledge required to Illustrate Milling Machines, Categorize Milling Attachments and Work Holding Devices, Categorize Milling Cutters, Perform Milling Operations, Categorize Gears and Perform Gear Calculation, Categorize Indexing Methods and perform Indexing, Perform Spur Gear Cutting, Perform Helical Gear Cutting, Generate Gears on Gear Hobbing Machine, Operate Precision Grinding Machines, Perform Grinding Operations, Illustrate Non-Traditional Machining Processes, Design & Manufacture Project.

| Competency Units | Performance Criteria |
| --- | --- |
| - Interpret Milling Machines | - Identify components of milling machine. - Identify Milling Machines P2. Follow safety protocols while using milling machine. |
| - Categorize Milling Attachments and Work Holding Devices | - Identify milling attachments - Identify work-holding devices. - Identify cutter holding devices. |
| - Categorize Milling Cutters | - Identify types of milling cutters. - Arrange milling cutters according to the milling operations. - Follow safety protocols while handling the milling cutters. |
| - Perform Milling Operations | - Arrange tools and equipment according to the required drawing. - Mount cutter and blank on the machine. - Perform milling operation as per standard procedure. - Check dimensions of workpiece at regular intervals with the help of measuring instruments. |
| - Categorize Gears and Perform Gear Calculation | - Identify gears to be cut on milling machine. - Carry out calculations for making a Spur gear. - Follow safety protocols while working on milling machine. |
| - Perform setting before Indexing | - Carry out calculations for a specific indexing as per the job requirement. - Set machine for specific indexing method as per the job requirement. - Choose indexing plate as per calculations. |
| - Cut Spur Gear on Milling Machine | - Select gear cutter and indexing plate on a milling machine - Mount indexing plate on indexing head. - Centre indexing head and its tail stock. - Fix indexing head and tail stock on milling table. - Engage worm shaft from worm wheel - Adjust speed feed and direction of the cutter. - Mount Gear blank on mandrel. - Hold one side of mandrel on chuck of indexing head and other side in tail stock - Start machine and carry out cutter at zero point vertically. - Carry out cutter at zero point horizontally. - Apply depth for rough cut and engage machine automatically in longitudinal direction - Move table back at zero point. - Apply full depth for final cut and engage machine automatically in forward direction. - Repeat the process simultaneously until tooth is obtained. |
| - Cut Helical Gear on Milling Machine | - Select an indexing plate to machine a helical gear on a manual machine. - Mount gear set or Gear train to engage lead screw and indexing head spindle - Centre indexing head with its tail stock. - Fix indexing head and tail stock on milling table. - Adjust speed feed and direction of the cutter. - Mount Gear blank on mandrel. - Hold one side of mandrel on chuck of indexing head and other side in tail stock - Start machine and carry out cutter at zero point vertically. - Carry out cutter at zero point horizontally. - Apply depth for rough cut and engage machine automatically in longitudinal direction - Move table back at zero point. - Apply full depth for final cut and engage machine automatically in forward direction. - Repeat the process simultaneously until tooth is obtained. |
| - Cut Gears on Gear Hobbing Machine | - Prepare the work-piece by required machining (sawing and filing etc.) and get it ready for turning the blank - Clamp gear blank and hob cutter into their holding devices as per standard practice - Adjust safe distance between gear blank and hub cutter as per prescribed method - Adjust parameters of speed and feed from control unit as per prescribed method - Perform simple/angled/differential indexing and divide the gear into required number of divisions - Check hob cutter and gear blank that both are positioned properly and adjust them - Hold one side of mandrel on chuck of indexing head and other side in tail stock - Start machine and carry out cutter at zero point vertically. - Apply depth for rough cut and engage machine automatically in longitudinal direction - Move table back at zero point. - Apply full depth for final cut and engage machine automatically in forward direction. - Repeat process simultaneously until tooth is obtained. |
| - Operate Precision Grinding Machines | - Align machine's spindle and work table precisely - Check grinding wheels are correctly dressed and balanced. - Adjust spindle speed according to the material and wheel specifications. - Set feed rate for the workpiece and grinding wheel. - Control depth of cut to avoid excessive wear on grinding wheel - Inspect workpiece surface for desired finish quality. - Measure workpiece dimensions using precise tools to ensure they meet specified tolerances. - Conduct routine checks of the machine’s components, including spindle, bearings, and coolant system so that they are in working order. - Lubricate all moving parts to reduce friction and wear. |
| - Perform grinding operations | - Adjust optimal cutting speed for the grinding wheel and workpiece material to ensure effective material removal and desired surface finish. - Set correct feed rate to balance material removal with surface finish quality. - Adjust depth of cut to avoid overloading the wheel and to achieve desired material removal without compromising surface quality. - Measure and verify surface finish of the workpiece - Check dimensions of workpiece to conform with specified tolerances. - Dress regularly grinding wheel to maintain its cutting efficiency and prevent clogging. - Balance grinding wheel correctly to minimize vibrations and achieve smooth grinding operation. - Calibrate regularly grinding machine to maintain alignment and accuracy. - Hold workpiece Securely to prevent movement during grinding. |
| - Identify Non-Traditional Machining Processes | - Draw block diagram of Wire cut EDM machine. - Interpret block diagram of Wire cut EDM machine. - Draw block diagram of Wire EDM Sinker machine. - Interpret block diagram of EDM Sinker machine. - Draw block diagram of Laser Engraving machine. - Interpret block diagram of Laser Engraving machine. - Draw block diagram of Laser cutting machine. - Interpret block diagram of Laser cutting machine. - Draw block diagram of Water-jet cutting machine. - Interpret block diagram of Water-jet cutting machine. |
| - Design & Manufacture Project | - Clearly outline the project’s scope. - Prepare feasibility of your project in terms of technical, financial, and time constraints. - Develop detailed specifications for the chosen concept, including dimensions, materials, and performance criteria. - Prepare part and detail drawing. - Identify materials used. - Estimate quantity and cost of materials. - Develop sequential flow chart for manufacturing processes used. - Identify tools and machines used and their sequence of operation. - Perform all operations on the machine tools as per the sequence. - Adopt quality standards and specified tolerances. - Identify and correct common manufacturing issues in the project. - Apply safety procedures relevant to the manufacturing environment. - Perform and document quality control checks so that final product meets the required specifications. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Knowledge of milling machines and their working.

- Knowledge of Milling attachments and Work holding devices and their working.

- Understand working of milling cutters and their working.

- Knowledge of milling operations.

- Understand Gears and their types.

- Carry out gear calculations.

- Understand the working principle of gear hobbing machine.

- Understand the procedure for making gears through gear hobbing machine.

- Explain types and characteristics of grinding wheels.

- Explain grinding operations.

- Understand the working principle of precision grinding machine.

- Define Non-traditional machining.

- Explain types of Non-traditional machines.

- Explain the working principle of Wire cut EDM, Sinker EDM, Water-jet cutting, Laser Cutting, Laser Engraving machines.

Critical Evidences:

- Identify components of a milling machine.

- Identify types of milling cutters.

- Perform milling operations.

- Indexing on milling machine using Indexing Head.

- Make gears on milling machine.

- Make gears on gear hobbing machine.

- Perform surface finish operation using precision grinding machine.

Tools & Equipment:

Equipment:

- Milling machine

- Gear Hobbing machine

- Precision Grinding machine

- Surface Grinding machine

Cutting Tools:

- End Mills

- Face Mills

- Slot Drills

- Ball Nose Mills

- Shell Mills

- Tapered End Mills

- Reamers

Tool Holders:

- Collets

- Mandrels

- End mill holders

- Tool Changers

Work holding Devices:

- Vices

- Clamps & T-slot Nuts

- Rotary tables

- Angle plates

- Dividing heads

Measuring Instruments:

- Steel Rule

- Tri-square

- Surface gauge

- Vernier Calipers

- Dial Indicators

- Height Gauges

Safety Equipment:

- Machine Guards

- Ear Protection

- Goggles

- Gloves

- Apron

- Safety shoes

## Mech-246. Workshop Practice-II (B). Perform Foundry & Casting Works

Overview:

This competency standard covers the skills and knowledge required to Perform sand testing, Perform Core Making, Perform Moulding, Perform casting, Perform cleaning and finishing of casting action work object and qualifier

| Competency Units | Performance Criteria |
| --- | --- |
| - Perform sand testing | - Check moisture content to maintain within acceptable limits - Conduct Clay contents test of moulding sand - Perform Permeability number test for moulding sand - Conduct Green compressive strength test of moulding sand - Carry out Testing fineness number of sand samples |
| - Perform Core Making | - Practice making round core - Practice making half core - Practice baking and assembling half core |
| - Perform Moulding | - Verify mould cavity accuracy to match design specifications - Practice Basic moulding with two-piece pattern - Practice bedding in pit moulding method - Practice moulding irregular pattern with CO2 process - Practice moulding with three-piece pattern by CO2 process |
| - Perform casting | - Practice optical pyrometer - Practice complete operation of cupola furnace - Practice casting of non ferrous metal |
| - Perform cleaning and finishing of casting | - Remove surface impurities to achieve a clean finish - Grind excess material to smooth out rough areas - Perform Cleaning and finishing by tumbling barrel machine - Inspect defects for quality compliance - Apply surface treatments to meet specifications |

Knowledge & Understanding

The candidate must possess underpinning knowledge and understanding required to carry out tasks covered in this competency standard. Therefore he/she must be able to:

- Understanding Metal Casting Processes

- Material Properties and Selection

- Pattern and Mold Making Techniques

- Core Making Procedures

- Melt Preparation and Control

- Pouring Techniques and Safety

- Cooling and Solidification Control

- Defect Identification and Correction

- Cleaning and Finishing Techniques

K10 Quality Control and Inspection Procedures

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Identify Tools & equipment including furnaces, moulds, patterns, core making tools, and finishing tools.

- Identify raw materials such as metals and alloys, core materials, and mould sands.

- Identify Personal protective equipment (PPE) like gloves, goggles, and heat-resistant clothing, along with established safety procedures.

- Apply safety measures during casting.

Tools and Equipment

- Furnaces

- Molds

- Patterns

- Core Making Tools

- Pouring Equipment

- Cooling Racks

- Grinding and Finishing Tools

- Inspection Tools

- Cleaning Equipment

- Safety Gear

## Mech-246. Workshop Practice-II (C). Perform Advance Welding

Overview:

This competency standard covers the skills and knowledge required to Cut Metal by Oxy Acetylene/Oxy-Fuel Flame/Grinding (Manually and Auto Cutting), Perform Gas Metal Arc Welding (GMAW) in Flat (1F,1G), Hori (2F,2G), Ver(3F,3G) and Over Head(4F,4G) Positions, Perform Shield metal Arc Welding (SMAW) in Flat (1F,1G) Horizontal (2F,2G) Vertical (3F,3G) and overhead (4F,4G) positions, Perform Gas Tungsten Arc Welding (GTAW) in Flat (1F, 1G), Horizontal (2F, 2G), Vertical (3F, 3G) and Overhead (4F, 4G) Positions, Perform Submerged Arc Welding, Perform Friction Welding, Perform Non-Destructive Welding Tests and Perform Destructive Welding Tests

| Competency Units | Performance Criteria |
| --- | --- |
| - Cut Metal by Oxy Acetylene/Oxy-Fuel Flame/Grinding (Manually and Auto Cutting) | - Take Work piece as per drawing for cutting metal by oxyacetylene Manually and Automatically - Straighten it with the help of hammer and anvil - Set flame of welding torch in oxidizing flame as per standard - Start cutting for one side of work piece - Maintain standard distance between welding torch nozzle and work piece - Complete cut as per standard for Manual cut - Set cutting torch in Auto Feed machine - Set Feed Rate of machine - Take Work piece as per drawing for cutting metal by grinding manually and automatically - Set disk grinder on grinding machine - Set r.p.m of disk grinding machine as per standard - Cut on marked line with disk grinder by hand for manual cutting - Set guide blade of disk grinding machine as per requirement for automatic cutting - Set feed of disk grinding machine - Cut on marked line with disk grinding machine |
| - Perform Gas Metal Arc Welding (GMAW) in Flat (1F,1G), Horizontal (2F,2G), Vertical (3F,3G) and Over Head(4F,4G) Positions | - Identify welding requirements from technical drawings - Prepare GMAW welding machine as per SOP - Set up welding machine accessories as per SOP - Connect welding machine to an independent power supply - Set polarity indicated in the welding procedure specifications - Adjust welding parameters (current, voltage, wire feed speed etc.) as per welding procedure specifications/job requirements to produce Fillet Welds on Carbon Steel Plate, Groove welds on Carbon Steel Plate (1F,1G,2F,2G) - Maintain gap between electrode and base metal as per standard practices - Adopt manufacturing codes and standards for acceptance criteria of visual welding defects - Deposit root pass as per welding procedure specifications - Deposit filling passes as per welding procedure specifications - Deposit capping pass as per welding procedure specifications - Check root, filling and capping passes for any visual discontinuities as per acceptance standards - Adjust welding parameters (current, voltage, wire feed speed etc.) as per welding procedure specifications/job requirements to make Fillet Welds on Carbon steel plate, Groove Welds on Carbon Steel Plate (3F,3G,4F,4G) - Adjust welding parameters (current, voltage, wire feed speed etc.) as per welding procedure specifications/job requirements to make - Maintain and store tools/equipment/consumable materials in accordance with organization guidelines |
| - Perform Shield metal Arc Welding (SMAW)in Flat(1F,1G) Horizontal (2F,2G) Vertical (3F,3G) and overhead (4F,4G) positions | - Identify welding requirements from the job, welding procedure specifications or technical drawings - Prepare SMAW welding machine in accordance with welding procedure specifications/ manufacturer instructions - Set welding machine accessories and consumables as per job requirements, welding procedure specifications - Connect welding machine to an independent power supply - Set polarity indicated in the welding procedure specifications - Adjust welding parameters (current, voltage etc.) as per welding procedure specifications/job requirements to make Fillet welds on Carbon Steel Plate, groove welds on carbon steel plate (1F,1G,2F,2G) Maintain gap between electrode and base metal as per standard practices - Deposit root pass as per welding procedure specifications - Deposit filling passes as per welding procedure specifications - Deposit capping pass as per welding procedure specifications - Check root, filling and capping passes for any visual discontinuities as per acceptance standards - Adjust welding parameters (current, voltage etc.) as per welding procedure specifications to make Fillet welds on Carbon Steel plate, Groove weld on Carbon Steel Plate (3F,3G,4F,4G) - Carry out finishing work of welds following standard procedures - Inspect weld visually and mark any visual defects. - Clean work area in accordance with workplace safety practices - Maintain and store tools/equipment/consumable materials in accordance with organization guidelines |
| - Perform Gas Tungsten Arc Welding (GTAW) in Flat (1F, 1G), Horizontal (2F, 2G), Vertical (3F, 3G) and Overhead (4F, 4G) Positions | - Identify welding requirements from the job, welding procedure specifications. - Prepare GTAW welding machine in accordance with welding procedure specifications. - Set up welding machine accessories and consumables as per job requirements, welding procedure specifications and/or manufacturer’s instructions - Connect welding machine to an independent power supply - Set polarity indicated in the welding procedure specifications - Adjust welding parameters (current, voltage etc.) as per welding procedure specifications/job requirements to make Fillet weld on Steel plate - Maintain gap between electrode and base metal as per standard practices - Carry out welding in Flat and Horizontal (1F, 2F) and Flat (1G, 2G) positions following standard procedures - Adjust welding parameters (current, voltage etc.) as per welding procedure specifications make Grooves welds on Carbon Steel Plate - Deposit root pass as per welding procedure specifications - Deposit filling passes as per welding procedure specifications - Deposit capping pass as per welding procedure specifications - Check root, filling and capping passes for any visual discontinuities as per acceptance standards - Adjust welding parameters (current, voltage etc.) as per welding procedure specifications/job requirements to make Fillet Welds on Carbon Steel Plate (3F,3G,4F,4G) - produce Groove welds on Carbon Steel Plate - Maintain and store tools/equipment/consumable materials in accordance with organization guidelines |
| - Perform Submerged Arc Welding | - Identify welding requirements as per technical drawings - Prepare SAW welding machine in accordance with welding procedure specifications/manufacturer instructions - Set welding machine accessories and consumables as per job requirements, welding procedure specifications and/or manufacturer instructions - Connect welding machine to an independent power supply - Set polarity indicated in the welding procedure specifications - Adjust welding parameters (current, voltage, wire feed speed, welding speed etc.) as per welding procedure specifications/job requirements to produce Fillet weld on Carbon Steel Plate - Maintain gap between electrode and base metal as per standard practices - Carry out welding in 1F and 1G position following standard procedures - Adjust welding parameters (current, voltage, wire feed speed, welding speed etc.) as per welding procedure specifications/job requirements to make Groove weld on Carbon steel plate - Deposit root pass as per welding procedure specifications/job requirements - Deposit filling passes as per welding procedure specifications/job requirements - Deposit capping pass as per welding procedure specifications/job requirements - Check root, filling and capping passes for any visual discontinuities as per acceptance standards - Carry out finishing work of welds following standard procedures - Inspect weld visually and mark any visual defects. - Clean work area in accordance with workplace safety practices - Maintain and store tools/equipment/consumable materials in accordance with organization guidelines |
| - Perform Friction Welding | - Prepare both work pieces for smooth square surface for Inertia Friction welding - Mount one piece on a rotor driven chuck - Mount other piece in stationary chuck - Set r.p.m to rotate mounted work piece - Apply Low pressure on the stationary work piece - Apply high pressure force to the stationary work piece and force it toward rotating work piece to generates high friction force - Apply pressure until the plastic forming temperature is achieved. - Stop rotor and apply pressure force increasingly until the whole weld is formed. - Prepare both work pieces for smooth square surface for Linear Friction Welding - Mount one piece on lateral motion chuck - Mount other piece in stationary chuck - Set oscillating speed of lateral motion chuck - Apply high pressure force to the stationary work piece and force it toward oscillating work piece to generates a high friction force - Apply pressure until the plastic forming temperature is achieved. - Stop rotor and apply pressure force increasingly until the whole weld is formed. |
| - Perform Non-Destructive Welding Tests | - Prepare weld joint as per standard for Visual Inspection - Check for cracks - Check for pits - Check for surface pores - Check for Under cut - Check for Missed joint - Prepare weld joint as per standard for X-Ray test - Place work piece in x-ray machine - Set x-ray machine as per standard - Expose welded area to x-ray beam - Produce negative film of welded area - Mark dark spots on negative film identifying defects - Prepare weld joint as per standard for Magna flux Test - Sprinkle very small iron particles on welded joint - Slide magnet underneath the welded joint - Mark area where iron particles are gathered |
| - Perform Destructive Welding Tests | - Prepare weld joint as per standard for Tensile Test - Place work piece on universal testing machine (UTM) - Mount work piece in jaws of UTM - Start UTM machine as per standard - Note ultimate tensile strength of welded joint - Note Poisson’s ratio of welded joint - Prepare weld joint as per standard for Impact Test - Set machine for Izod test method a per standard - Prepare and Mount work piece for Izod test method as per standard - Strike test piece with striker - Note test readings |

Knowledge & Understanding

The candidate must possess underpinning knowledge and understanding required to carry out tasks covered in this competency standard. Therefore he/she must be able to:

- Feed Rate for Automatic machine

- Fore hand welding method

- Explain advantages of GMAW

- Describe the principle of MIG welding

- List Personal Protective Equipment required for MIG welding and state their use

- Explain safe working practices to be followed while carrying out MIG welding

- Identify hazards associated with MIG welding and take remedial measures

- Explain the factors to be considered in TIG welding like type and thickness of the base metal, current type and polarity, type of shielding gas to be used

- Define Visual welding defects

- State the purpose of using shielding gas in TIG welding

- Explain advantages of GTAW

- Describe the principle of TIG welding

- Explain safe working practices to be followed while carrying out TIG welding

- Identify hazards associated with TIG welding and take remedial measures

- Define Visual welding defects

- Describe various welding processes including Submerged Arc Welding process

- Explain non-destructive tests

- Describe advantages of non-destructive tests

- Describe visual inspection

- Describe x- ray welding

- Explain wave length of x-rays

- Explain difference between x-ray and gamma ray

- Describe dis advantages of magna flux test

- Describe applications of magna flux tests

Critical Evidence(s) Required.

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Start cutting for one side of work piece

- Set Feed Rate of machine

- Make Fillet welds on Carbon Steel plate, Groove weld on Carbon Steel Plate (3F,3G,4F,4G)

- Set oscillating speed of lateral motion chuck

- Apply high pressure force to the stationary work piece and force it toward oscillating work piece to generates a high friction force

- Apply pressure until the plastic forming temperature is achieved.

- Prepare weld joint as per standard for Magna flux Test

- Sprinkle very small iron particles on welded joint

- Slide magnet underneath the welded joint

- Mark area where iron particles are gathered

- Prepare weld joint as per standard for Impact Test

- Mount work piece in jaws of UTM

- Prepare weld joint as per standard for Tensile Test

Tools and Equipment

- Layout tools.

- Measuring devices.

- Hand held calculator.

- Fabrication and welding equipment

- Work bench

- Bench vice

- Hammer

- Tri-square

- Hand hacksaw

- Scriber

- Vernier caliper

- Flat File

- Number/alphabet punch

- Scriber

- Hand drill machine

- Disk grinder

- Pin grinder

- Oxygen cylinder

- Acetylene gas cylinder

- Pressure regulators

- Cylinder key

- Welding torch

- Rubber house pipe

- Back fire arrester

- Flash back arrester

- Spark lighter

- Steel wire brush

- Scriber

- Welding electrode

- Metal Filler rod

- Welding flux

- Magnifying glass

- x-ray machine

- Magnet

- Iron particles

## Mech-262. Metallurgy

Overview:

This competency standard covers the skills and knowledge required to Illustrate Metallurgy, Categorize Ores, Illustrate Production Process, Categorize Refractory Materials, Distinguish Treatment of Iron Ores, Illustrate Pre Smelting Treatment of Ores, Categorize parts and Zons of Blast Furnace, Illustrate Manufacturing of Wrought Iron, Illustrate Steel Manufacturing Processes, Categorize Types of Steel, Categorize Comparators, Categorize Digital Instruments, Illustrate and use of Coordinate Measuring Machine, Perform Thread Measurements, Perform Gear Measurements.

| Competency Units | Performance Criteria |
| --- | --- |
| - Illustrate Metallurgy | - Demonstrate metallurgy - Categories types of metallurgy - Interpret refining process - Interpret alloying process |
| - Categorize Ores | - Demonstrate Ores - Categories types of ores - Categories classification of ores |
| - Illustrate Production Process | - Categories production process - Interpret extractions and its method - Interpret refining and its method - Interpret casting and its method |
| - Categorize Refractory Materials | - Identify refractory materials - Categories types of refractory materials |
| - Illustrate Treatment of Iron Ores | - Categories iron ores - Interpret processing of iron ores - Interpret treatment of iron ores |
| - Illustrate Pre-Smelting Treatment of Ores | - Interpret pre-smelting of iron ores - Interpret process of pre smelting |
| - Categorize parts and Zones of Blast Furnace | - Illustrate blast furnace - Interpret zones of blast furnace - Interpret parts of blast furnace - Interpret pre heating zone of blast furnace - Make diagram of blast furnace |
| - Illustrate Manufacturing of Wrought Iron | - Identify wrought iron - Interpret manufacturing process of wrought iron |
| - Illustrate Steel Manufacturing Processes | - Identify Steel - Categorize types of steel - Interpret manufacturing process of steel |
| - Categorize Types of Steel | - Illustrate types of steel - Identify carbon steel - Identify stainless steel |
| - Distinguish nonferrous metals | - Illustrate nonferrous metals - Categorize nonferrous metals - Interpret uses of nonferrous metals |
| - Illustrate industrial shaping of metals | - Interpret shaping of metals - Illustrate industrial metals - Categorize metals shaping process |
| - Illustrate Die casting | - Interpret process of die casting - Categorize types of die casting - Interpret materials used in die casting - Illustrate techniques of die casting |
| - Illustrate powder metallurgy | - Interpret powder metallurgy - Categories types of powder metallurgy - Categories equipment used in powder metallurgy - Interpret characteristics of powder metallurgy - Interpret application of powder metallurgy |

Knowledge & Understanding

The candidate must possess underpinning knowledge and understanding required to carry out tasks covered in this competency standard. Therefore he/she must be able to:

- Explain metallurgy

- Enlist the types of metallurgy

- Analyse iron ores

- Explain production process in metallurgy

- Define the pre smelting process

- Define the zones of blast furnace

- Define the manufacturing process of wrought iron

- Define the steel

- Define the type of Steel

- Characteristics of Carbon, Alloy and Stainless steel

- Define uses of digital instruments.

- Explain the coordinate measuring machine

- Know about the technique of thread measurements

- Explain the method of gear measurements

- Identification of the refractory materials

- Define categories of ores

- Characteristics of copper, Carbonate, Oxides and Sulphur ores

- Define the method in production of steel

- Explain types of alloying

- Characteristics of alloys

- Explain the technique of alloying

- Define industrial shaping

- Define metal shaping

- Define die casting

- Define types of die casting

- Define Powder metallurgy

- Define application of powder metallurgy

- Explain the knowledge of comparators

- Identification of stainless steel

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Identify ores, wrought Iron and steel material and its type

- Illustrate refine processing

- Illustrate alloying processing

- Identify coordinate Measuring Machine

Tools and Equipment

- Raw materials (iron, pig iron, wrought iron)

- Plant and equipment for iron making

- Reduction processes (electric, direct, melting)

- Steel production plant and equipment

- Metal production plant and equipment

- Casting pattern shapes

- Dies

- Metal cutting machinery

- rollers

## Mech-272. Metrology

Overview:

This competency standard covers the skills and knowledge required to ensure accuracy and reliability of measurements in mechanical systems, instruments and processes. Establishing and maintaining standards for measurements, verifying and adjusting accuracy of measuring instruments, evaluating uncertainty associated with measurements, developing and implementing measurement techniques for mechanical, thermal and optical measurement, data analysis and interpretation, quality control and assurance, traceability and competence in metrology.

| Competency Units | Performance Criteria |
| --- | --- |
| - Take accurate Measurement to Control Quality | - Inspect products and processes to detect defects - Conduct relevant tests to verify quality and conformance of a product - Perform corrective actions to address defects - Implement preventive measures to avoid recurrence of defects - Establish relevant metrics to measure quality - Make comparison of quality performance with industry standards |
| - Take Linear Measurement through Supporting Tools | - Take measurement through cast iron surface plate - Take measurement through granite surface plate - Take measurement through glass surface plate - Take measurement through straight edges - Take measurement through spirit levels - Take measurement through Engineer’s parallels - Take measurement through universal surface gauge |
| - Take measurements through gauges | - Take measurement through ring gauge - Take measurement through plug gauge - Take measurement through snap gauge - Take measurement through radius gauge - Take measurement through angle gauge - Take measurement through screw pitch gauge - Take measurement through standard wire - Take measurement through feeler gauge |
| - Take measurement through adjustable measuring tools | - Take measurement through vernier callipers - Take measurement through micro-meter - Take measurement through dial indicator |
| - Take measurement through angle measuring tools | - Take measurement through fixed angle measuring tools - Take measurement through angle gauges - Take measurement through adjustable angle measuring tools with and without graduations |
| - Take measurement through precision measuring instruments | - Take measurement through vernier height gauge - Take measurement through vernier depth gauge - Take measurement through inside micro-meter - Take measurement through depth micro-meter - Take measurement through thread micro-meter - Take measurement through hot gauge micro-meter - Take measurement through vernier micro-meter |
| - Adopt accuracy in measurements | - Categories elements of metrology - Identify between controllable and random errors - Perform instruments calibration - Interpret repeatability of readings through measuring instruments |
| - Take measurement through dial instruments | - Take measurement through dial calliper - Take measurement through dial thickness gauge - Take measurement through dial bore gauge |
| - Take measurement through taper measuring devices | - Take measurement through gauge block - Take measurement through sine bar |
| - Take measurement through optical measuring devices | - Operate tool makers microscope to take readings - Operate profile projector and take readings - Operate optical flats |
| - Take measurement through comparators | - Take measurements through mechanical comparator - Take measurements through electrical comparator - Take measurements through electronic comparator |
| - Take measurement through digital instruments | - Take measurements through digital micro-meter - Take measurements through digital calliper - Take measurements through digital indicator - Take measurements through digital depth gauge - Take measurements through digital height gauge - Take measurements through digital readout |
| - Take measurement through coordinate measuring machine | - Interpret working principle of CMM and its coordinates - Take measurement through coordinate measuring machine (CMM) - Take measurement through digital 3D scanner |
| - Perform Thread Measurements | - Identify thread type - Select correct thread measuring tool - Take measurement of thread outside and pitch diameter - Take measurement of thread angle - Take measurement of thread inside and pitch diameter |
| - Perform Gear Measurements | - Take measurements through gear testing machine - Perform backlash measurement |

Knowledge & Understanding

- Measurement techniques

- Calculation of area, volume of geometric shapes

- Knowledge of dimensioning of drawings and layouts

- Knowledge of dimensioning and measurement techniques

- Knowledge of Tool’s usage

- Knowledge of dimensioning and layouts

- ISO standards of fits and tolerance

- ISO systems of fits and limits

- Knowledge of components e.g. shafts, holes, etc.

- Threads and their types

- Gears, terminologies related to gears and its types

- Knowledge about Instrumentation

Critical Evidence Required

- Take measurements through Steel rule

- Check level of surfaces with straight edge and spirit level

- Take measurement with adjustable gauge

- Take measurement with outside micro-meter

- Take measurement with Vernier caliper

- Measure angles with Dial protractor

- Take measurements through mechanical comparator

- Measure threads with thread gauges

- Take measurements through mechanical comparator

- Maintain Quality control and assurance records

- Maintain Equipment maintenance and repair records

Tools & Equipment

  - Steel rule

  - Work piece

  - Surface plate

  - Steps and collars

  - Hook rule

  - Folding rule

  - Trammel

  - Combination set

  - Surface plate

  - Scriber

  - Tri square

  - Divider

  - Round stock

  - Surface plates

  - Straight edge and spirit level

  - Engineer’s level

  - Engineer’s parallel

  - Fix gauge

  - Surface plate

  - Surface gauge

  - Dial indicator

  - Adjustable gauge

  - Hole gauge

  - Telescope

  - Micrometre

  - Vernier callipers

  - Bevel protractor

  - Vernier bevel protractor

  - Dial protractor

  - Steel protractor

  - Sine bar

  - Dial thickness gauge

  - Dial indicator

  - Gauge blocks

  - Tool makers microscope

  - Digital instruments

  - Coordinate measuring machines

  - ISO tables of fits and tolerance

  - Measurement tools

  - Thread gauge

  - Gear tooth caliper

  - Gear testing machine

  - Mechanical comparator

  - Electronic comparator

  - Electrical comparator

# 3rd Year DAE Mechanical Technology

## Mech-302. Fluid Mechanics & Hydraulics Machines

Overview:

This competency standard covers the skills and knowledge required to Categorize fluid mechanics, Illustrate Hydro kinetics, Measure Flow of fluid Through Pipes, Measure fluid pressure through Impact of Jet, Identify and use of Water Turbines, illustrate water Pumps, Categorize Hydraulic Valves and Seals, Classify and use of Hydraulic machines, Illustrate Hydraulic Actuators, Identify Hydraulic Circuits and Accessories and Operate Pneumatic Simple Machines

| Competency Units | Performance Criteria |
| --- | --- |
| - Categorize fluid mechanics | - Categorize various fluids - Distinguish properties of fluids - Illustrate application of fluid mechanics principles |
| - Illustrate Hydro kinetics | - Illustrate basic principles of hydrokinetics - Derive equations of motion for fluids - Show velocity and pressure distribution in fluid flow - Show impact of external forces on fluid |
| - Measure Flow of fluid Through Pipes | - Identify flow measurement device - Calibrate measurement device - Measure flow rate |
| - Measure fluid pressure through Impact of Jet | - Test jet impact apparatus - Calibrate measurement instruments - Record forces exerted by the jet - Calculate fluid pressure from the force readings |
| - Operate Water Turbines | - Categorize types of water turbines - Implement working principles of water turbines - Select water turbine for specific applications - Install water turbine system - Operate water turbine |
| - Operate water Pumps | - Identify types of water pumps - Adopt working principles of water pumps - Install water pump - Calculate Suction head, Delivery head and Total lift of the pump |
| - Operate Hydraulic Valves and Seals | - Identify hydraulic valves - Operate hydraulic valves - Identify hydraulic seals - Check leakage of Hydraulic Seals |
| - Calculate the performance of Hydraulic machines | - Identify different types of hydraulic machines - Illustrate operation principles of hydraulic machines - Operate hydraulic controls - Calculate efficiency of hydraulic machines |
| - Operate Hydraulic Actuators | - Identify hydraulic actuators - Illustrate operating principles of hydraulic actuators - Install hydraulic actuators - Calculate efficiency hydraulic actuators |
| - Identify Hydraulic Circuits and Accessories | - Draw hydraulic circuits - Identify components in hydraulic circuits - Analyse performance of hydraulic circuits |
| - Operate Pneumatic Simple Machines | - Start pneumatic simple machine - Control pneumatic machine's movement - Monitor machine's performance - Trouble shoot any operational issues |

Knowledge & Understanding

The candidate must possess underpinning knowledge and understanding required to carry out tasks covered in this competency standard. Therefore he/she must be able to:

- Understand Fluid Properties

- Analyse Fluid Flow Types

- Understand Hydraulic Pressure

- Design and Analyse Hydraulic Circuits

- Select Appropriate Hydraulic Machines

- Perform Simple Fluid Mechanics Calculations

- Implement Safety Procedures

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Measure the following fluid properties

- Density

- Viscosity

- Flow rates

- Solves Problems Using Bernoulli’s equation and the continuity equation

- Design simple hydraulic circuits and check their performance

- Record hydraulic machine performance

- Adopt safety standards and procedures with hydraulic systems

Tools and Equipment

- Manometer

- Flowmeter

- Hydraulic Pump

- Pressure Gauge

- Impact of jet Apparatus

- Turbine models

- Pump models

- Viscometer

- Hydraulic Cylinder

- Orifice Plate.

- Flow Control Valves

- Reservoir

- Hydraulic Test Rig

- Pressure Transducer

- Digital Calliper

## Mech-313. Applied Thermodynamics

Overview

The competency standard covers knowledge and skills of fundamentals of thermodynamics, laws and properties of perfect gases, thermodynamic processes and cycles, formation and properties of steam, boilers, steam and gas turbines, IC engines, air compressors, and refrigeration & air conditioning systems.

| Competency Units | Performance Criteria |
| --- | --- |
| - Illustrate Fundamentals of Thermodynamics | - Interpret Thermodynamics - Categorise thermodynamics systems - Apply unit conversion of thermodynamic properties - Calculate temperature, pressure, volume, energy, heat, and work - Interpret laws of thermodynamics |
| - Analyse laws and properties of perfect gases | - Analyse following laws of perfect gases; - Boyle’s Law - Charles Law - Gay Lussac’s Law - Joule’s Law - Avogadro’s Law - Dalton’s Law - Regnault’s Law - Derive general gas equation - Calculate specific heats of gas and its mathematical relations - Calculate enthalpy and entropy of gas and relations between heat and entropy |
| - Identify Thermodynamic processes and cycles | - Categorize thermodynamic processes - Apply 1st law of thermodynamics for work done during a non-flow reversible process - Interpret thermodynamic cycles |
| - Draw Thermodynamic Cycles | - Draw CARNOT CYCLE - Derive mathematical relations of Carnot cycle for Air Standard Efficiency during the cycle of operation - Draw OTTO CYCLE - Derive mathematical relations of Otto cycle for Air Standard Efficiency during the cycle of operation - Draw DIESEL CYCLE - Derive mathematical relations of diesel cycle for Air Standard Efficiency during the cycle of operation. - Draw dual combustion cycle - Derive mathematical relations for Air Standard Efficiency during the cycle of operation |
| - Illustrate properties of Steam | - Illustrate steam and classify its properties - Draw T-h and T-s diagrams for steam formation - Perform application of steam tables |
| - Calculate performance of Steam Boilers | - Illustrate boilers - Categorize the following boilers - Simple vertical boiler - Cochran Boiler - Locomotive boiler - Marine boiler - Babcock and Wilcock’s boiler - Identify boiler mountings and accessories - Calculate efficiency of the boilers |
| - Calculate Performance of Steam and Gas turbines | - Draw cycle of steam turbine - Calculate performance of the following steam turbines. - De-Laval impulse turbine - Parson’s reaction turbine - Draw cycles of gas turbines - Calculate performance of Gas Turbines |
| - Calculate Performance of Internal Combustion Engines | - Categorize Internal Combustion (IC) Engines - Draw 2-stroke and 4-stroke petrol engines with the help of P-V and T-s diagrams - Draw valve timing diagram for 2-stroke and 4-stroke petrol engine - Draw 2-stroke and 4-stroke diesel engines with the help of P-V and T-s diagrams - Draw valve timing diagram for 2-stroke and 4-stroke diesel engine - Calculate indicated, brake power, frictional power and efficiencies of IC engines |
| - Calculate performance of Air Compressors | - Categorize air compressors - Draw construction and working of single-stage reciprocating air compressors with the help of PV diagram - Calculate work done per cycle by a single-stage reciprocating air compressor with and without clearance volume - Draw multistage compression and its advantages - Calculate work done per cycle and power required to drive a rotary compressor |
| - Illustrate Refrigeration and Air Conditioning | - Categorize refrigeration systems - Draw vapour compression and vapour absorption cycles - Categorize refrigerants - Categorize air conditioning systems |

Knowledge & Understanding

The candidate must possess the underpinning knowledge and understanding required to carry out tasks covered in this competency standard. Therefore he/she must be able to comprehend:

- Basic Concepts of Thermodynamics

- Laws of thermodynamics

- Different types of energies

- Perfect Gas laws

- Perfect gases and their properties

- Specific heats of gases

- Thermodynamic processes

- Thermodynamic cycles

- Properties of steam

- Steam boilers and their performance

- Steam and gas turbines

- IC Engines

- Air compressors

- Refrigeration and air conditioning systems

Critical Evidences Required

The candidate needs to produce following critical evidence(s) to be competent in this competency standard:

- Perform conversions of measurements for thermodynamic properties such as:

  - Temperature

  - Pressure

  - Specific heats

- Write perfect gas equations to investigate the relations between different thermodynamic properties

- Record of performance parameters of

  - Boilers

  - Steam turbines

  - Gas Turbines

  - Air compressor

  - IC Engines

  - Refrigeration

  - Air conditioning

Tools and Equipment

- Barometer

- Dead weight pressure gauge

- Thermometer

- Thermocouple

- Section model of boilers

- Working model of steam turbine

- Working model of gas turbine

- Section model reciprocating air compressor

- Section model of rotary air compressor

- AC Trainer

- Refrigeration Trainer

- Working model of petrol engine

- Working model of diesel engine

## Mech-321. Industrial Planning and Adopt Production Methods

Overview:

This competency standard covers the skills and knowledge required to Carry out Industrial planning, Select Site for industry, Draw Plant layout, Illustrate Production methods, Perform Job analysis, Perform Production planning and control, Manage Quality, Perform Plant Maintenance activities, Perform Cost determination and control, Categorize Types of Cost, Operate Ware house in an industry

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Carry out Industrial planning | - Illustrate Planning Principles - Enlist Industrial Planning Requirements - Identify resources and data required for industrial planning - Draw flow chart diagram for industrial planning process - Identify Assets and Liabilities in Financial Planning - Prepare reports based on financial planning - Analyse reports and mention the feasibility - Identify Product and Raw Materials in Product Planning Phase - Draw Product Life Cycle - Identify all stages of the product life cycle - Report marketing analysis of the product - Estimate unit cost of manufacturing in Process Planning - Select right equipment required for production - Select manufacturing process - Estimate quantity envisaged |
| CU2. Select Site for industry | - Illustrate importance of site selection - Compare Land cost of optional areas - Create surveys based on appropriate geography, cost of utilities, road network, community facilities etc. - Develop Criteria for the selection of Site - Conduct Survey of the Site for an Industry - Analyse results of surveys to choose the best site - Create product focused surveys based on Skilled man power, Raw material availability, finished product transportation, spares and servicing |
| CU.3 Draw Plant layout | - Paraphrase Plant Layout Importance - Establish relation between operational cost and material handling - Identify importance of access to equipment for quick maintenance and reduction of downtime - Identify importance of flexibility of Layout - Identify safety requirements for smooth operation - Mention Objectives of Layout - Identify material handling, material control, and good house keeping - Identify effective utilization of people, equipment, space and energy - Ensure minimum capital investment - Ensure route flexible - Promote ease of maintenance - Provide employee safety and job satisfaction - Mention Types of Layouts - Enlist details of fixed position layout - Enlist details of Process Layout - Enlist details of Product layout - Identify Criteria of a good layout - Inspect arrangement of machines, equipment and work areas that material is moved smoothly - Eliminate all possible delays - Adopt Unique flow pattern of each product - Follow Safe working conditions - Identify Factors in Preparation of a Layout - Paraphrase Material factor - Paraphrase Machine factor - Paraphrase Man factor - Paraphrase Movement Factor - Paraphrase Waiting Factor - Paraphrase Service Factor - Paraphrase Building Factor - Paraphrase Change Factor |
| CU.4 Illustrate Production methods | - Enlist important activities involved in production - Highlight Need for Production - Categorize all types of production - Paraphrase Intermittent or Batch Production - Paraphrase Continuous production - Paraphrase Group Technology - Paraphrase Mass Production - Paraphrase Just-in-time - Paraphrase applications of each production methods |
| CU5. Perform Job analysis | - Demonstrate Motion Study - Identify task to be observed - Prepare Process chart with columns for to and from destinations - Perform activity and note all the movements during the process - Mark time required for each movement - Analyse important and non-important activities and the activities that took maximum time - Develop method to improve activity completion time and movement reduction - Perform any activity according to the principles of motion study - Demonstrate Time Study - Identify task to be analysed - Ensure equipment required to carry out the Time and Motion study e.g. Time Study Form, Timing Device, Pen/Pencil, Safety equipment, Clipboard - Prepare Process charts - Perform activity and note all the movements during the process - Mark time required for each movement - Analyse important and non-important activities and the activities that took maximum time - Develop method to improve activity completion time and movements reduction - Perform any activity according to the principles of Time study |
| CU6. Perform Production planning and control | - Illustrate defining points of Production Planning and Control - Illustrate activities important for Production Planning and Control - Illustrate data required for a good Production Planning and Control process completion - Develop Complete Manufacturing plan for any identified product - Follow sequence of operation - Focus on issue coordinated work schedule - Highlight and maintain sufficient inventories - Illustrate and maintain production and employment level - Identify Pre-Planning Functions including Product Design, Process Design, Flow Design and Forecast Demand - Identify Planning Functions including Planning Resources, Process Planning, Scheduling and Estimation - Identify Control Function including Dispatching, Inspection, Expediting and Evaluation - Perform Routing - Identify specification of flow or sequence of operation and the process to be followed in producing a particular manufacturing lot. - Enlist Factors of routing - Perform Scheduling - Perform Loading - Perform Packaging - Enlist activities involved in Packaging - Identify release of resources along with the necessary instruction for the production function |
| CU7. Manage Quality | - Illustrate Inspection and its Types - Carryout Pre-production inspection - Carryout During production inspection - Carryout Final random inspection - Carryout Container loading inspection - Illustrate important aspects of quality - Enlist quality traits of a product - Enlist the techniques involved in quality control - Enlist the tools and techniques involved in quality assurance |
| CU8. Perform Plant Maintenance activities | - Identify Duties of Maintenance Department - Carry out Repair of equipment in shortest possible time - Ensure Minimum breakdown equipment - Ensure Existing equipment maintenance - Carry out Equipment inspection, lubrication and cleaning - Perform Alteration of Existing equipment - Ensure Protection of Equipment waste, disposal, overhaul - Develop Repair/maintenance Schedule - Perform Preventive maintenance - Develop Facilities of erection and maintenance of machinery - Identify Types of Maintenance - Interpret Breakdown Maintenance - Illustrate Preventive Maintenance - Illustrate Predictive Maintenance - Illustrate Corrective Maintenance - Illustrate Replacement Studies - Locate trouble - Stop operation - Carry out trouble shooting - Carry out trade-off between the need of replacement or repair - Perform replacement of obsolete machine |
| CU9. Perform Cost determination and control | - Procure data from Production Planning Department - Identify Administrative overheads - Identify Packaging and delivery charges - Determine Total cost - Determine Sale Price - Identify Discount Allowed - Decide Delivery Time - Analyse external factors like market supply and demand competition - Demonstrate that Cost is internal to the organisation |
| CU10. Categorize Types of Cost | - Identify Material Cost - Identify Standard Time - Identify Labour Cost - Identify Prime Cost - Identify Factory Overheads - Identify Direct Cost - Identify Indirect Cost |
| CU11. Operate Ware house in an industry | - Verify Materials Report at Central Receiving section - Unload goods - Verify consignment - Record on goods receive note - Inspect material by using gauges - Check Stores in-ward Receipt - Read Inspection Report - Inspect Rejection Memo - Check form and identify date, requisition number, name, department, and job number, material specifications, and quality, place of delivery, unit cost, total cost, and signature of the person making requisition. - Issue Materials against the requisition - Keep up to date Records |

Knowledge & Understanding

The candidate must possess underpinning knowledge and understanding required to carry out tasks covered in this competency standard. Therefore he/she must be able to:

- Define Planning

- Describe Industry

- Describe Industrial Planning

- Distinguish between simple planning and industrial planning

- Explain various phases of Industrial Planning

- Describe the importance of Report writing

- Explain Life Cycle

- Describe the methods required to analyses a potential site

- Describe difference between economic and technical factors of site selection

- Explain results presentation method

- Describe market Analysis importance up and adjusting the machine.

- Describe plant Layout

- Describe the elements that must be considered while designing a plant layout

- Describe a floor Layout

- Describe the important attributes of good layout

- Compare between various types of layouts

- Give examples specific to each layout type

- Differentiate a good layout from a bad layout

- Differentiate flexible layout from fixed layout

- Describe the indirect advantages of a good layout

- Describe indirect cost saving benefits

- Describe time and motion study

- Describe synchronization

- Differentiate between production and service industry

- Define production

- Describe importance of production

- Describe production types

- Explain the time and motion study process

- Explain the importance of time and motion study

- Identify an area in your knowledge where this study would be relevant

- Describe functions of Production Planning and Control

- Describe routing, scheduling and loading

- Differentiate between routing, scheduling and loading

- Describe Packaging process

- Describe Dispatching process

- Describe the importance of inspection

- Describe various inspection methods

- Identify various national and international standards of Quality

- Describe cost calculation procedure and benefits

- Describe supply-demand relationship

- Describe store keeping in an industry

- Describe store keepers’ duties and operations

- Describe the importance of record keeping

- Describe various methods of store keeping

Critical Evidences Required

The candidate needs to produce following critical evidence(s) to be competent in this competency standard:

- Conduct Situation Analysis

- Set Goal

- Carry out Resource Allocation

- Optimize Process

- Carry out Risk Management

- Perform measurement

## Mech-332. Material Testing and Heat Treatment

Overview:

This competency standard covers the skills and knowledge required to Illustrate Mechanical properties of Metallic Materials, Pattern Destructive Test, Perform Non-Destructive Tests, lustrate Heat treatment Process, Categorize Heat Treatment Equipment, Perform Heat Treatment of Steels, Perform Case Hardening of Steel, Perform Heat Treatment of Non-Ferrous and Cast iron, Draw Carbon Equilibrium Diagram for Steel.

| Competency Units | Performance Criteria |
| --- | --- |
| - Illustrate Mechanical properties of Metallic Materials | - Illustrate mechanical properties of materials - Categories types of metallic materials - Identify characteristics of metallic materials |
| - Perform Destructive Test | - Prepare sample according to standard requirements. - Flat and clean the specimen surface with file or emery paper - Colour/Etch the indent ball to make the indentation diameter visible - Apply load /pressure for specific time according to material requirement. - Note load /pressure. - Measure diameter of indent with microscope and calculate the average diameter - Calculate hardness with the help of Brinell Hardness Number formula. - Record results. - Install required indenter according to B-scale. - Set anvil of the Rockwell tester. - Apply minor and major load respectively. - Note reading from B-scale of the dial indicated by red color. - Install required indenter according to C-scale - Place sample on anvil and lift the sample with handle - Apply minor and major load respectively. - Note reading from C-scale of the dial indicated by black color. - Prepare sample according to standard requirement. - Create V-notch in the sample as per standard requirements - Set pendulum in its position and attach the activator knob with pendulum arm. - Hold sample in the clamp as per requirement of Izod impact test. - Release pendulum. - Note reading from the dial. - Calculate impact resistance according to formula. - Prepare sample as per standard requirement. - Prepare universal testing machine for testing. - Install fixture of tensile test specimen according to sample dimension and shape. - Perform tearing of the main scale. - Clamp samples in the grips properly. - Clamp extensometer on the sample - Set machine speed according to sample. - Note yield force and maximum force at break. - Draw stress/strain curve from data. - Calculate % elongation and % area reduction, yield strength, tensile strength and modulus of elasticity. - Measure dimensions of concrete sample. - Prepare universal testing machine for test. - Install fixture of compression - Note maximum/breaking force - Calculate compression strength. - Prepare sample as per ASTM standard. - Power on machine control panel, grip panel and move machine stroke upward and downward. - Install and adjust fixture of bending test specimen according to sample dimension. - Observe specimen for marks or cracks. - Measure bend strength / modulus of elasticity by calculating deflection from formula. - Prepare universal testing machine for test. - Install fixture of shear test - Set machine speed according to sample. - Note maximum/breaking force - Calculate shear strength - Adjust speed, torque angle and time of machine as per material requirement. - Draw torque vs angle graph. - Calculate torsion strength and observe fractured surface of the specimen. - Grip samples in fixture - Apply load as per material requirement. - Observe number of rotation once the material breaks. - Calculate fatigue strength by using formula |
| - Perform Non-Destructive Tests | - Perform pre-cleaning of samples. - Apply dye penetrant. - Remove excess dye penetrant. - Apply developer. - Inspect specimen for defects. - Interpret results. - Install ultrasonic probe according to requirement of specimen. - Calibrate ultrasonic equipment with respect to calibration block - Apply coolant on the given specimen - Observe peaks. - Interpret peaks and record the results - Prepare surface of specimen to be examined. - Apply magnetic field is to the specimen - Apply ferro magnetic medium - Remove excess ferromagnetic medium. - Interpret indications. - Evaluate results. |
| - Perform Heat treatment Process | - Prepare samples of appropriate size. - Place sample in the furnace - Adjust temperature of furnace and soaking time of the furnace according to steel grade. - Select suitable quenching media. - Quench samples in quenching media. - Prepare sample according to standard metallographic techniques - Check microstructure - Record results |
| - Categorize Heat Treatment Equipment | - Categorize types of heat treatment equipment - Identify furnaces used in heat treatment - Categories types of ovens used in heat treatment |
| - Perform Heat Treatment of Steels | - Take hardened sample of steel - Place sample in the furnace - Adjust tempering temperature of furnace and soaking time of the furnace according to require microstructure. - Cool specimen in air - Prepare sample according to standard metallographic techniques. - Check microstructure. - Calculate results. |
| - Perform Case Hardening of Steel | - Cut and prepare the samples of appropriate sizes. - Pack samples in carbonaceous material in steel box and seal boxes as per SOP. - Place boxes in the furnace - Heat samples for suitable time and temperature. - Turn off furnace and remove the steel boxes from furnace and recover the specimen. - Prepare sample according to standard metallographic techniques - Check hardness of the sample’s core and case. - Check microstructure of case and core. - Record results of hardness and microstructure. |
| - Perform Heat Treatment of Non-Ferrous and Cast iron | - Perform heat treatment of non-ferrous - Apply method of heating for non-ferrous metals - Perform heat treatment of Cast Iron |
| - Draw Carbon Equilibrium Diagram for Steel | - Draw diagram consists of two axes. (X & Y) - Plott Temperature is on Y axis - Plott carbon Percentage on X axis - Show different stages of steel due to carbon’s percentage with respect to temperature. |

Knowledge & Understanding

The candidate must possess underpinning knowledge and understanding required to carry out tasks covered in this competency standard. Therefore he/she must be able to:

- Explain mechanical properties of metals

- Enlist the types of metals

- Heat treatment of steel

- Microstructure of common metallic materials

- Knowing about the hardness of materials

- Explain the method of destructive test

- Explain the method to finding the hardness of metals

- Explain bending test of metals

- Define the method of case hardening

- Explain process of tensile test

- Characteristics of steel during heat treatment

- Define stress and strain

- Explain elongation percentage during tensile test

- Define Brinell hardness number

- Explain b scale and c scale in Rockwell test

- Define Izod impact test to calculate hardness of materials

- Explain universal testing machine

- Define compression test

- Define non-destructive test

- Explain dye penetrant test

- Define ultrasonic test

- Explain magnetic test

- Explain the knowledge of heat treatment process

- Identification of case hardening of steel

- Explain heat treatment of non-ferrous metals

- Explain iron carbide diagram

- Explain the method involve to make the process to make steel

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Interpret metals requirements according to mechanical properties of metals

- Identify material hardness to used destructive test

- Interpret steel according to heat treatment process

- Perform heat treatment through furnaces

Tools and Equipment

- Heating furnace

- Long tongue

- Quenching bath

- Mounting press

- Grinder

- Polisher

- Beaker

- Optical microscope

- Brinell hardness tester

- Optical Microscope

- Filer

- Rockwell Hardness Tester

- Spirit Level

- Izod Impact Tester

- Vernier Caliper

- Tongs

- Charpy Impact Tester

- Holder for sample cantering

- Universal Testing Machine

- Vernier Caliper

- Steel rule

- Torsion test machine

- Vernier caliper

- Fatigue test machine

- Dye penetrant accessories

- Ultrasonic flaw detector

## Mech-333. Machine Design & Analysis

Overview:

After completing this Competency Standard, the trainee will be able to develop the skills and competency required to calculate tensile, compressive, and shear loads, thickness of shells, hoop and longitudinal stresses, transverse and longitudinal failure of pressure vessels, tensile and shear stresses in transverse and parallel fillet weld, length of a sunk key, diameters of shafts (solid and hollow) under torsion. Design flange coupling, helical springs, calculate bearing modulus and velocity, and acceleration diagram for knife edge follower. Sketch follower displacement, velocity, and acceleration diagram and CAM profile.

| Competency Units | Performance Criteria |
| --- | --- |
| - Illustrate Design Methodology | - Identify design requirements and constraints - Analyze data and identify key design factors - Create detailed drawings, models, and specifications - Create final design document |
| - Calculate Simple Stresses in Machine Parts | - Calculate tensile, compressive, and shear stresses - Calculate strain, modulus of elasticity, % age elongation, % age reduction in area - Calculate factor of safety for simple machine parts |
| - Design Pressure Vessels | - Classify pressure vessels - Calculate hoop and longitudinal stresses in cylindrical shells - Identify causes of transverse and longitudinal failure of pressure vessel |
| - Design Welded Joints | - Design parallel fillet weld under static load - Calculate tensile stresses in parallel fillet weld - Calculate shear stresses in parallel fillet weld - Design transverse fillet weld under static load - Calculate tensile stresses in transverse fillet weld - Calculate shear stresses in transverse fillet weld |
| - Design Screwed Joints | - Identify stress area of a screw - Fix screw tightening torques - Calculate external load impact on static bolts and screws |
| - Design Keys | - Categorize types of keys - Design optimal key size and dimensions proportional to shaft diameter - Design optimal key width and thickness proportional to shaft and pulley - Calculate torque transmitted by rectangular and square keys against shearing |
| - Design Shafts and Couplings | - Select correct material for shaft and couplings - Design optimal length and diameter as per application - Design shaft profile - Interpret tolerances and fits for assembly and operation - Design efficient torque transmission between shafts |
| - Perform Selection of Belt Drives | - Calculate required power transmission capacity of belt drive - Calculate speed ratio between driver and driven shafts - Select belt type and its suitable material - Calculate optimal belt tension for efficient power transmission |
| - Design Springs | - Calculate maximum load and stress of the spring - Calculate stiffness of spring - Select suitable wire diameter and material - Calculate required number of coils and end type - Calculate fatigue and endurance limit |
| - Perform Selection of Bearings | - Calculate magnitude of loads on bearing - Select correct type, material, size and geometry of bearing - Select appropriate lubricant - Calculate bearing modulus - Calculate coefficient of friction in the design of journal bearings |
| - Design Cam and Followers | - Calculate optimal cam profile, size and shape - Calculate follower offset for proper clearance - Calculate lift and dwell periods for followers - Draw displacement, velocity, and acceleration diagram of knife edge follower |

Knowledge & Understanding

The candidate must possess underpinning knowledge and understanding required to carry out tasks covered in this competency standard. Therefore he/she must be able to:

- Understanding of stress and strain

- Modulus of elasticity

- Interpret relationship of percentage elongation

- Interpret relationship of percentage reduction in area

- Define factor of safety for simple machine parts

- Demonstrate Stress-Strain curve

- Show thin and thick cylinders’ relationships

- Explain hoop and longitudinal stress

- Explain failure criteria for pressure vessel

- Define and explain stresses inside the thick wall

- Classify various welding types

- Classify various types of loads

- Compare nut and bolts

- Explain stresses inside the bolts

- Demonstrate failure criteria of the screw w.r.t diameters

- Explain torque theory

- Explain key’s theory

- Classify categories of the keys

- Define coupling mechanism

- Explain torque transmission

- Explain shearing and torsional stresses

- Compare twisting, bending and shearing moment.

- Compare types of springs

- Different terminologies used for spring

- Explain dimensional analysis of the helical spring

- Define follower and cam theory

- Compare follower types

- Explain follower displacement behaviour

- Explain acceleration behaviour.

- Illustrate CAM profile with the help of available data

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Design helical spring of circular wire

- Draw CAM profile as per given data

- Design specifications and calculations for:

- Shafts

- Belt drives

- Springs

- Bearings

- Keys

- Cotters

- Screws

List of Tools and Equipment

- Calculator

- Thin and Thick cylinders (model)

- Different types of welded joints (model)

- Nut bolt assembly (model)

- Shaft with various types of keyways (model)

- Various type of keys (model)

- Solid and hollow shaft (model)

- Flanges (model)

- Various types of couplings (model)

- Various type of Springs (model)

- Various type of bearings (model)

- Various types of Cams and follower mechanisms (model)

List Of Consumables

- Graph paper / Drawing sheet

- Fasteners (nuts, bolts, screws)

- Seals and gaskets

- Lubricants

- Filters and cleaning supplies

- Safety equipment

- Welding consumables

## Mech-352. CAD/CAM

Overview:

This competency standard covers the skills and knowledge required to install software and Performance of CAD/CAM Operations.

| Competency Units | Performance Criteria |
| --- | --- |
| - Develop 2D and 3D CAD Model | - Draw Lines in 2D CAD - Draw Circle in 2D CAD - Draw Ellipse in 2D CAD - Draw Arc in 2D CAD - Draw Rectangle in 2D CAD - Draw Polygon in 2D CAD - Apply Fillet in 2D CAD - Apply chamfer in 2D CAD - Apply Rotate in 3D CAD - Apply Trim in 2D CAD - Draw Spline in 3D CAD - Apply Hatch in 2D CAD - Create Cylinder in 3D CAD - Create cube in 3D CAD |
| - Draw 2D Sketch | - Draw different Lines - Draw Rectangles in 2D - Draw Arc, Circle, Polylines, Multiline, Ellipse, Polygon, Triangle. - Apply following commands: Copy, Paste, Move, Trim, Fillet, Chamfer, Break, Join, Hatch Mirror, Rotate, Stretch, Match, Array, Offset, Scale and Explode. |
| - Apply Dimension and Constraint to 2D sketch | - Set Dimension - Apply perpendicular constraint - Apply Parallel constraint - Apply Tangent Constraint - Apply Coincident constraint - Apply Concentric Constraint - Apply Collinear constraint - Apply horizontal, Vertical, Equal, Fix Constraint - Apply symmetric, smooth constraint |
| - Apply Tolerances | - Apply tolerance standards and symbols - Apply critical feature and parameters - Perform tolerance analysis (in CAD Design) - Evaluate the results and make adjustments |
| - Develop 3D Solid Model | - Create 3D Sketching Entities - Create 2D drawing and then Extrude - Create solid model and then Revolve - Create Hole in solid model - Create Fillet in solid model - Create Chamfer in solid model - Create Rib in solid model - Create Thicken Offset in solid model - Create Emboss in solid model - Create Decal in solid model - Create Sweep in solid model - Create Loft in solid model - Create Coil in solid model - Create Thread in solid model - Create Shell in solid model - Create Face Draft in solid model - Create Split in solid model - Create Boundary Path in solid model - Create Trim and Extend Surface in solid model - Create Stich Surface in solid model - Create Replace Face in solid model - Create Sculpt in solid model |
| - Perform assembly Modelling | - Apply Top-Down Assembly - Create Bottom-up Assembly - Create Assemble Parts with Mate Constraint - Create Assemble parts with Angle Constraint - Create Assemble parts with Tangent Constraint - Create Assemble parts with Insert Constraint - Create Assemble parts with Rotation Constraint - Create Assemble parts with Rotation-Transition Constraint - Create Assemble parts with Transitional Constraint |
| - Create Drawing View | - Apply Six Drawing Views - Create Six Drawing Views of a Rectangular Box - Apply Drawing Standards - Make list of Standard Drawing Sheets - Apply Dimension Styles on objects - Create Machine Parts lists |
| CU8. Prepare Presentation Module | - Create Presentation View - Create Assembly Animation |
| CU9. Develop 3D Solid Modelling | - Apply boundary commands - Apply Extrude Command - Apply Revolve Command - Apply Sweep Command - Apply Loft Command - Apply Press pull Command - Create Box, Cone, Wedge, Sphere, Pyramid, Torus - Apply 3D Orbit Command - Apply Subtract Command - Apply Intersect Command - Apply Union Command - Apply fillet edges - Create Spline - Create 3D polyline - Create Poly solid |
| CU10. Generate CAM Programming | - Interact with user Interface - Perform Materials Setup - Perform Cutting tools setup - Perform strategy and Cutting processes - Generate Tool paths - Simulate Tool paths - Perform Holes Drilling - Perform Face Cutting - Perform Side Cutting - Perform Pocket Cutting - Perform Slot Cutting - Perform Basic Milling Operations - Perform Different Finishing Operations - Perform the following lathe operations: - Facing - Turning - Grooving - Threading - Cutting P15 Perform Die Cutting P16 Perform Punch Cutting P17 Perform Taper Cutting P18 Perform 4 axis Wire Cutting P19 Perform Core Cutting P20 Draw Contouring of various Machine Parts P21 Perform Pocketing P22 Perform Engraving P23 Perform Chamfering P24 Perform restoring P25 Perform Nesting Operations P26 Perform Multi Axis (4AXIS,5AXIS) Milling Operations |
| CU11. Perform Process Planning in CAM | - Perform Stock Setup - Perform tool setup - Identify Types of stocks - Tool - Equipment - Material - Determine sequence of individual manufacturing operations - Perform Scheduling and Production Control - Select Machine Tools - Produce given model |

Knowledge & Understanding

The candidate must possess underpinning knowledge and understanding required to carry out tasks covered in this competency standard. Therefore he/she must be able to:

- Define basic measurement

- Describe Modules in 3D CAD

- Describe Toolbars in 3D CAD

- Explain Perpendicular Constraint

- Explain Parallel Constraint

- Explain Tangent Constraint

- Explain Coincident Constraint

- Explain Concentric Constraint

- Describe shortcut key for line

- Describe shortcut key to open a new file

- Describe Extrude Feature

- Describe Revolve Feature

- Describe methods of drawing circle in sketching

- Describe shortcut key to save a file

- Describe Fillets Feature

- Describe Chamfers Feature

- Describe Ribs Feature

- Describe Thicken and Offset Feature

- Describe Holes Feature

- Describe Sweep Feature

- Describe Lofted Feature

- Describe Coil Feature

- Describe Thread Feature

- Describe Shell Feature

- Describe Face Draft Feature

- Describe Replacing Face Feature

- Describe Boundary Patch Feature

- Describe Stitching Surfaces Feature

- Describe Sculpt Feature

- Describe Types of Assembly

- Describe Assembly Component

- Explain Mate Constraint

- Explain Angle Constraint

- Explain Tangent Constraint

- Explain Insert Constraint

- Explain Rotation Constraint

- Explain Rotation-Translation Constraint

- Explain Transitional Constraint

- Describe Holes Drilling

- Describe Face cutting

- Describe Side cutting

- Describe Pocket cutting

- Describe Slot cutting

- Explain Facing

- Explain Boring

- Explain Grooving

- Explain Inside Threading

- Explain Outside Threading

- Explain Cut off

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

      - Saved Drawing Sheets,

      - Prepared Jobs

      - Operate CAD/CAM Software

      - Knowledge & Understanding CAD/CAM Software.

Tools and Equipment

- Computer with all accessories

- CAD software disk

- Models

- Jobs

- CNC Machines

- Turning, Center,

## Mech-354. Workshop Practice-III (Manufacture Mechanical Projects)

Overview:

This competency standard covers the skills and knowledge required to Perform Planning for a Mechanical Project to Manufacture, Draw Assembly & detail Drawings, Prepare Cost Estimate of Materials required for the Project, Manufacture Surface gauge, Manufacture Rack and gear Assembly, Manufacture Index Plate of dividing head, Manufacture spur gear assembly (Simple gear train, Manufacture spur gear assembly (Compound gear train, Manufacture Helical gear assembly (Simple gear train), Manufacture Helical gear assembly (Compound gear train), Manufacture Lathe Tool Post (Square type), Manufacture Shaper Tool Post, Manufacture Drill Press Vice, Manufacture Screw Jack, Manufacture Working model of drill machine, Manufacture Working model of Shaper machine, Develop Project Report

| Competency Units | Performance Criteria |
| --- | --- |
| - Perform Planning for a Mechanical Project | - Develop sequential plan for carrying out different tasks in a project. - Develop a Gantt chart for all the tasks in the project. |
| - Draw Assembly & detail Drawings | - Prepare assembly and detail drawings of the project. - Prepare Conceptual 3D model using CAD software. |
| - Prepare Cost Estimate of Materials required for the Project | - Select materials as per design specifications. - Prepare Bill of materials. - Carry out cost estimation of the project. |
| - Manufacture Surface gauge | - Prepare part and detail drawing. - Identify materials used. - Estimate quantity and cost of materials. - Develop sequential flow chart for manufacturing processes - Perform operations on the machines as per drawing - Adopt quality standards and specified tolerances. - Identify common manufacturing issues in the project. - Follow safety procedures relevant to the manufacturing environment. - Perform and document quality control checks to ensure the final product meets the required specifications. |
| - Manufacture Rack and gear Assembly | - Prepare part and detail drawing. - Identify materials used. - Estimate quantity and cost of materials. - Develop sequential flow chart for manufacturing processes - Perform operations on the machine tools as per drawing - Adopt quality standards and specified tolerances. - Identify common manufacturing issues in the project. - Apply safety procedures relevant to the manufacturing environment. - Perform quality control checks to ensure the final product meets the required specifications. |
| - Manufacture Index Plate of dividing head | - Prepare part and detail drawing. - Identify materials used. - Estimate quantity and cost of materials. - Develop sequential flow chart for manufacturing processes - Identify tools and machines used and their sequence of operation. - Perform all operations on the machine tools as per the sequence. - Apply quality standards and specified tolerances. - Identify common manufacturing issues in the project. - Apply safety procedures relevant to the manufacturing environment. - Perform quality control checks to ensure the final product meets the required specifications. |
| - Manufacture spur gear assembly (Simple gear train | - Prepare part and detail drawing. - Identify materials used. - Estimate quantity and cost of materials. - Develop sequential flow chart for manufacturing processes - Identify tools and machines used and their sequence of operation. - Perform operations on the machine tools as per drawing - Follow quality standards and specified tolerances. - Identify common manufacturing issues in the project. - Apply safety procedures relevant to the manufacturing environment. - Perform and document quality control checks so that the final product meets the required specifications. |
| - Manufacture spur gear assembly (Compound gear train | - Prepare part and detail drawing. - Identify materials used. - Estimate quantity and cost of materials. - Develop sequential flow chart for manufacturing processes - Identify tools and machines used and their sequence of operation. - Perform operations on the machine tools as per the sequence. - Follow quality standards and specified tolerances. - Identify common manufacturing issues in the project. - Apply safety procedures relevant to the manufacturing environment. - Perform and document quality control checks so that final product meets the required specifications. |
| - Manufacture Helical gear assembly (Simple gear train) | - Prepare part and detail drawing. - Identify materials used. - Estimate quantity and cost of materials. - Develop sequential flow chart for manufacturing processes - Identify tools and machines used and their sequence of operation. - Perform all operations on the machine tools as per the sequence. - Ensure meeting the quality standards and specified tolerances. - Identify and correct common manufacturing issues in the project. - Apply safety procedures relevant to the manufacturing environment. - Perform and document quality control checks so that final product meets the required specifications. |
| - Manufacture Helical gear assembly (Compound gear train) | - Prepare part and detail drawing. - Identify materials used. - Estimate quantity and cost of materials. - Develop sequential flow chart for manufacturing processes - Identify tools and machines used and their sequence of operation. - Perform operations on the machine tools as per the sequence. - Follow quality standards and specified tolerances. - Identify common manufacturing issues in the project. - Apply safety procedures relevant to the manufacturing environment. - Perform and document quality control checks so that final product meets the required specifications. |
| - Manufacture Lathe Tool Post (Square type) | - Prepare part and detail drawing. - Identify materials used. - Estimate quantity and cost of materials. - Develop sequential flow chart for manufacturing processes - Identify tools and machines used and their sequence of operation. - Perform operations on the machine tools as per the sequence. - Follow quality standards and specified tolerances. - Identify common manufacturing issues in the project. - Apply safety procedures relevant to the manufacturing environment. - Perform and document quality control checks so that final product meets the required specifications. |
| - Manufacture Shaper Tool Post | - Prepare part and detail drawing. - Identify materials used. - Estimate quantity and cost of materials. - Develop sequential flow chart for manufacturing processes - Identify tools and machines used and their sequence of operation. - Perform operations on the machine tools as per the sequence. - Follow quality standards and specified tolerances. - Identify common manufacturing issues in the project. - Apply safety procedures relevant to the manufacturing environment. - Perform and document quality control checks so that final product meets the required specifications. |
| - Manufacture Drill Press Vice | - Prepare part and detail drawing. - Identify materials used. - Estimate quantity and cost of materials. - Develop sequential flow chart for manufacturing processes - Identify tools and machines used and their sequence of operation. - Perform operations on the machine tools as per the sequence. - Follow quality standards and specified tolerances. - Identify common manufacturing issues in the project. - Apply safety procedures relevant to the manufacturing environment. - Perform and document quality control checks so that final product meets the required specifications. |
| - Manufacture Screw Jack | - Prepare part and detail drawing. - Identify materials used. - Estimate quantity and cost of materials. - Develop sequential flow chart for manufacturing processes - Identify tools and machines used and their sequence of operation. - Perform operations on the machine tools as per the sequence. - Follow quality standards and specified tolerances. - Identify common manufacturing issues in the project. - Apply safety procedures relevant to the manufacturing environment. - Perform and document quality control checks so that final product meets the required specifications. |
| - Manufacture Working model of drill machine | - Prepare part and detail drawing. - Identify materials used. - Estimate quantity and cost of materials. - Develop sequential flow chart for manufacturing processes - Identify tools and machines used and their sequence of operation. - Perform operations on the machine tools as per the sequence. - Follow quality standards and specified tolerances. - Identify and correct common manufacturing issues in the project. - Apply safety procedures relevant to the manufacturing environment. - Perform and document quality control checks so that final product meets the required specifications. |
| - Manufacture Working model of Shaper machine | - Prepare part and detail drawing. - Identify materials used. - Estimate quantity and cost of materials. - Develop sequential flow chart for manufacturing processes - Identify tools and machines used and their sequence of operation. - Perform operations on the machine tools as per the sequence. - Follow quality standards and specified tolerances. - Identify and correct common manufacturing issues in the project. - Apply safety procedures relevant to the manufacturing environment. - Perform and document quality control checks so that final product meets the required specifications. |
| - Develop Project Report | - Prepare Title page of the project report. - Develop table of contents. - Develop Abstract of the project. - Prepare project report as per the standard format. - Include part and detail drawings in the report. - Binding report using hard cover. |

Important Note: - Each Trainee or a Group of 5 Trainees, Select any one Project to be Manufactured by applying the above Criteria.

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Define project.

- Differentiate between project and task.

- Knowledge of project planning.

- Knowledge of preparing and studying Gantt chart.

- Knowledge of material quantity and cost estimation of a project.

- Knowledge of CAD software’s.

- Prepare and interpret the drawings.

- Knowledge of materials and their physical properties.

- Understand the components and working of lathe, milling, shaper, drilling, power hacksaw, grinding machines.

- Understand different types of welding processes.

- Knowledge of carpentry and pattern making.

- Knowledge of foundry work.

Critical Evidences Required:

The candidate needs to produce the following critical evidences in order to be competent.

- Prepare detailed drawings, schematics, and specifications.

- Select and estimate materials for project.

- Perform different operations on Lathe machine.

- Perform different operations on milling machine.

- Operate drilling machine.

- Operate shaper machine.

- Perform different welding processes.

List of Tools & Equipment:

| Machining Tools | Measuring Tools | Precision Tools | Assembly Tools | Finishing Tools | Welding Equipment |
| --- | --- | --- | --- | --- | --- |
| Lathe | Micrometer | Bench Vise | Screwdrivers and Allen keys | Files | Electric Arc Welding plant |
| Milling Machine | Vernier Caliper | Clamps | Wrenches | Deburring Tool | Oxy-Acetylene Gas welding plant |
| Drill Press | Dial Indicator | Parallel Blocks | Tweezers | Polishing Equipment |  |
| Surface Grinder | Height Gauge |  |  |  |  |
| Band Saw |  |  |  |  |  |
| Lathe |  |  |  |  |  |
| Power Cutter |  |  |  |  |  |
| Pedestal Grinder |  |  |  |  |  |

## Mech-363. Tool and Mould Design

Overview:

This competency standard covers the skills and knowledge required to Design Jigs and Fixture, Apply Form Tools, Operate Press for various Press Works, Design Punches and Dies, Operate Sheet Metal Dies for Blanking, Piercing, Progressive, and Drawing Dies and Perform Die Casting, Design Moulds.

| Competency Units | Performance Criteria |
| --- | --- |
| - Design Jigs and Fixture | - Draw standard parts of jig - Draw Standard Parts of fixture - Draw clamps like (a) hook type (b) latch type (c) Quarter turn bolt (d) cam clamp |
| - Apply Form Tools | - Arrange form tools - Apply Flat form tools - Apply Circular form tools |
| - Operate Press Machine for Press Works | - Categrize press work operations - Perform Punching operation on Press Machine - Perform Piercing on Press Machine - Perform Blanking on Press Machine - Perform Perforating on Press Machine - Perform Cutting off & Parting on Press Machine - Perform Notching on Press Machine - Perform Lancing on Press Machine - Perform Drawing on Press Machine - Perform Bending on Press Machine - Perform Forming on Press Machine - Perform Embossing on Press Machine - Perform Coining on Press Machine |
| - Design Punches and Dies | - Illustrate Punch & Die - Enlist types of punches - Enlist types of Dies - Perform Punching Operations - Plain - Pedestal - Perforated - Compound - Pilot - Perform Die Operations - Blanking & piercing - Progressive Die - Drawing die - Inverted die - Design of Punch & Die with respect to: - Cutting Force (increasing & decreasing forces) - Types - Job Materials |
| - Operate Sheet Metal Dies for Blanking, Piercing, Progressive, and Drawing Dies | - Draw blanking die & calculate the size of punch making a blank of diameter (…) - Draw progressive die & calculate size of punch & die for appropriate material & size - Perform Piercing & Blanking operation on progressive & drawing dies |
| - Perform Die Casting | - Interpret Die Casting - Perform Die Casting for engineering elements - Design Simple Die Casting Mould |
| - Adopt Steel Standards used in Tool Making | - Enlist Steel Standards Being Used in Tool Making - Interpret SAE Standard - Interpret AISI Standard - Interpret DIN Standard - Interpret BSS Standard |
| - Perform Injection & Blow Mould | - Illustrate injection & Blow Moulding - Perform Injection Moulding Process - Perform Blow Moulding Process - Enlist Control parameters of moulding machine |
| - Design Moulds | - Identify Types of Mould - Design Injection Mould - Perform Ejection Method |

Knowledge & Understanding

The candidate must possess underpinning knowledge and understanding required to carry out tasks covered in this competency standard. Therefore he/she must be able to:

- Define jigs

- Define Fixtures

- Define Mold

- Define Die

- Enlist the types of Die & Mould

- Difference between jigs & fixtures

- Define Punching

- Define Piercing

- Define Blanking

- Define Perforating

- Define Cutting off & Parting

- Define Notching

- Define Lancing

- Define Drawing

- Define Bending

- Define Forming

- Define Embossing

- Define Coining

- Differentiate:

- Plain

- Pedestal

- Perforated

- Compound

- Pilot

- Blanking & piercing

- Progressive Die

- Drawing die

- Inverted die

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

      - Prepare Drawing Sheets,

      - Maintain Accuracy of the Jobs made by the trainee

      - Maintain documentary proofs of Design calculations

Tools and Equipment

- Drawing instruments

- Drawing Sheets

- PPE’s

- Injection Molding Machines & Plastic Materials

- Die Casting Machine

## Mech-372. CNC Machines

Overview:

This competency standard covers the skills and knowledge required to Illustrate Numerical Control, Categorize NC Machine Tool Characteristics, Distinguish Types of NC Machines Tools, Illustrate Configuration and Control Method of NC Machine Tools, Categorize Tools, Accessories, and Cutting Fluid, Program (Turning Centre), Program (Machining Centre), Illustrate Automated Manufacturing System, Illustrate Cutting Tools and Chips, Apply Condition for Cutting

| Competency Units | Performance Criteria |
| --- | --- |
| - Identify CNC Machines and associated Tools | - Identify different types of CNC Machines. - Identify associated parts of - Lathe / Turner machine - Milling machine - EDM Wire cut - CNC Router - EDM Sinker - Laser Engraving machine - Laser Cutting machine - Identify cutting tools of CNC lathe, CNC milling, EDM Wire cut, CNC Router, EDM Sinker, Laser Engraving, Laser Cutting. - Identify Air Compressor, AVR, Vices and its specification. |
| - Perform initial operations before Starting CNC machine | - Operate On/Off switches. - Mount job on machine. - Adjusting and zero setting of the machine. - Mount tool and tool holder on the machine. - Run Machine |
| - Set Programming of CNC machine before starting | - Apply G-codes and M-codes for machines. - Set parameters as per requirement. - Apply programming strategies in software. - Optimize the processes in CNC Programming. |
| - Perform Simulation (Swansoft/CIMCO Simulators) | - Set job and tool in simulation software as per drawing. - Insert part program into simulation software. - Run simulation of program to identify errors in part program. - Obtain practical experience in operating virtual CNC machines, for real machine handling. |
| - Operate NC program on NC machining and turning center | - Develop and operate a program code for setting coordinates of machine. - Develop and operate a program for interpolation and feed functions. - Develop and operate a program code for Miscellaneous, Cycle and Compensation functions of CNC machine. |
| - Perform CAM Operations on turning Centre (Canned Cycles) | - Apply G-Codes and M-codes in MDI mode to check working of chuck, tool and coolant. - Feed program into CNC machine using PROGRAM, EDIT, INSERT, ALTER and EOB keys. - Perform machine zero positioning. - Perform work reference setting for different tools. (Tool offsetting) - Apply simulation of tool movement (dry run) to check the program for errors. - Run it in single block mode. - Change Feed and Spindle Speed using Override Keys. - Perform turning operations using canned cycles. - Push emergency stop button in case of any emergency |
| - Preform CAM Operations on Machining Centre | - Apply G-Codes and M-codes in MDI mode to check working of ATC, Spindle and coolant. - Feed program into controller using PROGRAM, EDIT, INSERT, ALTER and EOB keys. - Perform machine zero positioning. - Perform work reference setting of different tools. (Tool and work offsetting) - Apply simulation of tool movement (dry run) to check the program for errors. - Run it in single block mode. - Change Feed and Spindle Speed using Override Keys. - Perform milling operations (Drilling, Contouring, and Pocketing) according to the job. - Push emergency stop button in case of any emergency |
| - Perform CAM Operations on EDM Wire cut | - Rewind EDM Wire for CNC operations. - Make tool path of the 2D sketch on wire cut controller. - Perform simulation of the tool path and modify tool path for errors. - Set up and align the job on the holding device at workstation. - Set up machine to machine zero position according to the job requirement. - Set Wire and feed roller according to required sequence of operations. - Adjust machine parameters as per drawing. - Run program. - Check cutting path, finish and accuracy. - Finish and remove the job after completion from holding device. - Rewind program for zero position of the machine. |
| - Perform CAM Operations on EDM Die Sinker | - Ensure proper dielectric fluid for cooling and flushing. - Perform zero positioning of machine - Feed CNC program of CAD model. - Adjust machine-setting parameters. - Create machining strategies using electrodes, paths and machining operations. - Adjust and control feed, speed, current, pulse, spark gap and depth of cut. - Run program in dry run mode. - Execute program for final product. - Perform visual and dimensional inspection of the finished part. |
| - Perform CAM Operations on Laser Cutting machine | - Ensure to control the safe operation of working on CNC laser cutting machine before execution of part program according to the safety measures. - Adjust feeds, speeds by adjusting amperes and current setting before operating according to the prescribed procedure - Switch machine to execution mode and start to work on defined toolpath as per prescribed method - Compare movements of machining sequence thoroughly during operating of machine according to the part program file - Complete job and inspect its accuracy and precision according to the drawing/design |
| - Perform CAM Operations on CNC Router | - Perform zero positioning of machine. - Feed CNC program of CAD model. - Adjust machine-setting parameters. - Create machining strategies of cutting, pocketing, drilling and engraving. - Adjust and control feed, spindle speed, plunge speed and depth of cut. - Set cutting tools according to diameter, length and number of flutes. - Run program in dry run mode. - Execute the program for final product. - Perform visual and dimensional inspection of the finished part. |
| - Perform Preventive maintenance of Machines and Tools | - Perform oiling and greasing of machines according to schedule. - Check condition and level of coolant, lubricant and cutting oil. - Identify sounds of bearings and other rotating parts. - Report faults and problems of the machines to the concerned person (Machine shop in charge) |

Knowledge & Understanding

The candidate must possess underpinning knowledge and understanding required to carry out tasks covered in this competency standard. Therefore, he/she must be able to understand:

- Understand Numerical Control

- Types of CNC machines and its applications.

- Uses of measuring instruments, attachment, cutting tools and clamping devices associated with CNC machines.

- Distinguish between corrective and preventive maintenance.

- Differentiate between G-codes and M-codes

- Usage of Simulation software.

- Use of Turret / Magazine and their sequence of tool mounting.

- Concept of sub program and main program.

- Setting parameters of Lathe Machine.

- Use of Lathe tools and their types with respect to operations and materials

- Lathe operations such as Facing, Turning, Drilling, Grooving, Threading, Knurling, Boring etc.

- Setting parameters of milling machine.

- Use of milling tools and their types with respect to operations and materials

- Milling machine Operations such as contouring, Profiling, drilling, Pocketing

- Wire specification of Electric Discharge machine (EDM) wire cut

- Dialing and Machine parameters of EDM Wire cut.

- Application of machine setting parameters in EDM DIE Sinker.

- CNC Router and its setting parameters.

- CNC Laser Cutting machine and its setting parameters

- Various operations on CNC Router as per the job.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Identify Machines, cutting tools and its related Attachments

- Perform Lubrication, oiling and greasing of machines.

- Apply G-codes and M-codes in CNC Programming.

- Apply strategies for process optimization.

- Feed the program in CNC Lathe.

- Perform Lathe Operations using canned cycles.

- Set machine parameters, vice attachment.

- Perform s milling operation with process optimization strategies.

- Set wire in EDM Wire cut.

- Perform wire cut operation as per drawing.

- Perform Dialing and machine setting parameters of EDM DIE Sinker.

- Perform operations on EDM DIE Sinker as per requirement.

- Perform operations on Laser cutting machine

- Perform dialing of CNC Router.

List of Tools and Equipment for CNC Lab:

| S# | Items | Specification | Quantity |
| --- | --- | --- | --- |
|  | CNC Turning Center | CNC Turning Centers with 8-12 tool pocket turret, Spindle Bore Size 50-60 mm, Swing over bed 250-300 mm, Chuck Size 200-250 mm, maximum turning length 400-500 mm with all standard and optional accessories. | 1 No |
|  | Pedestal Grinder | Pedestal grinder with safety rod and grinding wheel on both sides 10 in Dia, 1.5 in thickness | 2 Nos |
|  | Disc Cutter | Disc cutter 12 in with all standard & optional accessories | 2 Nos |
|  | Power Hacksaw | Hydraulic/Mechanical Power Hacksaw, blade size 18 inch, cutting capacity 300-350mm, 3 phase, automatic with all standard and optional accessories. | 1 No |
|  | HSS Lathe Tooling | HSS Lathe Tooling (Turning Tools, Facing, Drilling (1mm-12 mm drill set), Boring bars of different size (150mm, 200mm, 250mm, 300mm), Reamers (10mm, 15mm, 20mm), Tap set of different size (6mm, 9mm, 12mm, 15mm, 20mm, 25mm), Knurling Tool (Straight, Diamond), Threading tools (v—type) | 5 Nos each |
|  | Carbide Lathe Tooling | Carbide type with thrown away inserts Lathe Tooling (Turning Tools, Facing, Boring bars of different size (150mm, 200mm, 250mm, 300mm), Tap set of different size (6mm, 9mm, 12mm, 15mm, 20mm, 25mm), Threading tools (v—type) | 5 Nos each |
|  | CAM and Simulator Software | CAM Software and Simulator software (e.g. Swan soft, CIMCO etc.) (For 25 seats) | 1 No |
|  | Computer System | Computer System (i7, 16 GB, 5th-7th generation, Keyboard mouse, 19 in LCD) with Multimedia Projector | 1 No each |
|  | CNC EDM Wire Cut | 5 axis CNC EDM Wire-cut machine long 740-780mm, wide 550-600mm, maximum feed rate 700-850 mm/mint, maximum wire feed rate 250-350 mm/mint, object length 350-400 mm, object width 250-300 mm, u & v axis 50-70mm, tilt angle range 140-160 with all standard and optional accessories. | 1 No |
|  | CNC EDM Die Sinker | 5 axis CNC EDM Wire-cut machine long 740-780mm, wide 550-600mm, maximum feed rate 700-850 mm/mint | 1 No |
|  | CNC Router |  | 1 No |
|  | Hand Disc Grinder | Hand Disc Grinder with safety guard etc. | 5 Nos |
|  | CNC Machining Center | CNC Machining Center (table size 200-250mm, Length 1000-1200 mm, XYZ travel 600-650 mm, 320-400 mm, 350-400mm, spindle revolutions 8000-10,000 rpm, repeatability of XYZ coordinates ±0.01 mm, ATC with 12-16 cutters) with all standard and optional accessories. | 1 No |
|  | HSS Milling Cutters | HSS Milling cutters (end mill, ball mill, bull nose, ball nose, rasp cutter of different sizes (2mm, 3mm, 4mm, 6mm, 8mm, 10mm, 12mm, 14mm, 16mm, 18mm, 20mm, 25mm, 30mm) | 2 Nos each |
|  | Carbide Milling Cutters | Carbide Milling cutters (end mill, ball mill, bull nose, ball nose, rasp cutter of different sizes (2mm, 3mm, 4mm, 6mm, 8mm, 10mm, 12mm, 14mm, 16mm, 18mm, 20mm, 25mm, 30mm)) | 2 Nos each |
|  | Measuring Instruments | Measuring Instruments (Vernier (0-200mm, 0-300mm), Dial Vernier (0-200mm, 0-300mm), Vernier Dial Caliper (0-250mm), Digital Vernier (0-200mm, 0-300mm), Outside Micrometer (0-25mm, 25mm-50mm, 50mm-75mm), Inside micrometer(30mm-55mm), Depth micrometer of different insert sizes (0-25mm, 25mm-50mm, 50mm-75mm, 75mm-100mm), Steel Rule, Tri-Square(250mm), Vernier Bevel Protractor (±5 mint accuracy) with all standard and optional accessories, Vernier Height Gauge (0-300 mm) with all standard and optional accessories, Digital Vernier height gauge(0-300mm), precision level (12 in & 24 in) with all standard and optional accessories etc. | 25 Nos each |
|  | Engineering drawing equipment | Engineering drawing equipment (table (half impairer size) & stool, T-square(600mm), set square (450, 300-600), drawing pencils (1H, 2H, 3H, 4H two each), Pencil eraser (pelican, saddler or best one), Drawing sheets (scholar or equivalent 30 for every student), | 25 Nos each |
|  | Work Holding Devices | Work Holding Devices (Parallel clamp, engineering parallels, machine vice, hand vice, universal vice, 3 Jaw & 4 jaw chuck, faceplate, dog carrier, tailstock, boring bar etc.) | 5 Nos each |
|  | Personal Protective Equipment | Personal Protective Equipment (PPE) Safety helmet, Goggles, Ear plug, leather gloves, Safety shoes etc. | 25 Nos each |
|  | Hacksaw Frame | Hacksaw frame 12 inches | 25 Nos |
|  | Maintenance Box | Maintenance Box with following tools (Flat nose plier, wire cutter, screw driver, straight and Philips head tips, screw driver kit of different size bits, Grip plier, WD-40, Oil cane, Greece gun) | 5 Nos |
|  | CNC Laser Cutting Machine | CNC Laser cutting machine with minimum cutting depth of 2mm on sheet metal. | 1 No |