Course / Subject Name: NQ-PGA-314 Perform Off-Set Printing II

- Assessment Tasks (Summative Assessment)

To streamline and organize these tasks into three summative assignments, we can focus on the key areas of the printing process. These tasks can be grouped into related skill sets, ensuring that learners develop a holistic understanding of each area while also being able to demonstrate their competence.

Summative Task 1: Printing Machine Setup and Maintenance

Tasks:

1.	Cylinder Cleaning and Machine Setup:

o	Clean and inspect the cylinders (plate, blanket, and impression cylinders) of an offset printing machine.

o	Examine and mount printing machine cylinders and blankets.

o	Ensure the proper alignment and secure installation of components.

o	Perform a "dry run" to verify machine settings and ensure readiness for production.

2.	Ink and Dampening System Preparation:

o	Prepare the inking system, ensuring uniform ink distribution by adjusting ink levels and rollers.

o	Check and prepare the dampening system to maintain the proper balance between ink and water.

o	Ensure the correct flow of water, inspect dampening rollers, and check water pH levels.

o	List eco-friendly chemicals used in the inking and dampening systems.

________________________________________

Summative Task 2: Printing Operations and Quality Control

Objective: Conduct printing operations, ensure print quality, and make necessary adjustments during production.

Tasks:

1.	Inking and Dampening Operations:

o	Perform spreading of ink in the ink duct according to the given instructions or docket.

o	Ensure the ink and water balance is maintained throughout the printing process.

o	Monitor the inking and dampening systems to ensure consistency in print quality.

2.	Feeders and Delivery System Operations:

o	Adjust feeders to ensure seamless paper feeding, checking paper stack alignment and tension.

o	Set up the delivery system for smooth output of printed sheets, adjusting stacking mechanisms and inspecting sheet alignment.

3.	Printing Process Monitoring and Adjustments:

o	Perform the printing process while continuously monitoring print quality.

o	Make necessary adjustments during the print run to ensure consistent results and maintain high-quality output.

________________________________________

Summative Task 3: Post-Production and Documentation

Objective: Perform post-production tasks, ensure proper finishing, and maintain production records.

Tasks:

1.	Post-Production Inspection and Organization:

o	Inspect the printed materials for quality, ensuring they meet the required standards.

o	Organize the printed materials for further processing or delivery.

2.	Cleaning and Securing the Printing Press:

o	Clean and secure the printing press to ensure it is ready for future operations.

o	Record and report production data, documenting any issues or adjustments made during the print run.

3.	Job Card and Documentation:

o	Draft and interpret a job card (docket), ensuring all instructions are followed.

o	Check artwork, shade cards, paper grammage, quality, and ink types to ensure everything is in line with the production requirements.