|  | Assessment Evidence Guide For “Automobile Mechanic” Level-2 Formative Assessment 0716MSA1505. Service Electronic Fuel Injection System |
| --- | --- |
|  | Date 4-9November, 2024 Lahore |
|  | National Vocational & Technical Training Commission |

Instruction Sheet for the Candidate

| Qualification | Automobile Mechanic (Level-2) |
| --- | --- |
| Competency Standard(s) | 0716MSA1505. Service Electronic Fuel Injection System |
| Purpose of Assessment | Formative Assessment |

| Candidate Details | Name____________________________________________________ Registration/Roll Number_______________________________________________ |
| --- | --- |
| Guidance for Candidate | To meet this standard, you are required to complete the following within the given time frame. - Diagnose System Faults In EFI - Perform Service of the EFI System - Service Air Intake System - Service exhaust system Activity: Perform Cleaning/flushing of catalytic convertor with recommended cleaner. |
| Time: 4 hrs. | - Measure the fuel pressure with the help of gauge - Measure the resistance of the sensors and check continuity of different sensors such as IAT Sensor, ECT sensor, TP sensor, CKP and CMP sensor and injectors. - Trace of the fault by using engine diagnostic scanner - Replace the fuel feed pump, fuel filter, pressure regulator - Service throttle body - Replace idle speed control valve - Identify symptoms of common air intake issues, such as rough idling, Decreased engine performance, or increased fuel consumption. - Replace or cleans components as needed, such as changing the air filter, cleaning the throttle body, or replacing damaged intake hoses. - Test the air intake system - Dismantle the exhaust system and check for any leaks, condition of exhaust clamps, Muffler. - Dissemble the exhaust manifold. - Examine the catalytic converter using a bulb and clean the catalytic converter using specified cleaning agents |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Automobile Mechanic (Level-2) |
| Competency Standards | 0716MSA1505. Service Electronic Fuel Injection System |
| Assessment Task | - Diagnose System Faults In EFI - Perform Service of the EFI System - Service Air Intake System - Service exhaust system Activity: Perform Cleaning/flushing of catalytic convertor with recommended cleaner. |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Diagnose component faults using a vehicle diagnostic tool or scanner |  |  |
| - Check fuel pressure with a fuel pressure gauge to verify it meets the given standards |  |  |
| - check the wiring circuit and fuel injector with a multi-meter to ensure they operate within standard parameters |  |  |
| - Replace a clogged or contaminated fuel filter, fuel tank, or fuel pump strainer part of your assessment |  |  |
| - Assess the condition of each component, such as checking the air filter for dirt or ensuring hoses are free from cracks |  |  |
| - Replace or clean components like the air filter, throttle body, or intake hoses required during the assessment |  |  |
| - Test the air intake system after servicing to confirm restored engine performance |  |  |
| - Measure and assess the condition of exhaust components to ensure they meet manufacturer specifications |  |  |
| - Identify symptoms of common exhaust system problems, such as excessive emissions or unusual noises |  |  |
| - Identify symptoms of problems within the emission control system |  |  |
| - Use diagnostic tools, like emission testers or OBD-II scanners, to identify exhaust component issues |  |  |
| - Replace worn or damaged exhaust components, such as gaskets, hangers, or clamps, to ensure a leak-free system |  |  |
| - Use a fuel pressure gauge to check electrical circuits |  |  |
| - Diagnose air intake issues without using any tools |  |  |
| - Test the air intake system after servicing optional, rather than required, in the assessment |  |  |
| - Replace damaged or worn exhaust components if you identify issues with them |  |  |
| - Diagnose vehicle issues through visual inspection alone sufficient to meet assessment requirements |  |  |

Candidate’s Signature___________________Assessor’s Signature_____________________

Date: ________________________________

Assessors Judgment Guide

| Qualification | Automobile Mechanic (Level-2) |
| --- | --- |
| Competency Standard(s) | 0716MSA1505. Service Electronic Fuel Injection System |
| Candidate Details | Name: ______________________________________________ Registration/Roll Number: ________________________Signature: ____________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________ Assessor’s code: ________ Signature: __________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Diagnose System Faults In EFI - Perform Service of the EFI System - Service Air Intake System - Service exhaust system Activity: Perform Cleaning/flushing of catalytic convertor with recommended cleaner. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Measure the fuel pressure with the help of gauge. |  |  |  |  |  |
|  | Measure the resistance of the sensors and check continuity of different sensors such as IAT Sensor, ECT sensor, TP sensor, CKP and CMP sensor. |  |  |  |  |  |
|  | Measure the resistance of injectors. |  |  |  |  |  |
|  | Replace the fuel feed pump, fuel filter, pressure regulator |  |  |  |  |  |
|  | Trace of the fault by using engine diagnostic scanner |  |  |  |  |  |
|  | Replace the fuel feed pump, fuel filter, pressure regulator |  |  |  |  |  |
|  | Check and service of fuel injectors |  |  |  |  |  |
|  | Replace faulty fuel injectors |  |  |  |  |  |
|  | Replace faulty sensors |  |  |  |  |  |
|  | Service throttle body Replace idle speed control valve |  |  |  |  |  |
|  | Identify symptoms of common air intake issues, such as rough idling, Decreased engine performance, or increased fuel consumption. |  |  |  |  |  |
|  | Replace or cleans components as needed, such as changing the air filter, cleaning the throttle body, or replacing damaged intake hoses. |  |  |  |  |  |
|  | Dismantle the exhaust system and check for any leaks, condition of exhaust clamps, Muffler. |  |  |  |  |  |
|  | Dissemble the exhaust manifold. |  |  |  |  |  |
|  | Examine the catalytic converter using a bulb and clean the catalytic converter using specified cleaning agents |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | Automobile Mechanic (Level-2) |
| --- | --- |
| Competency Standard(s) | 0716MSA1505. Service Electronic Fuel Injection System |
| Candidate Details | Name: ________________________________________________________ Registration/Roll Number: _____________ Candidate Signature: ____________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: ____________________Assessor’s code: __________ Signature of the Assessor: _______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | What does EFI Stand for? |  |  |
|  | What is the function of CAM sensor? |  |  |
|  | What is the Fuel pressure of an EFI engine? |  |  |
|  | How many fuel injectors are there in 04-cylinder engine? |  |  |
|  | What is the main function of Air Flow sensor? |  |  |

Feedback to the Candidate

Candidate’s Signature___________________ Assessor’s Signature _______________________