|  | Assessment Evidence Guide For “Automobile Mechanic” Level-2 Formative Assessment 0716MSA1506. Repair Fuel System (Carburetor) |
| --- | --- |
|  | Date 4-9November, 2024 Lahore |
|  | National Vocational & Technical Training Commission |

Instruction Sheet for the Candidate

| Qualification | Automobile Mechanic (Level-2) |
| --- | --- |
| Competency Standard(s) | 0716MSA1506. Repair Fuel System (Carburetor) |
| Purpose of Assessment | Formative Assessment |

| Candidate Details | Name____________________________________________________ Registration/Roll Number_______________________________________________ |
| --- | --- |
| Guidance for Candidate | To meet this standard, you are required to complete the following within the given time frame. - Identify and inspect parts of Fuel system (Carburetor) - Diagnose Faults in Fuel System (Carburetor) - Remove Faults in Fuel System (Carburetor) Activity: Change the float needle of carburetor fuel system and clean the carburetor system with carburetor cleaner. |
| Time: 4 hrs. | During a practical assessment, under observation by an assessor, you are required to - Remove the carburetor from the engine carefully, taking note of connections and linkages. - Disassemble the carburetor, keeping track of screws, springs, and small parts. - Remove the float bowl, jets, idle screws, and choke components. - Use a carburetor cleaner spray or soak parts in a cleaning solution designed for carburetors. - Clean jets, passages, and air bleeds to remove gum, varnish, or debris. - Reassemble the carburetor and install on the engine carefully. Make adjustments of following: - Idle Speed Screw: Set to the manufacturer's recommended position. - Mixture Screws: Adjust the air-fuel mixture screws for optimal performance. - Float Level Adjustment: Verify that the float height is set to manufacturer specifications. - Choke Operation: Check that the choke opens and closes correctly and adjust as needed. |
| Minimum Evidence Required |  |

Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Automobile Mechanic (Level-2) |
| Competency Standards | 0716MSA1506. Repair Fuel System (Carburetor) |
| Assessment Task | - Identify and inspect parts of Fuel system (Carburetor) - Diagnose Faults in Fuel System (Carburetor) - Remove Faults in Fuel System (Carburetor) Activity: Change the float needle of carburetor fuel system and clean the carburetor system with carburetor cleaner. |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Remove the carburetor from the engine, noting all connections and linkages |  |  |
| - Disassemble the carburetor while keeping track of screws, springs, and small parts |  |  |
| - Remove the float bowl as part of the disassembly process. |  |  |
| - Remove the jets, idle screws, and choke components from the carburetor |  |  |
| - Use a carburetor cleaner spray or cleaning solution to clean the parts |  |  |
| - Clean the jets, passages, and air bleeds to remove any gum, varnish, or debris |  |  |
| - Reassemble all parts of the carburetor correctly before reinstalling on the engine |  |  |
| - Reinstall the carburetor carefully on the engine, ensuring secure connections |  |  |
| - Adjust the idle speed screw to the manufacturer’s recommended position |  |  |
| - Adjust the air-fuel mixture screws for optimal engine performance |  |  |
| - Verify that the float height was set according to manufacturer specifications |  |  |
| - Choke operation checked to ensure it opens and closes correctly |  |  |
| - Ensure that no part is skipped during the disassembly process |  |  |
| - Reinstall the carburetor without adjusting any of the screws |  |  |
| - Check the choke operation before finishing the installation |  |  |

Candidate’s Signature___________________	Assessor’s Signature____________________

Date: ________________________

Assessors Judgment Guide

| Qualification | Automobile Mechanic (Level-2) |
| --- | --- |
| Competency Standard(s) | 0716MSA1506. Repair Fuel System (Carburetor) |
| Candidate Details | Name: ______________________________________________ Registration/Roll Number: ____________________Signature: ________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor___________________ Assessor’s code: __________ Signature: __________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Identify and inspect parts of Fuel system (Carburetor) - Diagnose Faults in Fuel System (Carburetor) - Remove Faults in Fuel System (Carburetor) Activity: Change the float needle of carburetor fuel system and clean the carburetor system with carburetor cleaner. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Removed the carburetor from the engine, noting all connections and linkages |  |  |  |  |  |
|  | Disassembled the carburetor while keeping track of screws, springs, and small parts |  |  |  |  |  |
|  | Removed the float bowl as part of the disassembly process. |  |  |  |  |  |
|  | Removed the jets, idle screws, and choke components from the carburetor. |  |  |  |  |  |
|  | Used a carburetor cleaner spray or cleaning solution to clean the parts |  |  |  |  |  |
|  | Cleaned the jets, passages, and air bleeds to remove any gum, varnish, or debris |  |  |  |  |  |
|  | Reassembled all parts of the carburetor correctly before reinstalling on the engine |  |  |  |  |  |
|  | Reinstalled the carburetor carefully on the engine, ensuring secure connections. |  |  |  |  |  |
|  | Adjusted the idle speed screw to the manufacturer’s recommended position |  |  |  |  |  |
|  | Adjusted the air-fuel mixture screws for optimal engine performance |  |  |  |  |  |
|  | Verified that the float height was set according to manufacturer specifications |  |  |  |  |  |
|  | Choked operation checked to ensure it opens and closes correctly |  |  |  |  |  |
|  | Ensured that no part is skipped during the disassembly process |  |  |  |  |  |
|  | Reinstalled the carburetor without adjusting any of the screws |  |  |  |  |  |
|  | Check the choke operation before finishing the installation |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | Automobile Mechanic (Level-2) |
| --- | --- |
| Competency Standard(s) | 0716MSA1506. Repair Fuel System (Carburetor) |
| Candidate Details | Name: _________________________________________________________ Registration/Roll Number: _____________Candidate Signature: _________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: __________________________Assessor’s code: ________ Signature of the Assessor: _______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | What is the principle of carburetor? |  |  |
|  | What is the Idle Air fuel mixture ratio? |  |  |
|  | What is the Idle engine RPM? |  |  |
|  | How many circuits are there of a carburetor? |  |  |
|  | What is the function of idle screw of carburetor? |  |  |

Feedback to the Candidate

Candidate’s Signature___________________ Assessor’s Signature _______________________