|  | Assessment Evidence Guide For “Automobile Supervisor” Level-4 Formative Assessment 0716MSA1701. Perform Overhauling of Engine |
| --- | --- |
|  | Date 4-9November, 2024 Lahore |
|  | National Vocational & Technical Training Commission |

| Qualification | Automobile Supervisor Level-4 |
| --- | --- |
| Competency Standard(s) | 0716MSA1701. Perform Overhauling of Engine |
| Purpose of Assessment | Formative Assessment |

| Candidate Details | Name____________________________________________________ Registration/Roll Number_______________________________________________ |
| --- | --- |
| Guidance for Candidate | To meet this standard, you are required to complete the following within the given time frame. - Inspect Engine of the vehicle - Remove Engine from the Vehicle - Dismantle the components of the engine - Repair the Engine - Re-fit Engine in the Vehicle Activity: Carry out engine removal, disassemble parts, clean, inspect, repair and reassemble the engine. |
| Time: 04:00 hrs. | During a practical assessment, under observation by an assessor, you are required to - Drain all fluids, including oil, coolant, and fuel. - Inspect the engine for visible signs of wear or damage. - Unbolt the engine mounts and carefully lift the engine from the vehicle. - Remove the cylinder head, intake manifold, and exhaust manifold. - Remove the pistons, connecting rods, and crankshaft. - Measure and inspect the cylinder bores, crankshaft, and connecting rods for wear and deformation. - Perform any required machining, such as honing the cylinders or resurfacing the cylinder head. - Replace damaged or worn-out parts like piston rings, bearings, and gaskets. - Install the crankshaft, bearings, and main caps, ensuring proper torque specifications. - Reassemble the pistons and connecting rods, fitting them into the cylinders. - Refill with new oil and coolant, and reconnect the battery. - Reinstall the engine into the vehicle and connect all mounts, hoses, and wires. - Test drive the vehicle to ensure proper operation and performance. - Perform a final inspection of all connections, fluid levels, and adjustments. - Document any changes or repairs for maintenance records |
| Minimum Evidence Required |  |

Instruction Sheet for the Candidate

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Automobile Supervisor Level-4 |
| Competency Standards | 18. Perform Overhauling of Engine |
| Assessment Task | - Inspect Engine of the vehicle - Remove Engine from the Vehicle - Dismantle the components of the engine - Repair the Engine - Re-fit Engine in the Vehicle Activity: Carry out engine removal, disassemble parts, clean, inspect, repair and reassemble the engine. |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Drain all fluids, including oil, coolant, and fuel. |  |  |
| - Inspect the engine for visible signs of wear or damage. |  |  |
| - Unbolt the engine mounts and carefully lift the engine from the vehicle. |  |  |
| - Remove the cylinder head, intake manifold, and exhaust manifold. |  |  |
| - Remove the pistons, connecting rods, and crankshaft. |  |  |
| - Measure and inspect the cylinder bores, crankshaft, and connecting rods for wear and deformation. |  |  |
| - Perform any required machining, such as honing the cylinders or resurfacing the cylinder head. |  |  |
| - Replace damaged or worn-out parts like piston rings, bearings, and gaskets. |  |  |
| - Install the crankshaft, bearings, and main caps, ensuring proper torque specifications. |  |  |
| - Reassemble the pistons and connecting rods, fitting them into the cylinders. |  |  |
| - Refill with new oil and coolant, and reconnect the battery. |  |  |
| - Reinstall the engine into the vehicle and connect all mounts, hoses, and wires. |  |  |
| - Test drive the vehicle to ensure proper operation and performance. |  |  |
| - Perform a final inspection of all connections, fluid levels, and adjustments. |  |  |
| - Document any changes or repairs for maintenance records |  |  |

Candidate’s Signature___________________		Assessor’s Signature_____________________

Date: ________________________________

Assessors Judgment Guide

| Qualification | Automobile Supervisor Level-4 |
| --- | --- |
| Competency Standard(s) | 0716MSA1701. Perform Overhauling of Engine |
| Candidate Details | Name: ______________________________________________ Registration/Roll Number: ________________________Signature: ____________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________ Assessor’s code: ___________________ Signature: __________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Inspect Engine of the vehicle - Remove Engine from the Vehicle - Dismantle the components of the engine - Repair the Engine - Re-fit Engine in the Vehicle Activity: Carry out engine removal, disassemble parts, clean, inspect, repair and reassemble the engine. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Drain all fluids, including oil, coolant, and fuel. |  |  |  |  |  |
|  | Inspect the engine for visible signs of wear or damage. |  |  |  |  |  |
|  | Unbolt the engine mounts and carefully lift the engine from the vehicle. |  |  |  |  |  |
|  | Remove the cylinder head, intake manifold, and exhaust manifold. |  |  |  |  |  |
|  | Remove the pistons, connecting rods, and crankshaft. |  |  |  |  |  |
|  | Measure and inspect the cylinder bores, crankshaft, and connecting rods for wear and deformation. |  |  |  |  |  |
|  | Perform any required machining, such as honing the cylinders or resurfacing the cylinder head. |  |  |  |  |  |
|  | Replace damaged or worn-out parts like piston rings, bearings, and gaskets. |  |  |  |  |  |
|  | Install the crankshaft, bearings, and main caps, ensuring proper torque specifications. |  |  |  |  |  |
|  | Reassemble the pistons and connecting rods, fitting them into the cylinders. |  |  |  |  |  |
|  | Refill with new oil and coolant, and reconnect the battery. |  |  |  |  |  |
|  | Reinstall the engine into the vehicle and connect all mounts, hoses, and wires. |  |  |  |  |  |
|  | Test drive the vehicle to ensure proper operation and performance. |  |  |  |  |  |
|  | Perform a final inspection of all connections, fluid levels, and adjustments. |  |  |  |  |  |
|  | Document any changes or repairs for maintenance records |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | Automobile Supervisor Level-4 |
| --- | --- |
| Competency Standard(s) | 0716MSA1701. Perform Overhauling of Engine |
| Candidate Details | Name: ______________________________________________________________________ Registration/Roll Number: __________________ Candidate Signature: _________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: __________________________Assessor’s code: _________________ Signature of the Assessor: _______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | What is engine overhauling and why is it necessary? |  |  |
|  | Why is thorough cleaning of parts essential before installation? |  |  |
|  | Why is torque important when crankshaft and bearings installation carried? |  |  |
|  | How should the timing belt/chain and gears be aligned during reassembly? |  |  |
|  | Why is necessary adjusting valve timing and clearances in reassembly? |  |  |
|  | What is the sequence for removing major components (cylinder head, intake, exhaust) |  |  |
|  | What is the sequence for removing major components (cylinder head, intake, exhaust) |  |  |

Feedback to the Candidate

Candidate’s Signature___________________ Assessor’s Signature _______________________