|  | Assessment Evidence Guide For “Automobile Assistant Supervisor” Level-3 Summative Assessment |
| --- | --- |
|  | Date 4-9November, 2024 Lahore |
|  | National Vocational & Technical Training Commission |

Instruction Sheet for the Candidate

| Qualification | Automobile Assistant Supervisor Level-3 |
| --- | --- |
| Competency Standards | - Service Electronic Diesel Fuel Injection (Common rail) - Repair Suspension / Steering System - Carry out Wheel Balancing and Alignment - Communicate in the workplace to support customers and team - Work effectively in a customer service - sales environment - Develop professionalism - Comply with health and safety regulations |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name_______________________________________________________ Registration Number_________________________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within 05 Hrs. time frame (for practical demonstration & assessment): Dismantle and assemble inline fuel injection pump from diesel engine, perform the road test and diagnose the suspension system noise. Replace the coil spring /shock absorber. Repair/replace the power steering. Identify Wheel alignment faults and fix it. Perform wheel balancing on wheel balancing machine. |
| Time: 05 Hrs | During a practical assessment, under observation by an assessor, you are required to demonstrating the following criteria: |
| Minimum Evidence Required | - Dismantle the inline fuel pump from engine. - Reassemble the fuel pump after servicing the fuel pump from diesel laboratory. - Align / fix the fuel pump on the engine as per recommendation injection timing / firing order. - Remove air lock from diesel fuel supply system. - Perform the road test and diagnose the suspension system abnormal noise. - Raise the vehicle and replace leaked shock absorber, broken spring (coil spring/ leaf spring under the specified procedure. - Inspect the vehicle ride height/trim height. - Carryout inspection of wheel bearings faults. - Replace the power steering Rack ends. - Change the power steering fluid. - Adjust steering play. - Calibrate Electronic Power Steering with OBD-II Scanner. - Perform wheel balancing on wheel balancing machine - Perform Final Inspection and conduct a road test. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Automobile Assistant Supervisor Level-3 |
| Competency Standards | - Service Electronic Diesel Fuel Injection (Common rail) - Repair Suspension / Steering System - Carry out Wheel Balancing and Alignment - Communicate in the workplace to support customers and team - Work effectively in a customer service - sales environment - Develop professionalism - Comply with health and safety regulations |
| Assessment Task | Dismantle and assemble inline fuel injection pump from diesel engine, perform the road test and diagnose the suspension system noise. Replace the coil spring /shock absorber. Repair/replace the power steering. Identify Wheel alignment faults and fix it. Perform wheel balancing on wheel balancing machine. |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Dismantle the inline fuel pump from engine. |  |  |
| - Reassemble the fuel pump after servicing the fuel pump from diesel laboratory. |  |  |
| - Align / fix the fuel pump on the engine as per recommendation injection timing / firing order. |  |  |
| - Remove air lock from diesel fuel supply system. |  |  |
| - Perform the road test and diagnose the suspension system abnormal noise. |  |  |
| - Raise the vehicle and replace leaked shock absorber, broken spring (coil spring/ leaf spring under the specified procedure. |  |  |
| - Inspect the vehicle ride height/trim height. |  |  |
| - Carryout inspection of wheel bearings faults. |  |  |
| - Replace the power steering Rack ends. |  |  |
| - Change the power steering fluid. |  |  |
| - Adjust steering play. |  |  |
| - Calibrate Electronic Power Steering with OBD-II Scanner. |  |  |
| - Perform wheel balancing on wheel balancing machine |  |  |
| - Perform Final Inspection and conduct a road test. |  |  |

Candidate’s Signature__________________ Assessor’s Signature__________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | Automobile Assistant Supervisor Level-3 |
| --- | --- |
| Competency Standards | - Service Electronic Diesel Fuel Injection (Common rail) - Repair Suspension / Steering System - Carry out Wheel Balancing and Alignment - Communicate in the workplace to support customers and team - Work effectively in a customer service - sales environment - Develop professionalism - Comply with health and safety regulations |
| Candidate Details | Name: ______________________________________________________________ Registration Number: ________________________ Signature: _________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code: ___________________ Signature: _____________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | Dismantle and assemble inline fuel injection pump from diesel engine, perform the road test and diagnose the suspension system noise. Replace the coil spring /shock absorber. Repair/replace the power steering. Identify Wheel alignment faults and fix it. Perform wheel balancing on wheel balancing machine. |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes |  | No |  | Remarks |  |
|  | Dismantled the inline fuel pump from engine. |  |  |  |  |  |  |  |  |
|  | Reassembled the fuel pump after servicing the fuel pump from diesel laboratory. |  |  |  |  |  |  |  |  |
|  | Aligned / fixed the fuel pump on the engine as per recommendation injection timing / firing order. |  |  |  |  |  |  |  |  |
|  | Removed airlock from diesel fuel supply system. |  |  |  |  |  |  |  |  |
|  | Performed the road test and diagnosed the suspension system abnormal noise. |  |  |  |  |  |  |  |  |
|  | Raised the vehicle and replaced leaked shock absorber, broken spring (coil spring/ leaf spring under the specified procedure. |  |  |  |  |  |  |  |  |
|  | Inspected the vehicle ride height/trim height. |  |  |  |  |  |  |  |  |
|  | Carried out Inspection of wheel bearings faults. |  |  |  |  |  |  |  |  |
|  | Replaced the power steering Rack ends. |  |  |  |  |  |  |  |  |
|  | Changed the power steering fluid. |  |  |  |  |  |  |  |  |
|  | Adjusted steering play. |  |  |  |  |  |  |  |  |
|  | Calibrated Electronic Power Steering with OBD-II Scanner. |  |  |  |  |  |  |  |  |
|  | Performed wheel balancing on wheel balancing machine |  |  |  |  |  |  |  |  |
|  | Performed Final Inspection and conduct a road test. |  |  |  |  |  |  |  |  |
| Competent |  |  | Not Competent |  |  |  |  |  |  |

Knowledge Assessment

| Qualification | Automobile Assistant Supervisor Level-3 |
| --- | --- |
| Competency Standards | - Service Electronic Diesel Fuel Injection (Common rail) - Repair Suspension / Steering System - Carry out Wheel Balancing and Alignment - Communicate in the workplace to support customers and team - Work effectively in a customer service - sales environment - Develop professionalism. - Comply with health and safety regulations |
| Candidate Details | Name: __________________________________________________________ Registration Number: __________________ Candidate Signature: _____________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: __________________________ Assessor’s code: _________________ Signature of the Assessor: _______________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | What is the Common Rail fuel injection system? |  |  |
|  | How can airlocks affect the performance of a Common Rail diesel system? |  |  |
|  | Why is fuel pressure critical in a Common Rail system? |  |  |
|  | What role do fuel injectors play in the Common Rail system? |  |  |
|  | Which sensors is used to monitor suction Air in a Common Rail system? |  |  |
|  | What is the purpose of a vehicle suspension system? |  |  |
|  | What is the role of a stabilizer bar in suspension? |  |  |
|  | What is the purpose of the shock absorber? |  |  |
|  | What is the primary function of a steering system? |  |  |
|  | What is electric power steering (EPS)? |  |  |
|  | What is the primary purpose of wheel balancing? |  |  |
|  | What is toe-in and toe-out? |  |  |
|  | What is the KPI? |  |  |
|  | What is the difference between Caster and Camber angles? |  |  |
|  | What is the likely cause of steering vibration at high speed? |  |  |
|  | What is PPE? |  |  |
|  | What is the impact of smiling on customer interactions? |  |  |
|  | What is the purpose of using clear headings in written communication? |  |  |
|  | How can body language impact a customer interaction? |  |  |
|  | How does social media engagement improve customer relationships? |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature ___________________