|  | Assessment Evidence Guide For Hand and Machine Embroidery Level-3 Formative Assessment 0723TXT1003 Handle Embroidery Machine Troubleshooting |
| --- | --- |
|  | 04-09 November, 2024 Lahore |
|  | National Vocational & Technical Training Commission |

Instruction Sheet for the Candidate

| Qualification | Hand and Machine Embroidery Level-3 |
| --- | --- |
| Competency Standard | 0723TXT1003 Handle Embroidery Machine Troubleshooting |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within 04 Hrs. time frame (for practical demonstration & assessment): - Rectify thread related issues - Adjust the timing of needle bar - Replace broken needle Activity 1: Set up embroidery machine with a spool of thread and adjust needle bar time. Activity 2: Replace a broken needle on embroidery machine |
| Time: 4 Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | 1. Check the tension of thread on tension post 2. Adjust tension post screw as required 3. Check the bobbin case for thread tension 4. Adjust screw of bobbin with screw driver as required 5. Ensure bobbin is moving anti clock wise 6. Refill the bobbin if thread is finished 7. Loose the needle bar (gaz) with screw driver 8. Set the timing between shuttle and needle bar 9. Tighten the needle bar (gaz) with the screw driver 10. Loosen the needle clamp screw to draw out the needle 11. Attach the needle in the needle bar by keeping cut side of the needle on the correct side (groove) according to machine requirement 12. Tight the needle clamp screw on the needle bar |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Hand and Machine Embroidery Level-3 |
| Competency Standard | 0723TXT1003 Handle Embroidery Machine Troubleshooting |
| Purpose of Assessment | Formative Assessment |
| Assessment Task | - Rectify thread related issues - Adjust the timing of needle bar - Replace broken needle Activity 1: Set up embroidery machine with a spool of thread and adjust needle bar time. Activity 2: Replace a broken needle on embroidery machine |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Check the tension of thread on tension post |  |  |
| - Adjust tension post screw as required |  |  |
| - Check the bobbin case for thread tension |  |  |
| - Adjust screw of bobbin with screw driver as required |  |  |
| - Ensure bobbin is moving anti clock wise |  |  |
| - Refill the bobbin if thread is finished |  |  |
| - Loose the needle bar (gaz) with screw driver |  |  |
| - Set the timing between shuttle and needle bar |  |  |
| - Tighten the needle bar (gaz) with the screw driver |  |  |
| - Loosen the needle clamp screw to draw out the needle |  |  |
| - Attach the needle in the needle bar by keeping cut side of the needle on the correct side (groove) according to machine requirement |  |  |
| - Tight the needle clamp screw on the needle bar |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | Hand and Machine Embroidery Level-3 |
| --- | --- |
| Competency Standard | 0723TXT1003 Handle Embroidery Machine Troubleshooting |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________ Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code: ____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Rectify thread related issues - Adjust the timing of needle bar - Replace broken needle Activity 1: Set up embroidery machine with a spool of thread and adjust needle bar time. Activity 2: Replace a broken needle on embroidery machine |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Check the tension of thread on tension post |  |  |  |  |  |
|  | Adjust tension post screw as required |  |  |  |  |  |
|  | Check the bobbin case for thread tension |  |  |  |  |  |
|  | Adjust screw of bobbin with screw driver as required |  |  |  |  |  |
|  | Ensure bobbin is moving anti clock wise |  |  |  |  |  |
|  | Refill the bobbin if thread is finished |  |  |  |  |  |
|  | Loose the needle bar (gaz) with screw driver |  |  |  |  |  |
|  | Set the timing between shuttle and needle bar |  |  |  |  |  |
|  | Tighten the needle bar (gaz) with the screw driver |  |  |  |  |  |
|  | Loosen the needle clamp screw to draw out the needle |  |  |  |  |  |
|  | Attach the needle in the needle bar by keeping cut side of the needle on the correct side (groove) according to machine requirement |  |  |  |  |  |
|  | Tight the needle clamp screw on the needle bar |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | Hand and Machine Embroidery Level-3 |
| --- | --- |
| Competency Standard | 0723TXT1003 Handle Embroidery Machine Troubleshooting |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature: ___________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: ________________________________________________ Assessor’s code: ____________________________________________________ Signature of the Assessor: _______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |  |
| --- | --- | --- | --- | --- |
|  | Mention two thread related issues |  |  |  |
|  | What is timing of needle bar |  |  |  |
|  | What is a bobbin? |  |  |  |
|  | Which is the correct direction for a bobbin to move? |  |  |  |
|  | How to adjust timing of needle bar? |  |  |  |
|  | What is the difference between spool of thread and bobbin? |  |  |  |
| Feedback to the Candidate |  |  |  |  |
| Candidate’s Signature__________________ Assessor’s Signature _________________ |  |  |  |  |