National Vocational Certificate level-3, Machinist

Curriculum

National Vocational and Technical Training Commission (NAVTTC)

Government of Pakistan

Table of Contents

1.	Introduction	1

1.1. Date of Development	1

1.2. Date of Review and Validation	1

1.3 Entry Requirement	1

1.4. Minimum qualification for teachers	1

1.5. Medium of instruction	1

1.6. Duration of Course	1

1.7. Overall objectives of training program	2

2.	Scheme of Studies	3

3.	Overview of the Curriculum for Level-3 Machinist	3

4.	Detail of Modules.	8

0715MMT3501 Maintain Health and Safety at work place:	8

0715MMT3502 Develop Basic computer operating skills:	12

0715MMT3503 Develop Professionalism:	15

0715MMT3504 Perform off hand grinding:	19

0715MMT3505 Perform Surface Grinding:	22

0715MMT3506 Perform Cylindrical grinding and centerless grinding:	24

0715MMT3507 Perform Tool & Cutter Grinding:	29

0715MMT3508 Perform Milling Operation:	32

0715MMT3509 Generate Gears on Hobbing Machine:	46

0715MMT3510 Perform broaching operation:	52

0715MMT3511 Manufacture Mechanical Projects:	56

5.	List of Personal Protective Equipment	59

6.	List of Tools, Equipment, Machines and Consumables	60

7.	Members of Curriculum Development Committee,	96

# Introduction

Machinist is a multidimensional trade person specialized in machine shop, machine and related cutting & measuring tools and equipment or the maintenance and repair of existing machinery. In order to meet the demand of machine shop, National Vocational and Technical Training Commission (NAVTTC) in collaboration with TVET Sector Support Program (TVET – SSP) has developed National Vocational Qualifications comprising generic, functional and technical competency standards for Machinist occupation. It encompasses a broad range of knowledge and skills related to machining processes, workshop techniques, quality control, safety procedures, and environmental sustainability. The demand for skilled professionals in this field is steadily growing as the industry seeks individuals who can contribute to innovation, process improvement and safe handling of equipment. It aims to establish a framework that defines the essential knowledge, skills, and attitudes required for individuals to perform effectively in this field. These standards will serve as a benchmark for training providers, employers, and learners, ensuring that the workforce is equipped with the competencies necessary to meet industry requirements and contribute to Pakistan's economic growth. Green skills incorporation in TVET aims to build capacity of skilled workforce, which is aware of environmental hazards and accordingly adopts more sustainable and environment friendly practices at the workplace.

| ## 1.1. Date of Development |  |
| --- | --- |
| ## 1.2. Date of Review and Validation |  |
| ## 1.3 Entry Requirement | Level-2 in Machinist |
| ## 1.4. Minimum qualification for teachers | - Should have completed intermediate or equivalent qualifications - Must be a holder of G II certificate in Machinist or Three years DAE in Mechanical Technology. - Must be able to communicate effectively. - Must have at least 4 years teaching experience. |
| ## 1.5. Medium of instruction | Urdu, local language, English |
| ## 1.6. Duration of Course | Total. 600 hours. Theory. 120 hours (20 %) Practical. 480 hours (80 %) |

  - Overall objectives of training program

The Machinist qualification of level-3 consists of 20 % Theory and 80 % Practical details and having the following Modules. Details are as follows:

0715MMT3501 Maintain Health and Safety at work place

0715MMT3502 Develop Basic computer operating skills

0715MMT3503 Develop Professionalism

0715MMT3504 Perform off hand grinding

0715MMT3505 Perform Surface Grinding

0715MMT3506 Perform Cylindrical grinding and centerless grinding

0715MMT3507 Perform Tool & Cutter Grinding

0715MMT3508 Perform Milling Operations

0715MMT3509 Generate Gears on Hobbing Machine

0715MMT3510 Perform broaching operation

0715MMT3511 Workshop Projects-II

# Scheme of Studies

| Module Code | Name of Competency | NVQF Level | Category | Contact Hour |  |  | Credit | Periods per week |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Th | Pr | Total |  | Th | Pr | Total |
| 0715MMT3501 | Maintain Health and Safety at work place | 3 | Generic | 7 | 15 | 22 | 2.2 | 0.44 | 0.94 | 1.38 |
| 0715MMT3502 | Develop Basic computer operating skills | 3 | Generic | 10 | 15 | 25 | 2.5 | 0.63 | 0.94 | 1.56 |
| 0715MMT3503 | Develop Professionalism | 3 | Generic | 14 | 21 | 35 | 3.5 | 0.88 | 1.31 | 2.19 |
| 0715MMT3504 | Perform off hand grinding | 3 | Technical | 7 | 15 | 22 | 2.2 | 0.44 | 0.94 | 1.38 |
| 0715MMT3505 | Perform Surface Grinding | 3 | Technical | 9 | 21 | 30 | 3.0 | 0.56 | 1.31 | 1.88 |
| 0715MMT3506 | Perform Cylindrical and centerless grinding | 3 | Technical | 10 | 30 | 40 | 4.0 | 0.63 | 1.88 | 2.50 |
| 0715MMT3507 | Perform Tool & Cutter Grinding | 3 | Technical | 9 | 27 | 36 | 3.6 | 0.56 | 1.69 | 2.25 |
| 0715MMT3508 | Perform Milling Operations | 3 | Technical | 22 | 150 | 172 | 17.2 | 1.38 | 9.38 | 10.75 |
| 0715MMT3509 | Generate Gears on Hobbing Machine | 3 | Technical | 12 | 30 | 42 | 4.2 | 0.75 | 1.88 | 2.63 |
| 0715MMT3510 | Perform broaching operation | 3 | Technical | 12 | 30 | 42 | 4.2 | 0.75 | 1.88 | 2.63 |
| 0715MMT3511 | Workshop Projects-II | 3 | Technical | 8 | 126 | 134 | 13.4 | 0.50 | 7.88 | 8.38 |
| Total |  |  |  | 120 | 480 | 600 | 60 | 7.5 | 30.0 | 37.5 |

# Overview of the Curriculum for Level-3 Machinist

| Module Title | Learning Units | Theory hours | Workplace hours | Timeframe of modules |
| --- | --- | --- | --- | --- |
| 0715MMT3501 Maintain Health and Safety at work place | - Ensure personal protective equipment (PPE) - Protect Tools and Equipment - Maintain First aid Box - Maintain Fire Extinguisher - Adopt Environmental Regulation - Adopt company policies and procedures - Implement safety programme - Prepare for emergencies - Respond to emergencies | 7 | 15 | 22 |
| 0715MMT3502 Develop Basic computer operating skills | - Operate MS word - Operate MS Excel - Operate MS Power Point - Perform Browsing - Print Document | 10 | 15 | 25 |
| 0715MMT3503 Develop Professionalism | - Demonstrate work ethics - Resolve problems or disagreements with others - Participate in professional development - Work in team - Work independently - Interpret documentation - Interpret rules and regulation of regulatory authorities | 14 | 21 | 35 |
| 0715MMT3504 Perform off hand grinding | - CU1. Adjust Grinding wheel - CU2. Adjust tool Rest - CU3. Prepare a single point cutting tool for Basic machining purposes - Dress the wheel - CU5. Perform Angle Grinding for finishing | 7 | 15 | 22 |
| 0715MMT3505 Perform Surface Grinding | - Determine job requirements and sequence of operations - Operate surface grinding - Dress the Wheel | 9 | 21 | 30 |
| 0715MMT3506 Perform Cylindrical grinding and centerless grinding | - Determine job requirements and sequence of operations - Adjust Grinding wheel - Operate Universal cylindrical grinding - CU4.Operate centerless grinding | 10 | 30 | 40 |
| 0715MMT3507 Perform Tool & Cutter Grinding | - Adjust Grinding wheel - Adjust tool Rest - Operate the Tool and cutter grinding | 9 | 27 | 36 |
| 0715MMT3508 Perform Milling Operations | - Prepare Blank for Generating the Gear - Select Tools and Equipment for Gear Cutting - Produce a square shaped work piece - Generate spur gear (Direct Indexing) On Milling Machine. - Generate spur gear (Differential Indexing) On Milling Machine. - Generate Helical Gear on Milling Machine - Perform slotting or grooving on work piece - Perform drilling or boring using milling machine - Cut T- slots on Milling machine - Cut Bevel gear on milling machine. - Practice of spur rack cutting. - Practice of helical rack cutting | 22 | 150 | 172 |
| 0715MMT3509 Generate Gears on Hobbing Machine | - Prepare Blank for Generating the Gear - Select Tools and Equipment for Gear Cutting - Set Hobbing Machine for Operations - Carry out Hobbing Operations for spur Gear Generating - Carry out Hobbing Operations for Helical Gear Generating | 12 | 30 | 42 |
| 0715MMT3510 Perform broaching operation | - Machine Irregular Surfaces - Make keyways - Make internal gears | 12 | 30 | 42 |
| 0715MMT3511 Workshop Projects-II | - Select The Relevant Project. - Identify The Right Material. - Select The Right Machining Process and Sequence. - Calculate The Cost of Machining and Material. - Make The Project. - Ensure Dimensional Accuracy and Quality as Per Drawing or Standard. | 8 | 126 | 134 |
| TOTAL |  | 120 | 480 | 600 |

# Detail of Modules.

## 0715MMT3501 Maintain Health and Safety at work place:

Objective: This module covers the knowledge and  skills required to learn about personal protective equipment (PPE), protect tools and equipment, maintain first aid box, maintain fire extinguisher, adopt environmental regulation, adopt company policies and procedures, implement safety programs, prepare for emergencies, and respond to emergencies.

Duration:  23 Hours		                       	Theory: 07 Hours		     			Practice: 15 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Ensure personal protective equipment (PPE) | Trainee will be able to: - Arrange PPEs as per requirement - Wear proper PPE as per nature of job - Store PPE at appropriate place after use | - Selection of Appropriate PPE - Storage - Personal Protective Equipment (PPE) uses Practical Activity: Practice of using PPE’s as per nature of job | Theory-01Hrs Practical- 03Hrs Total- 4Hrs | PPE’s | Class Room and workshop |
| - Protect Tools and Equipment | Trainee will be able to: - Ensure insulation of tools and equipment - Store tools and equipment safely - Clean tools on a regular basis before stacking | - Maintenance Procedures - Storage and Handling - Safety Practices Practical Activity: Clean Tools & Equipment and store them in storage | Theory-01Hrs Practical-1.5Hrs Total- 2.5Hrs | PPE’s | Class Room and workshop |
| - Maintain First aid Box | Trainee must be able to: - Ensure availability of first aid box - Check first aid box for requisite emergency medicines - Check expiry date of medicines - Perform first aid treatment against electric shock - Perform first aid treatment / bandages against minor injuries | - first-aid-box items. - First-Aid-Box use. - Clean and Organized Practical Activity: Perform mock exercise for first aid treatment / bandages against minor injuries | Theory-0.5Hrs Practical-1.5Hrs Total- 02Hrs | First aid box | Class Room and workshop |
| - Maintain Fire Extinguisher | Trainee must be able to - Check expiry of fire extinguisher - Operate fire extinguisher - Replace fire extinguisher | - Introduction to Fire Extinguisher, types and its uses - Types of fire extinguishing materials. Practical Activity: Operate fire extinguisher on fire correctly | Theory-01Hrs Practical-1.5Hrs Total- 2.5Hrs | Fire Extinguisher | Class Room and workshop |
| - Adopt Environmental Regulation | Trainee must be able to - Locate applicable permits on job site - Ensure work friendly environment - Follow environmental regulations | - Environmental Regulations - Compliance Requirements Practical Activity: Interpret environmental regulations | Theory-0.5Hrs Practical-1.5Hrs Total- 02Hrs | Environmental Regulation Manuals | Class Room |
| - Adopt company policies and procedures | Trainee must be able to - Ensure company policy and procedures - Follow company procedures | - company policy and procedures. Practical Activity: - Interpret the company procedures | Theory-0.5Hrs Practical-1.5Hrs Total- 02Hrs | Company Policies manual | Class Room |
| - Implement safety programs | Trainee must be able to - Take required health and safety training - Implement work place hazardous materials information system - Adopt first aid and cardio respiratory resuscitation (CPR) | - Occupational Health & Safety Regulations. - Safety Equipment and Resources - Incident Reporting and Investigation - Safety Culture and Communication Practical Activity: De Develop a report of an incident at workshop | Theory-01Hrs Practical-1.5Hrs Total- 2.5Hrs | Health and safety manuals | Class Room and workshop |
| - Prepare for emergencies | Trainee must be able to - Take emergency response training - Ensure emergency response exercises - Adopt first aid and Cardio Respiratory Resuscitation (CPR) | - Emergency Preparedness - Assessment and Decision-Making - Safety and Evacuation Procedures Practical Activity: Perform mock exercise for Resuscitation procedure | Theory-01Hrs Practical-1.5Hrs Total- 2.5Hrs |  | Class room |
| - Respond to emergencies | Trainee must be able to - Follow emergency plan - Communicate instructions - Assess risk and determine course of action - Operate emergency equipment and supplies | - Emergency Communication - First Aid and Medical Response - Use of emergency equipment - Post-Emergency Actions - Training and Drills Practical Activity: Perform Emergency evacuation through Exit Point | Theory-0.5Hrs Practical-1.5Hrs Total- 02Hrs |  | Class room/ Workshop |

## 0715MMT3502 Develop Basic computer operating skills:

Objective: This module covers the knowledge and  skills required to Operate MS word, Operate MS Excel, Operate MS Power Point, Perform Browsing, Print Document.

Duration: 25 Hours					Theory: 10 Hours					Practice: 15 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Operate MS word | Trainee will be able to: - Perform Microsoft basic commands in MS word - Open File - Format a file - Font (Type/size/bold/Italic) - Header Footer - Page number - Insert pics / table/hyperlink - Save a File - Save a folder | - Introduction to MS Word - Description User Interface - Procedure of writing a document and saving it. Practical Activity: Prepare a word file including inserting pics and saving it in a folder. | Theory-3Hrs Practical-03Hrs Total- 6Hrs | Computer/Laptop With Microsoft Installed | Class Room and workshop |
| - Operate MS Excel | Trainee will be able to: - Perform basic commands in Microsoft MS Excel - Open a worksheet - Sum, division, multiplication, subtract, filter functions - If functions - Basic calculations - Table and graphs - Save a worksheet/folder | - Introduction to MS Excel - Description User Interface of MS excel - Procedure of performing basic functions on worksheet Practical Activity: Making an attendance sheet, calculating attendance percentage and showing it in graph | Theory-3Hrs Practical-03Hrs Total- 6Hrs | Computer/Laptop With Microsoft Installed | Class Room and workshop |
| - Operate MS Power Point | Trainee must be able to: - Prepare Microsoft power point presentation with basic commands - Make a power point file - Insert pics/table/hyperlink - Design a theme for slides - Save a power point file | - Introduction to MS Power Point - Description User Interface of MS Power Point - Techniques of making PPT slides. Practical Activity: Making a PPT file on implementing Safety Precautions in Workshop. | Theory-2Hrs Practical-03Hrs Total- 5Hrs | Computer/Laptop With accessories, Microsoft Installed | Class Room and workshop |
| - Perform Browsing | Trainee must be able to - Perform browsing on the internet as per needs - Perform search online on new trends in the market with the help of internet | - Introduction to Web Browsing - Basic Browsing Actions - Interacting with Websites Practical Activity: Search latest machinery of machinist and downloading the file. | Theory-01Hrs Practical-3Hrs Total- 4Hrs | Computer/Laptop With accessories, Internet access | Class Room and workshop |
| - Print Document | Trainee must be able to - Select Printer - Select page setup - Print relevant pages | - Description of Printer. - Procedure of page setup - Procedure of taking a print. Practical Activity: Print a document. | Theory-01Hrs Practical-3Hrs Total- 4Hrs | Computer/Laptop With accessories, Printer | Class Room |

## 0715MMT3503 Develop Professionalism:

Objective: This module covers the knowledge and  skills required to Demonstrate  work ethics, Resolve problems or disagreements with others, Work in team, Work independently, Interpret documentation, Interpret rules and regulation of regulatory authorities.

Duration: 35 Hours					Theory: 14 Hours					Practice: 21 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Demonstrate work ethics | Trainee must be able to - Follow principles of work ethics in all situations - Adopt professional behaviour | - Introduction to Work Ethics - Core Elements of Work Ethic - Professionalism Practical Activity: Perform mock exercise regarding principles of work ethics. | Theory-02Hrs Practical-03Hrs Total- 05Hrs |  | Class Room and work shop |
| - Resolve problems or disagreements with others | Trainee must be able to - Communicate effectively - Adopt peaceful approach - Determine cause of problem or disagreement - Discuss and resolve issues | - Understanding Conflict - Identifying the Causes of Conflict - Effective Communication in Conflict Resolution - Problem-Solving Techniques - Resolving Conflict through Collaboration Practical Activity: - Role-Play: Conflict Resolution in the Workplace - Case Study Analysis: Problem-Solving in Conflict Situations | Theory-02Hrs Practical-03Hrs Total- 05Hrs | Case study | Class Room and work shop |
| - Participate in professional development | Trainee must be able to - Assess own knowledge and skills - Acquire information about training opportunities - Learn through various methods, such as on job training, reading, courses and co-workers | - Importance of Professional Development - Identifying Career Goals - Continuing Education/Training - Developing Soft Skills - Receiving and Acting on Feedback Practical Activity: Personal Development Plan Workshop | Theory-02Hrs Practical-03Hrs Total- 05Hrs |  | Class Room and work shop |
| - Work in team | Trainee must be able to - Work as a team member to achieve common goals - Keep open mind - Participate in work place meetings - Communicate accurately and clearly - Co-ordinate job related activities - Cooperate with others | - Team Concepts - Roles and Responsibilities - Effective Communication - Collaboration and Cooperation - Conflict Resolution - Setting and Achieving Team Goals - Decision-Making as a Team Practical Activity: Team Roles and Responsibilities Workshop | Theory-02Hrs Practical-03Hrs Total- 05Hrs |  | Class Room and work shop |
| - Work independently | Trainee must be able to - Confirm and clarify assignment - Take initiative, anticipate and prepare for next steps in job - Identify and resolve potential and actual problems - Communicate with other site personnel - Complete assignment | - Independent Work - Setting Personal Goals - Time Management - Self-Discipline and Accountability - Problem-Solving Skills - Decision-Making - Effective Communication Practical Activity: - Personal Goal Setting Workshop - Self-Assessment Exercise | Theory-02Hrs Practical-03Hrs Total- 05Hrs |  | Class Room |
| - Interpret documentation | Trainee must be able to - Access and maintain documents - Provide complete, legible and accurate information in documents - Interpret equipment inspection documentation from previous shifts before conducting pre-operational inspection | - Different Types of Documentation - Reading Identifying Key Information - Techniques for complete, legible and accurate documents Practical Activity: Document Interpretation Workshop | Theory-02Hrs Practical-03Hrs Total- 05Hrs |  | Class Room |
| - Interpret rules and regulation of regulatory authorities | Trainee must be able to - Follow Health and Safety Rules - Ensure environmental safety - Follow compliance of IMEI Rules. - Ensure workplace safety by following safety standards - Ensure safety while operating electricity. | - Description about Reading and Interpreting Regulations - Interpreting Health and safety rules, environmental safety, IMEI rules, safety standards. Practical Activity: Perform Regulatory Interpretation Exercise | Theory-02Hrs Practical-03Hrs Total- 05Hrs |  | Class Room |

## 0715MMT3504 Perform off hand grinding:

Objective: This module covers the knowledge and  skills required to Adjust Grinding wheel, Adjust tool Rest, Prepare a single point cutting tool for Basic machining purposes, Dress the wheel, Perform Angle Grinding for finishing.

Duration: 22 Hours					Theory: 07 Hours					Practice: 15 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Adjust Grinding wheel | Trainee will be able to: - Select the right tool for job - Remove the safety guard of grinding wheel. - Select the wheel type according to material/speed. - Mount the wheel on the spindle. - Balance the wheel - Put back the safety covers. - Ensure that wheel is properly mounted | - Grinding Wheel Basics - Types of grinding machines and grinding wheels - Safety Precautions Practical Activity: Mount the grinding wheel and balance it. | Theory-02Hrs Practical-03Hrs Total- 05Hrs | Grinding wheel | Class Room and workshop |
| - Adjust tool Rest | Trainee will be able to: - Select the right tool for job - Remove the safety guard of grinding wheel. - Mount the tool rest. - Balance the wheel - Put back the safety covers - Ensure that wheel is properly mounted | - Introduction Tool Rest - Safety Considerations - Describing safe distance from grinding wheel for tool rest - the Impact of Improper Adjustment Practical Activity: Set the tool rest in right position. | Theory-01Hrs Practical-03Hrs Total- 04Hrs |  | Class Room and workshop |
| - Prepare a single point cutting tool for Basic machining purposes | Trainee will be able to: - Select marking tools - Cut the material as per requirement. - Perform surface finishing with file - Level the surface with tri-square - Give the angle to the tool according to cutting requirement. | - Description of basic cutting tools especially Single-Point Cutting Tools - Types of Materials for Cutting Tools - Tool Grinding Techniques - Safety and geometry of single point cutting tool Practical Activity: Prepare a single point cutting tool according to drawing. | Theory-02Hrs Practical-03Hrs Total- 05Hrs | Mild steel for single point cutting tool | Class Room and workshop |
| - Dress the wheel | Trainee must be able to - Select dressing tools - Ensure that wheel is properly mounted. - Dress tool to remove the material from grinding wheel which is creating problems. - Balance the grinding wheel. | - Wheel Dressing and the types of wheel dresser - Types of grinding discs - Safety Compliance Practical Activity: Dress grinding wheel with diamond dresser. | Theory-01Hrs Practical-03Hrs Total- 4Hrs | Wheel dresser | Class Room and workshop |
| - Perform Angle Grinding for finishing | Trainee must be able to - Select marking tools - Cut the material as per requirement. - Perform surface finishing with angle grinder machine - Level the surface with tri-square | - Angle Grinding - Techniques for Achieving a Uniform Finish - Safety Practices in Finishing Practical Activity: Perform angle grinding according to drawing. | Theory-01Hrs Practical-03Hrs Total- 04Hrs | Square or rectangle piece of mild steel | Class Room and workshop |

## 0715MMT3505 Perform Surface Grinding:

Objective: This module covers the knowledge and  skills required to Determine job requirements and sequence of operations, Operate surface grinding, Dress the Wheel.

Duration: 30 Hours					Theory: 09 Hours					Practice: 21 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Determine job requirements and sequence of operations | Trainee will be able to: - Interpret and understand drawings, instructions and specifications. - Determine sequence of operations including job specifications and set-up for maximum efficiency - Select appropriate grinding wheel for job. - Mounted and positioned grinding wheel correctly | - Drawing and surface symbols for grinding - Demonstrating sequence of operation for surface grinding operations Practical Activity: Inspect the positioning. Of grinding wheel | Theory-01Hrs Practical-03Hrs Total- 04Hrs |  | Class Room and workshop |
| - Operate surface grinding | Trainee will be able to: - Select the suitable size and type of grinding wheel. - Mount the work piece over the holding devices to ensure proper clamping. - Dress the wheel with diamond tip if required. - Identify reference points on work piece before grinding. - Adjust depth of cut according to speed of machine table. - Provide coolant continuously to avoid over heating of the job. - Observe personal and workplace safety | - Surface Grinding Process - Methods and techniques for surface grinding - Types of Surfaces Grinders - Selecting right standing position during grinding - Work holding methods which include: - Magnet Table - Vice - Angle Plate - Machine base - Importance of using coolant - Fine Grinding and Finishing - Specific safety precautions and guidelines. Practical Activity: Grind square and step job on surface grinding machine as prescribed method | Theory-07Hrs Practical-15Hrs Total- 22Hrs | Mild steel | Class Room and workshop |
| - Dress the Wheel | Trainee must be able to: - Select dressing tools - Ensure that wheel is properly mounted. - Dress tool to remove the material from grinding wheel which is creating problems. - Balance the grinding wheel. | - Describe the purposes of dressing - Describe the different type of dressing tools. - Dressing Technique - Inspecting the Wheel Practical Activity: Carry out dressing of the wheel | Theory-01Hrs Practical-3Hrs Total- 4Hrs |  | Class Room and workshop |

## 0715MMT3506 Perform Cylindrical grinding and centerless grinding:

Objective: This module covers the knowledge and  skills required to Determine job requirements and sequence of operations, Adjust Grinding wheel, Operate Universal cylindrical grinding, Operate centerless grinding.

Duration: 40 Hours					Theory: 10 Hours					Practice: 30 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Determine job requirements and sequence of operations | Trainee will be able to: - Interpret and understand drawings, instructions and specifications. - Determine sequence of operations including job specifications and set-up for maximum efficiency - Select appropriate grinding wheel for job. - Mounted and positioned grinding wheel correctly | - Universal Cylindrical grinding machine, its attachments and functions. - internal and external grinding methods - Interpret basic drawings Practical Activity: Mount the appropriate grinding wheel | Theory-02Hrs Practical-03Hrs Total- 05Hrs |  | Class Room and workshop |
| - Adjust Grinding wheel | Trainee will be able to: - Select the right tool for job - Select the wheel type according to material/speed. - Set the wheel on tool post - Balance the wheel - Ensure that wheel is properly mounted | - different kinds and type of grinding wheel for internal and external grinding & its applications. - Testing and mounting of grinding wheel. Practical Activity: Carry out wheel balancing | Theory-01Hrs Practical-03Hrs Total- 04Hrs |  | Class Room and workshop |
| - Operate Universal cylindrical grinding | Trainee must be able to: - Select the suitable size and type of grinding wheel. - Mount work piece according to procedure (between two centers, chuck, collet, face plate). - Ensure the grinding wheel is balanced. - Follow suitable method for universal cylindrical grinding to ensure work specifications. - Provide coolant continuously to avoid overheating of job. - Observe personal and workplace safety. | - Apply proper grinding methods and techniques for Universal Cylindrical grinding. (Taper grinding, parallel grinding, step grinding.) - Use of different kinds of measuring tools (inside, outside micrometer, snap plug gauge taper plug and bush gauge. - mounting of workpiece on required holding device. (Between Two Centers; Chuck; Collet; and Face Plate). - Demonstrate the setting of table feed and depth of cut. - Identify the importance of continuous use of coolants, - Adopt Safety on Universal Cylindrical grinding machine Practical Activity: - Carry-out wheel balancing and mounting as per given instructions - Carry-out wheel dressing as per prescribed method - Perform Taper grinding, parallel grinding and step grinding on workpiece, etc. - Perform internal grinding as per drawing. | Theory-04Hrs Practical-16Hrs Total- 20Hrs |  | Class Room and workshop |
| - Operate centerless grinding | Trainee must be able to - Select the suitable size and type of grinding wheel. - Mount work piece with a blade instead of by centers or chucks. - Ensure the grinding wheel is balanced. - Follow suitable method for universal cylindrical grinding to ensure work specifications. - Apply coolant continuously to avoid overheating of job. - Observe personal and workplace safety | Centre less grinding machine and its attachments - procedure of grinding of workpiece on Centreless Grinding Machine - Techniques of Grinding of cylindrical pin/hollow pipe on centreless grinding machine as prescribed method Practical Activity: Grinding of cylindrical pin on centreless grinding machine as per prescribed method | Theory-03Hrs Practical-08Hrs Total- 12Hrs | PIn | Class Room and workshop |

## 0715MMT3507 Perform Tool & Cutter Grinding:

Objective: This module covers the knowledge and  skills required to Adjust Grinding wheel, Adjust tool Rest, Operate the Tool and cutter grinding.

Duration: 36 Hours						Theory: 09 Hours				Practice: 27 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Adjust Grinding wheel | Trainee will be able to: - Select the right tool for job - Remove the safety guard of grinding wheel. - Select the wheel type according to material/speed. - Mount the wheel on the spindle. - Balance the wheel - Put back the safety covers. - Ensure that wheel is properly mounted | - Description of different kinds and types of grinding wheel for grinding & its applications. - Procedure and techniques to adjust grinding wheel correctly. - Implementation of safety precautions. Practical Activity: Mount the appropriate wheel and adjust it in accurate position. | Theory-01Hrs Practical-03Hrs Total- 04Hrs |  | Class Room and workshop |
| - Adjust tool Rest | Trainee will be able to: - Select the right tool for job - Remove the safety guard of grinding wheel. - Mount the tool rest. - Balance the wheel - Put back the safety covers. - Ensure that wheel is properly mounted. | - Procedure of mounting the tool rest accurately - Ensure safety precautions Practical Activity: Mount the tool rest in right position. | Theory-01Hrs Practical-03Hrs Total- 04Hrs | grinding wheels, | Class Room and workshop |
| - Operate the Tool and cutter grinding | Trainee must be able to: - Select the suitable size, type and shape of grinding wheel. - Mount work piece onto correct attachment for required procedure. - Adjust the attachments according to different types of tools and cutter grinding. - Follow procedure for sharpening of tools and cutter that is safe and appropriate. - Observe personal and safety precautions. | - Description of Tool/cutter grinding machines and its attachments. - Techniques to mounting the Cutters on required holding device. (Collets; and setting of spiral attachment) - Procedure of Grinding of milling cutter(s) and special tools on tool/cutter grinding machine as per prescribed method - Apply proper grinding methods and techniques for Tool/Cutter grinding. - Implementation of safety precautions. - Measurement systems - Apply proper grinding methods and techniques for Tool/Cutter grinding. Practical Activity: Test and mount wheel according to job requirement and Grinding end mill cutter, side and face cutter, shell end mill cutter and plain milling cutter. | Theory-07Hrs Practical-21Hrs Total- 28Hrs | end mill cutter, side and face cutter, shell end mill cutter and plain milling cutter | Class Room and workshop |

## 0715MMT3508 Perform Milling Operation:

Objective: This module covers the knowledge and  skills required to Prepare Blank for Generating the Gear, Select Tools and Equipment for Gear Cutting, Produce a square shaped work piece, Generate spur gear (Direct Indexing) On Milling Machine, Generate Helical Gear on Milling Machine, Perform slotting or grooving on work piece, Perform drilling or boring using milling machine, Milling a T-slot, Bevel gear cutting on milling machine, Practice of spur rack cutting, Practice of helical rack cutting.

Duration:  172 Hours			Theory: 22 Hours				Practice: 150 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Prepare Blank for Generating the Gear | Trainee will be able to: - Interpret drawing and arrange the material according to job requirement - Prepare the work-piece by required machining (sawing and filing etc.) and get it ready for turning the blank - Check and verify the dimensions of blank for generating gear as per drawing | - Personal protective equipment and workplace safety - Introduction to various milling machines and their types - Interpret basic drawings and cutting required material on power hacksaw. - measuring tools - procedures of drilling and boring operation on lathe. - Calculate cutting speed, RPM and feed for drilling and boring operations on lathe. Practical Activity: - Interpret Drawing. - Cutting the job by power hacksaw. - To accurately measure the length, width, and thickness of work piece using a Vernier caliper. Mount workpiece on lathe Centre to Centre, complete the length and outside diameter, and chamfer the both end of blank on 15 degrees. | Theory-02Hrs Practical-12Hrs Total- 14Hrs | Blank of Teflon, Aluminum scrape or bar | Class Room and workshop |
| - Select Tools and Equipment for Gear Cutting | Trainee will be able to: - Select the material, type, shape and size of cutter(s) according to the job requirements Arrange the measuring instruments and holding devices to attain accuracy of the work as per prescribed method | - Types of tools and equipment for gear cutting - Introduction to gear generating - Safety precautions for gear cutting. - Housekeeping of tools and equipment Practical Activity: - Adjust speed and feed of milling machine. - Mount milling cutter on milling machine - Set indexing head on machine table | Theory-02Hrs Practical-9Hrs Total- 11Hrs |  | Class Room and workshop |
| - Produce a square shaped work piece | Trainee must be able to: - Identify safety hazards related with milling operations and take appropriate steps to avoid any injury or accident. - Dial the machine vice according to job requirement. - Select cutters and set machine as per requirements. - Mount cutters and work piece in the machine. - Produce a part matching the process plan and the part print specifications. - Check quality of the component at suitable intervals. Shut down the machine at safe position after finishing the work. | - Introduction to standard accessories, tooling attachments and work holding devices Types of milling cutter and their functions - square milling procedure. Practical Activity: - Mounting and setting up of work-holding devices - Mount suitable milling cutter on machine. - Mount machine vice and dial with Dial indicator. - Perform squaring as per drawing. Perform cleaning and oiling of machine after completing the work. | Theory-02Hrs Practical-12Hrs Total- 14Hrs | Square shape job of mild steel/ Teflon/ aluminum | Class Room and workshop |
| - Generate spur gear (Direct Indexing) On Milling Machine. | Trainee must be able to - Select gear cutter and indexing plate on a milling machine - Mount indexing plate on indexing head. - Centre indexing head and its tail stock. - Fix indexing head and tail stock on milling table. - ENGAGE worm shaft from worm wheel - Adjust speed feed and direction of the cutter. - Mount Gear blank on mandrel. - Hold one side of mandrel on chuck of indexing head and other side in tail stock - Start machine and carry out cutter at zero point vertically. - Carry out cutter at zero point horizontally. - Apply depth for rough cut and engage machine automatically in longitudinal direction - Move table back at zero point. - Apply full depth for final cut and engage machine automatically in forward direction. Repeat the process simultaneously until tooth is obtained. | - spur gear design - indexing plate and its types - Calculate spur gear pitch dia, outside dia, root dia, depth. - direct indexing method Practical Activity: - Fix indexing head and tail stock on milling table and Centre - Mount Gear blank on mandrel - Generate spur gear on the job through direct indexing method - Repeat the process till the completion of required no. of teeth - Finish and deburr the job. | Theory-02Hrs Practical-15Hrs Total- 17Hrs | Gear Blank of mild steel/ Teflon/ aluminum | Class Room and workshop |
| - Generate spur gear (Differential Indexing) On Milling Machine. | Trainee must be able to - Select gear cutter and indexing plate on a milling machine - Mount indexing plate on indexing head. - Centre indexing head and its tail stock. - Fix indexing head and tail stock on milling table. - ENGAGE worm shaft from worm wheel - Adjust speed feed and direction of the cutter. - Mount Gear blank on mandrel. - Hold one side of mandrel on chuck of indexing head and other side in tail stock - Start machine and carry out cutter at zero point vertically. - Carry out cutter at zero point horizontally. - Apply depth for rough cut and engage machine automatically in longitudinal direction - Move table back at zero point. - Apply full depth for final cut and engage machine automatically in forward direction. Repeat the process simultaneously until tooth is obtained. | - Types of milling operation - differential indexing method - Techniques to use differential indexing method - Calculate spur gear pitch dia, outside dia, root dia, depth. Practical Activity: - Fix indexing head and tail stock on milling table and Centre - Mount Gear blank on mandrel - Generate spur gear on the job through direct differential indexing method - Repeat the process till the completion of required no. of teeth - Finish and deburr the job. | Theory-03Hrs Practical-15Hrs Total- 18Hrs | Gear Blank of mild steel/ Teflon/ aluminum | Class Room and workshop Class Room and workshop |
| - Generate Helical Gear on Milling Machine | Trainee must be able to - Select a indexing plate to machine a helical gear on a manual machine, - Mount gear set to engage lead screw and indexing head spindle - Centre indexing head and its tail stock. - Fix indexing head and tail stock on milling table. - Adjust speed feed and direction of the cutter. - Mount Gear blank on mandrel. - Hold one side of mandrel on chuck of indexing head and other side in tail stock - Start machine and carry out cutter at zero point vertically. - Carry out cutter at zero point horizontally. - Apply depth for rough cut and engage machine automatically in longitudinal direction - Move table back at zero point. - Apply full depth for final cut and engage machine automatically in forward direction. Repeat the process simultaneously until tooth is obtained. | - helical gear - Calculate Helical Gear Pitch dia, outside dia, root dia, Depth and Lead for spiral angle. - Calculate the indexing as per No. of tooth - Calculate gear train set on indexing head and set table on required angle Practical Activity: - Fix indexing head and tail stock on milling table and Centre - Mount Gear blank on mandrel - Hold one side of mandrel on chuck of indexing head and other side in tail stock and Centre both - Ensure the setting of indexing plate and sector Aram. - Mounting of module cutter and set Centre position of gear blank. - Take the touching point for depth of cut. - Perform indexing after each tooth with care. Complete gear cutting and check with tooth Vernier | Theory-02Hrs Practical-15Hrs Total- 17Hrs | Gear Blank of mild steel/ Teflon/ aluminum | Class Room and workshop |
| - Preform slotting or grooving on work piece | - Identify safety hazards related with milling operations and take appropriate steps to avoid any injury or accident. - Set the work piece in machine vice according to procedure. - Select the appropriate cutter as per specifications. - Adjust the milling cutter for slotting and grooving. - Determine the touching point of the work piece. - Produce slotting or grooving on the work piece to the required quality. - Check quality of the component at suitable intervals. - Shut down the machine at safe position after finishing the work. Observe personal and workplace safety at all time. | - Description of slotting and grooving - Techniques of slotting or grooving on milling - Interpret information given in the engineering drawings and job specifications Practical Activity: - Mount arbor and Grooving or slotting milling cutter for Grooving or slotting. - Mount machine vice and dial with Dial indicator. - Perform clamping for Grooving or slotting as per proper procedure. - Follow proper Grooving or slotting procedure. Perform Deburring and check width/ depth of Grooving or slotting size | Theory-02Hrs Practical-12Hrs Total- 14Hrs | Raw material of mild steel/ Teflon/ aluminum | Class Room and workshop |
| - Perform drilling or boring using milling machine | - Identify safety hazards related with milling operations and take appropriate steps to avoid any injury or accident. - Select drill or boring tools according to drawings. - Mount and set the required work-holding devices, work piece and cutting tools. - Adjust the RPM of machine according to the standard chart. - Perform the boring operation according to the drawing. - Check quality of the component produced at different intervals. - Shut down the machine at safe position after finishing the work. Observe personal and workplace safety at all time. | - Introduction to vertical milling machine - Interpret information given in the engineering drawings and job specifications. - Differentiate between drilling and boring - Safety precautions during drilling and boring on milling machine Practical Activity: - Mount drill holding- devices as per requirement - Set cutting speed, RPM and feed for drilling - Perform setting the bore/drill bit in drill bore. - Perform boring/drilling as per procedures. Check boring/drilling size with inside micrometer and plug gauge | Theory-02Hrs Practical- 12Hrs Total- 14Hrs | Raw material of mild steel/ Teflon/ aluminum | Class Room and workshop |
| - Cut T-slot on Milling machine | - Layout position of a T slot. - Square vertical milling machine with machine table. - Mount work in milling machine. - Machine the center slot to proper depth of T slot by end mill. - Remove end mill and mount proper t slot cutter. Machine lower part of the slot. | - Procedure of T slotting - Interpret information given in the engineering drawings and job specifications Practical Activity: - Mount machine vice and dial with Dial indicator - Mount work piece, end mill cutter and machine the center slot - Remove end mill cutter and mount proper T slot cutter for machining the lower part of the slot. Perform Deburring and check width/ depth slotting size | Theory-01Hrs Practical-12rs Total- 13Hrs | Raw material of mild steel/ Teflon/ aluminum | Class Room and workshop |
| - Cut Bevel gear on milling machine. | - Cut The materials to size - Cut workpiece with lathe that is shaped into a gear blank. - Cut gear with a Conflux generator. Remove Burrs on the teeth with a deburring machine. | - Bevel gear design - Interpret information given in the engineering drawings and job specifications Practical Activity: - Mount machine vice and dial with Dial indicator - Mount work piece and cutter Cut gear with conflux generator | Theory-02Hrs Practical-12Hrs Total- 14Hrs | Raw material of mild steel/ Teflon/ aluminum | Class Room and workshop |
| - Practice of spur rack cutting. | - Hold the work piece in milling machine. - Hold milling cutter in rack milling attachment. - Hold cutter at 90 degrees for spur cutting. - Move the table for each tooth by rack indexing attachment. Continue operation until required length is obtained. | - Concept of spur rack - Procedure of rack milling - Interpret information given in the engineering drawings and job specifications Practical Activity: - Mount machine vice, cuter and work piece properly - Perform Deburring and check measurement as per drawing Cleaning of machine | Theory-1Hrs Practical-12Hrs Total- 13Hrs | Rectangle job mild steel/ Teflon/ aluminum | Class Room and workshop |
| - Practice of helical rack cutting | - Hold the work piece in milling machine. - Hold milling cutter in rack milling attachment. - Hold cutter at some angle for helical cutting. Move the table for each tooth by rack indexing attachment. Continue operation until required length is obtained. | - Procedure of helical rack - Concept of rack indexing - Interpret information given in the engineering drawings and job specifications Practical Activity: Hold cutter at specific angle for helical cutting | Theory-1Hrs Practical-12Hrs Total- 13Hrs | Rectangle job mild steel/ Teflon/ aluminum | Class Room and workshop |

## 0715MMT3509 Generate Gears on Hobbing Machine:

Objective: This module covers the knowledge and  skills required to Prepare Blank for Generating the Gear, Select Tools and Equipment for Gear Cutting, Set Hobbing Machine for Operations, Carry out Hobbing Operations for spur Gear Generating, Carry out Hobbing  Operations for Helical Gear Generating.

Duration:  42 Hours					Theory: 12 Hours					Practice: 30 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Prepare Blank for Generating the Gear | Trainee will be able to: - Interpret drawing and arrange the material according to job requirement - Prepare the work-piece by required machining (sawing and filing etc.) and get it ready for turning the blank - Check and verify the dimensions of blank for generating gear as per drawing | - Interpret information given in the engineering drawings and job specifications Practical Activity: Prepare blank and verify measurement as per specific drawing | Theory-02Hrs Practical-3Hrs Total- 05Hrs | Blank for generating Gears of mild steel/ Teflon/ aluminum | Class Room and workshop |
| - Select Tools and Equipment for Gear Cutting | Trainee will be able to: - Select the material, type, shape and size of cutter(s) according to the job requirements - Arrange the measuring instruments and holding devices to attain accuracy of the work as per prescribed method | - Introduction to gear forming - Introduction to gear generating - Types Gear forming tools and equipment - Types Gear generating tools and equipment - Difference between gear forming and gear generating - Hobbing Cutters and its usage. Practical Activity: Select and mount the right tool on machine. | Theory-02Hrs Practical-03Hrs Total- 05Hrs |  | Class Room and workshop |
| - Set Hobbing Machine for Operations | Trainee must be able to: - Clamp the gear blank and hob cutter into their holding devices as per standard practice - Maintain safe distance between gear blank and hub cutter as per prescribed method - Adjust the revolution per minute (rpm) of hob cutter and table according to the specifications of work-piece - Adjust the parameters of speed and feed from control unit as per prescribed method | - Introduction to the construction of hobbing machine and its parts - Introduction to standard accessories, attachments and work holding devices - Techniques to perform hobbing operations - Procedure of using measuring instruments and gauges for hobbing machine Practical Activity: Check the accuracy of mounted Hob cutter and set Centre position of cutting gear blank. | Theory-02Hrs Practical-03Hrs Total- 05Hrs |  | Class Room and workshop |
| - Carry out Hobbing Operations for spur Gear Generating | Trainee must be able to - Produce simple/angled/differential indexing and divide the gear into required number of divisions - Check the hob cutter and the gear blank that both are positioned properly and adjust them, if required - Adjust speed feed and direction of the cutter. - Mount Gear blank on mandrel. - Hold one side of mandrel on chuck of indexing head and other side in tail stock - Start machine and carry out cutter at zero point vertically. - Carry out cutter at zero point horizontally. Apply depth for rough cut and engage machine automatically in longitudinal direction - Move table back at zero point. - Apply full depth for final cut and engage machine automatically in forward direction. - Repeat the process simultaneously until tooth is obtained | - Personal protective equipment and workplace safety for hobbing machine - Procedure of hobbing operation - Calculation of spur Gear Pitch dia, outside dia, root dia, Depth and Lead - Calculation of indexing as per No. of tooth Practical Activity: Prepare spur gear on Hobbing Machine | Theory-03Hrs Practical-12Hrs Total- 15Hrs | Blank for generating Gears of mild steel/ Teflon/ aluminum | Class Room and workshop |
| - Carry out Hobbing Operations for Helical Gear Generating | Trainee must be able to - Check the hob cutter and the gear blank that both are positioned properly and adjust them, if required - Fix indexing head and tail stock on milling table. - Adjust speed feed and direction of the cutter. - Mount Gear blank on mandrel - Hold one side of mandrel on chuck of indexing head and other side in tail stock - Start machine and carry out cutter at zero point vertically. - Carry out cutter at zero point horizontally. - Apply depth for rough cut and engage machine automatically in longitudinal direction - Move table back at zero point. - Apply full depth for final cut and engage machine automatically in forward direction. - Repeat the process simultaneously until tooth is obtained | - Helical gear design - Helical gear terminologies Practical Activity: - Mounting of Hob cutter and set Centre position of gear blank. - Prepare helical gear on Hobbing Machine as per given drawing | Theory-03Hrs Practical-09Hrs Total- 12Hrs | Gear blank generating Gears of mild steel/ Teflon/ aluminum | Class Room And workshop |

## 0715MMT3510 Perform broaching operation:

Objective: This module covers the knowledge and  skills required to Machine Irregular Surfaces, Make keyways, Make internal gears.

Duration:  42 Hours					Theory: 12 Hours					Practice: 30 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Machine Irregular Surfaces | Trainee will be able to: - Identify safety hazards related with shaping operations and take appropriate steps to avoid any injury or accident. - Dial the machine vice according to job requirement. - Select point cutting tool and set machine according to job requirements. - Mount cutting tool and work piece in the machine. - Use Different feed and speed of cutting and different points according to given drawing - Check quality of the component at suitable intervals. - Shut down the machine in safe position after finishing the work | - construction of broaching machine and its parts - Introduction to various broaching machines - Techniques to use standard accessories and attachment Practical Activity: - Perform dialing the machine vice - Mount Cutting tool - Check the efficiency the machine by operating it. | Theory-05Hrs Practical-03Hrs Total- 08Hrs |  | Class Room and workshop |
| - Make keyways | - Identify safety hazards related with shaping operations and take appropriate steps to avoid any injury or accident. - Dial the machine vice according to job requirement. - Select broaching tool and set machine according to job requirements. - Mount broaching tool and work piece in the machine. - Apply Different feed and speed of cutting and different points according to given drawing - Check quality of the component at suitable intervals. - Shut down the machine in safe position after finishing the work. | - Adopt safety precautions - Procedure of dialing the machine vice - Select the appropriate broaching tool - Mounting of tool and clamping of work piece - Procedure to Operate machine correctly Practical Activity: - Interpret the drawing of the job - Clamp the job accurately - Broach a keyway on a flat surface using a surface broach. - Shut down the machine in safe work | Theory-04Hrs Practical-12Hrs Total- 16Hrs | Square/round job of mild steel/ Teflon/ aluminum | Class Room and workshop |
| - Make internal gears | - Identify safety hazards related with shaping operations and take appropriate steps to avoid any injury or accident. - Dial the machine vice according to job requirement. - Attach indexing head on the machine. - Select broaching tool and set machine according to job requirements. - Mount broaching tool and work piece in the machine. - Use Different feed and speed of cutting and different points according to given drawing - Check quality of the component at suitable intervals. - Shut down the machine in safe position after finishing the work | - Procedure to Attach indexing head on the broaching machine. - Techniques for Selecting and mounting of broaching tool Practical Activity: - Interpret the drawing of the job - Clamp the work piece accurately - Broach keyways on a shaft by using indexing plate according to the drawing - Shut down the machine in safe work | Theory-03Hrs Practical-15Hrs Total- 18Hrs | Round shaft/hollow shaft of mild steel/ Teflon/ aluminum | Class Room and workshop |

## 0715MMT3511 Manufacture Mechanical Projects:

Objective: This module covers the knowledge and  skills required to Select the Relevant Project, Identify the Right Material, Select the Right Machining Process and Sequence, Calculate the Cost of Machining and Material, Make the Project.

Duration:  134 Hours				Theory: 08 Hours					Practice: 126 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Select The Relevant Project. | Trainee will be able to: - Analyze project requirements and objectives. - Review technical drawings and specifications. - Evaluate feasibility and resources needed. - Confirm project selection with stakeholders. | - Purpose and Types of Projects - Procedure to make drawing and feasibility report Practical Activity: Select a project and making its drawing and feasibility report. | Theory-02Hrs Practical-18Hrs Total- 20Hrs |  | Class Room and work shop |
| - Identify The Right Material. | Trainee will be able to: - Assess material properties relevant to the project. - Compare material options based on cost and availability. - Select material that meets technical specifications. - Document material selection process. | - Techniques to assessing the material for project - Selecting an economical project Practical Activity: Select right material for project | Theory-01Hrs Practical-9Hrs Total-10Hrs |  | Class Room and work shop |
| - Select The Right Machining Process and Sequence. | Trainee will be able to: - Determine suitable machining processes for the material. - Plan the sequence of machining operations. - Verify compatibility of tools and equipment. - Optimize process for efficiency and quality. | - Procedure to making sequence of operation - Selecting right tool and equipment’s Practical Activity: Make sequence of operation and select right tool and equipment’s | Theory-01Hrs Practical-15Hrs Total- 16Hrs |  | Class Room and work shop |
| - Calculate The Cost of Machining and Material. | Trainee will be able to: - Estimate material costs based on current market prices. - Calculate machining costs including labor and equipment. - Factor in overhead and additional expenses. - Prepare a detailed cost report. | - Carry estimation of cost all over the project Practical Activity: Make a report of estimation of cost of project | Theory-02Hrs Practical-09Hrs Total- 11Hrs |  | Class Room and work shop |
| - Make The Project. | Trainee will be able to: - Follow the planned machining process and sequence. - Use appropriate tools and techniques for each operation. - Monitor progress and adjust, as necessary. - Complete the project within the planned timeframe. | - Implementation of sequence of operation - Techniques to complete the project in given time Practical Activity: Finish the project to its final shape | Theory-01Hrs Practical-66Hrs Total- 67Hrs |  | Class Room And work shop |
| - Ensure Dimensional Accuracy and Quality as Per Drawing or Standard. | Trainee will be able to: - Measure critical dimensions using precision instruments. - Compare measurements to the technical drawing or standard. - Identify and rectify any deviations. - Document the inspection results and quality checks. | - Techniques for measuring the prepared project according to drawing Practical Activity: Finish and deburr the product of project according to drawings dimensions. | Theory-01Hrs Practical-09Hrs Total- 10Hrs |  | Class Room And work shop |

# 5.	List of Personal Protective Equipment

(For A Class Of 25 Students)

| Name of Trade |  | Assistant Machinist Level-2 (Mechanical) |  |
| --- | --- | --- | --- |
| Duration of Course |  | 6 Months |  |
| Sr. # |  | Description |  | Quantity |
|  |  | First AID Box |  | 5 Nos |
|  |  | Fire Extinguisher Cylinder |  | 5 Nos |
|  |  | Fire Blanket |  | 25 Nos |
|  |  | Fire Bucket |  | 25 Nos |
|  |  | Safety Gloves |  | 25 Nos |
|  |  | Ear Protector |  | 25 Nos |
|  |  | safety googles |  | 25 Nos |
|  |  | Safety Helmet |  | 25 Nos |
|  |  | Safety mask |  | 25 Nos |
|  |  | Formal Uniform for Work |  | 25 Nos |
|  |  | Safety Shoes |  | 25 Nos |

# List of Tools, Equipment, Machines and Consumables

| S.# | Name of Tools Items | Specification | Quantity |
| --- | --- | --- | --- |
| 1 | All Kinds of Files | FLAT FILE: Length of file (mm) = 300, Type of cut = Double cut. FLAT FILE: Length of file (mm)= 300, Type of cut = Single cut FLAT FILE: Length of file (mm) = 300, Type of cut= Rasp cut HALF-ROUND FILE : Length of file (mm)= 300, Type of cut = Double cut ROUND FILE; Length of file (mm) = 200, Type of cut= Double cut TRIANGULAR FILE: Length of file (mm)=150, Type of cut = Double cut. SQUARE FILE: Length of file (mm) = 200, Type of cut = Double cut. KNIFE EDGE FILE: Length of file (mm) = 250, Type of cut = Double cut. NEEDLE FILE SET(Set of 6 pieces) Length of file (mm) = 200, Type of cut = Double cut. All files made of alloy steel, hardened with wooden/Plastic file handle. | 25 each |
| 2 | Aluminum Ladder – 8 To 10 Feet Height | 8 To 10 Feet Height made of best quality aluminum alloy channel with standard support spacing. | 1 No. |
| 3 | Anvil & Hammers (Assorted Range) | ANVIL Made of cast steel hardened face and horn, square hardy hole. Weight (Kg) = 50 HAMMER (BALL PEEN) Weight of hammer (Kg) = 0.5 HAMMER (CROSS PEEN) Weight of hammer (Kg) = 0.5 HAMMER (STRAIGHT PEEN) Weight of hammer (Kg) = 0.5 All hammers made of Forged steel hardened and ground peen and face. Suitable wooden handle. | 1 No. 25 each |
| 4 | Power Vice | POWER VICE (Pneumatic/Hydraulic Operated) Material = Close grain cast Iron Jaw width = 150 mm Jaw Height = 50 mm Max. opening = 400 mm | 1 (Power Vice) |
| 5 | Bearing Pullers | Size = 4, 6 & 8 inches Made of forged steel jaws | 1 each |
| 6 | Bench Vices, Machine Vices, C-Clamps, U-Clamps, Universal Clamping Vices & Swivel Vices | BENCH VICE Made of cast steel/cast iron, hardened jaws, gun metal nut, square threaded spindle. Width of jaws (mm) = 125. Jaws opening (mm) = 150 MACHINE VICE Material = Close grain cast Iron Jaw width = 150 mm Jaw Height = 50 mm Max. opening = 400 mm C-CLAMP Max. Opening = 8 in. Body made of cast iron U-CLAMP Max. Opening = 8 in., Body made of cast iron, UNIVERSAL CLAMPING VICES: Material = Close grain cast Iron, Jaw width = 150 mm, Jaw Height = 50 mm, Max. opening = 400 mm, Compound angle setting swivel bases SWIVEL VICES Material = Close-grain cast Iron Jaw width = 150 mm Jaw Height = 50 mm Max. opening = 400 mm With a swivel base able to rotate up to 360 degrees. | 25 (bench vice) 02 (M/C Vice) 15 (C-Clamp) 15 (U-Clamp) 02 (UCV) 02 (Swivel Vice) 10 (Hand Vice) |
| 7 | Cabinets & Steel Almirah (Assorted Range) | Cabinet No. of Door = 2 Dimension = 2.5 x 2 x 1.5ft Material = MS Sheet SWG 22 Steel Almirah No. of Door = 2 Dimension = 6 x 3.5 x 1.5ft Material = MS Sheet SWG 22 | 25 each |
| 8 | Center Drills (Assorted Range) | Misc sizes ranging from 1-8 Made of High-Speed Steel | 100 each |
| 9 | Dial Indicator | DIAL INDICATOR: - For surface roughness with a magnetic stand, dial the zero setting. Range (mm) = 10 Resolution (mm) = 0.01 | 2 Nos. |
| 10 | Diamond Disc Grinding Wheels for Carbide Materials (Assorted Range) | Resin or vitrified bond grinding wheel having diameter, thickness, and bore compatible with grinding machine | 10 Nos. |
| 11 | Diamond Dressing Tip with Stand | Length: 120 mm Shank Type: Taper Shank dia.: 9mm | 5 Nos. |
| 12 | Dies & Taps Set (Assorted Range) | TAP AND DIE SET: - Consists of suitable taps and dies with tap wrenches and die stokes. System = metric. Size/Capacity (mm) = 4, 6, 8, 10, 12, 14, 16. | 2 sets |
| 13 | Drafting Tables, T-Squares & Set-Squares | Drafting Table Portable (Taper Type) Table size 32'' x 24''Tee Square 600mm Transparent Set Square Transparent medium size 300 mm | 25 Nos. 25 Nos. 25 Nos. |
| 14 | Drill Bits (Assorted Range) | TWIST DRILL SET: - Made of H.S.S 118° lip angle, hardened and ground set of twist drills in steel Box. Twist drill range = 1 to 20 mm Number of drills = 26 pcs | 3 sets |
| 15 | Drill Chuck with Key | DRILL CHUCK Three jaws hardened and ground, Key holes hardened, with Key. Capacity = 1-13 mm | 2 Nos. |
| 16 | Electric Table Lamps, Torch Lights | Electric Table Lamps Tall enough to illuminate a large area Bright, even, pleasant light Adjustable brightness and color temperature Well-built, solid, sturdy while flexible Energy Efficient LED Torch Lights Adjustable brightness More than 5 hours run time Rechargeable battery (NiCd) Typical Life: 1000 cycles Size: 10 inches Water Resistant | 2 Nos. 10 Nos. |
| 17 | Fire Extinguisher | ABC dry powder/DCP, portable, wall mounted Capacity = 8 Kg | 2 Nos. |
| 18 | First Aid Box | Suitable size having large and small gauze pads, adhesive bandages, wound cleaning agent, scissors, tweezers, adhesive tape, latex gloves, pocket mask, elastic wraps, splints | 1 No. |
| 19 | Geometry Box with Pencil, Rubber, Sharpener, Eraser & Steel Rule | Instruments Box Special Compass Set with Pencil, Rubber, Sharpener, Eraser & Steel Rule | 25 Nos. |
| 20 | Grease & Greasing Guns (Assorted Range) | GREASE GUN Suitable for soft grease or oil, metallic body funnel and pointed nozzle. Reservoir = medium size | 15 Nos. |
| 21 | Grinding Wheel Balancing Stand with Standard Accessories | Heavy-duty ball screw and linear guide Spindle motor: 1.1 kW (2800 rpm) Wheel Grinding Range(Dia.): 75-190mm Wheel Grinding Range(thickness): 25mm X-Y axes wheel stroke: 100mm Grinding Wheel motor rating: 0.5 kW | 1 No. |
| 22 | Grinding Wheels (Surface Grinding Machine, Pedestal Grinding Machine, Cylindrical Grinding Machine (Assorted Size and Shape) | Resin or vitrified bond grinding wheel having diameter, thickness and bore compatible with grinding machine | 10 Nos. |
| 23 | Hammers & Shaft Pullers | HAMMER Made of Forged steel hardened and ground peen and face. Suitable wooden handle. Weight of hammer (g) = 250, 500, 1000 Shaft Pullers Sizes: 4, 6, 8 inches made of cast iron/steel | 05 Nos. 5 Nos. 5 Nos 5 Nos. 5 5 |
| 24 | Hammers (Assorted Range) | HAMMER Made of Forged steel hardened and ground peen and face. Suitable wooden handle. Weight of hammer (g) = 250, 500, 1000 | 5 Nos. 5 Nos. 5 Nos. |
| 25 | Hand Hacksaws with Blades, Complete Sets (Assorted Range) with Standard Accessories | HAND HACKSAW FRAME Adjustable frame made of steel tube elliptical section. Suitable for blade 300 mm | 25 Nos. |
| 26 | Height Gauge, Step Gauge, Filler Gauge, Stubs Wire Gauge (Assorted Range) | HEIGHT GAUGE:- Made of high quality steel, rust resistant, base highly polished and lapped, main scale adjustable magnifying glass, with offset scriber carbide point. Working range (mm) = 0-300. Step Gauge(90 pcs set) FEELER GAUGEL:- No. of Leaves = 100. Thickness (mm) = 0.05 - 1 Stubs Wire Gauge hardened steel Range: 08-36 SWG | 2 Nos. 1 set 5 Nos. 5 Nos. |
| 27 | Knurling Tools (Assorted Range) | Knurling, rough, Medium, fine, revolving rollers Dimension of knurling rollers = 20 x 6 x 6 mm | 5 Nos. |
| 28 | Machine Repairing Tool Kits | Machine Repairing Tool Kits containing Open End Spanner Set, Allen Key Set, measuring tape, Screw Driver Set, LED Flashlight, Hammers, Pliers, Hacksaw with blade, Adjustable wrench, punches, utility knife, super glue, safety gloves, safety glasses, mask, ear plugs | 4 Nos. |
| 29 | Machine Vice Universal Clamping Vices | MACHINE VICE Material = Close grain cast Iron Jaw width = 150 mm Jaw Heigt = 50 mm Max. opening = 400 mm UNIVERSAL CLAMPING VICES Material = Close grain cast Iron Jaw width = 150 mm Jaw Heigt = 50 mm Max. opening = 400 mm Compound angle setting swivel bases | 4 Nos. 2 Nos. |
| 30 | Maintenance Box | Made of Iron/hard plastic, shock resistant Size: 400 x 300 x 150 mm | 4 Nos. |
| 31 | Measuring Instruments, Marking Tools & Gauges (Assorted Range) | Scribers (straight and off-set), punches, Try Square, Dividers, Spirit Level, Folding Rule, Measuring Tape, inside calipers, outside calipers, odd leg calipers, steel rule, Vernier Calipers, Digital Vernier Calipers, Micrometer, Thread Pitch Gauge, v-block with clamp, Bevel Protractor, surface gauge, Layout ink/dye, Tool Grinding Gauge, plug gauges, snap gauges, ring gauges. | 2 Sets |
| 32 | Multiple Hand Tools | Hand hack saw, utility knife, scissors, pliers, c-clamp, measuring tape, steel rule, hammers, mallets, screw drivers, level, adjustable wrench, pipe wrench, spanners, Allen keys, socket wrench. | 5 Sets |
| 33 | Overall Combination | Overall Combination Arm pocket, back pocket (optional), chest pocket, elasticated waist band, hip pocket wide side access, stud fastened collar, two way zip, 65% polyester, 35% cotton | 25 Nos. |
| 34 | Personal Protective Equipment | Overalls, safety helmet, wide brimmed hat, safety footwear, safety glasses and goggles, gloves, respirators, masks, earmuffs and earpieces. | 30 Sets |
| 35 | Plug and Snape Gauge | Material: Tool Steel/chrome steel/carbide Set of 4 pcs 12, 15 ,18, 20 mm Material: Tool Steel/chrome steel/carbide Sizes: 10, 20, 30 mm according to ANSI Standard | 2 Sets (Each) |
| 36 | Plug Gauge (Assorted Range) | Material: Tool Steel/chrome steel/carbide Set of 4 pcs 12, 15 ,18, 20 mm | 2 Sets |
| 37 | Prick Punch (Assorted Range & Types) | PRICK PUNCH Made of high quality steel, hardened and ground point, knurled shank, point angle 60 deg. Length (mm) = 100, 125, 150 Diameter (mm) = 8, 10, 12 | 25(each) |
| 38 | Radius Gauge - Concave & Convex (Assorted Range) | Radius Gauge - Concave & Convex Individually marked, rust proof steel Range: 1-3 mm R---0.25 step & 3.5-7mm R---0.5 step No. of Plates: 30 | 5 Nos. |
| 39 | Reamers (Assorted Range) | High Speed Steel, Straight/Taper shank, straight flute, right hand cut. Sizes: 12, 16, 18, 20,22, 24 mm | 2 Sets |
| 40 | Safety Equipment - Safety Shoes, Safety Gloves, Safety Goggles, Safety Helmet and Ear Plugs | Made of plastic/ glass/leather of best quality to promise safe working in workshop and outdoor environment | 30 Nos. |
| 41 | Scientific Calculator & Multimedia Projector | Scientific Calculator 10 digits display, should be able to process mathematics and statistics functions efficiently Multimedia Projector Display: LCD/LED Pixels:1024 x 768 Brightness: 4000 ANSI lumens minimum Color System: PAL/SECAM/NTSC Computer Compatibility: UXGA/WXGA/SXGA/XGA/SVGA/VGA Projection Method: Front | 30 Nos. 1 Nos. |
| 42 | Shaper Tools (Assorted Range) | SHAPER TOOL Made of high-speed steel cobalt alloy hardened and ground 5/8” x 5/8” x 8” 3/4" x 3/4” x 8” | 10 Each |
| 43 | Smoke Detecting Alarm | Smoke Detecting Alarm (Best Quality) Able to detect smoke easily | 1 Nos. |
| 44 | Spray Guns & Oil Containers | Spray Guns Nozzle Size = 1.4 - 1.8mm Air Inlet = 1/4 inch Type of feed = Gravity Recommended Air Pressure = 2 bar Approx. Paint Capacity = 1000ml Oil Containers Transparent body having graduated marking 10 liter/quart drum body and stretch spout lid with proper sized opening. | 5 Nos. 5 Nos. |
| 45 | Standard Lathe Machine attachments | STANDARD LATHE MACHINE ATTACHMENTS 3-jaw self-centering chuck 4-jaws independent chuck Face plate with slots Square tool post Steady rest Follow rest Longitudinal stop, Mandrill Lathe Dog Dead centers Live Centers, Revolving Center Taper sleeve | 10 Sets |
| 46 | Steel Cabinet and Steel Almirah (03 for Each Machine) | Cabinet No. of Door = 2 Dimension = 2.5 x 2 x 1.5 Material = MS Sheet SWG 22 Steel Almirah No. of Door = 2 Dimension = 6 x 3.5 x 1.5 Material = MS Sheet SWG 22 | 25 Each |
| 47 | Stencil of Various Shapes - Circle, Square Box, Ellipse, French Curves etc. | Stencil of Various Shapes - Circle, Square Box, Ellipse, French Curves made of high quality thermosetting and durable plastic. | 25 Each |
| 48 | Step Gauges Set | Rectangular shape, micro lapped, heat treated, high corrosion resistance steel. No of pieces per set = 103 Size (mm) = 1.01-1.49 Steps = 0.01 mm | 1 Set |
| 49 | Surface Analysis and Inspection Gauges (Assorted Range) | Type: Flat Surface Processing: Paper Finish, Grinding, Shaping, Milling | 1 No. |
| 50 | Surface Plate with Stand (Cast Iron / Graphite) 24 X 24 Inches | CAST IRON SURFACE PLATE Made of close grain cast-iron high-quality finish. Length x Width x Thickness(inch) = 24 x 24 x 1 | 2 Nos. |
| 51 | Threading Tools (Assorted Range) | Made of H.S.S. able to cut BSW and American Standard Threads, Acme thread, square thread Angle: 55 and 60 degrees | 10 Sets |
| 52 | Threads Gauge -Inches / Millimeters (Assorted Range) | Threads Gauge -Inches / Millimeters Size = 4 - 42 TPI and 0.5-10mm No of leaves = 30 Angle (degrees) = 55 & 60 | 5 Nos. |
| 53 | Turning, Parting, Grooving and Forming Tools etc. (Assorted Range) | Made of H.S.S. 200 mm long with square and rectangular cross sections | 10 Sets |
| 54 | Twist Drill Bits and Boring Bars (Assorted Range) | TWIST DRILL SET: - Made of H.S.S 118° lip angle, hardened and ground set of twist drills in steel Box. Twist drill range = 1 to 20 mm Number of drills = 26 pcs Boring Bars with holders Sizes with H.S.S. tool bits = 6 mm | 3 Sets 10 Sets |
| 55 | Wooden Cabinet, Steel Almirah | Wooden Cabinet No. of Door = 2 Dimension = 6 x 3.5 x 1.5 Material = Deodar wood Steel Almirah No. of Door = 2 Dimension = 6 x 3.5 x 1.5 Material = MS Sheet SWG 22 | 5 Each |
| 56 | Wooden Cabinets | Wooden Cabinet No. of Door = 2 Dimension = 6 x 3.5 x 1.5 Material = Deodar wood | 5 Nos. |
| 57 | Work-Bench, Work-Table & Stools | Work-Bench L X W X H = 5 X 3 X 2.5 ft. Angle Iron used for legs 64x5mm Wooden Top 40mm thick Work-Table L X W X H = 8 X 4 X 2.5 ft. Stools Four legs made of MS pipe gauge 16SWG 2ft high, 1.25 x 1.25 ft square Wooden seat 25mm thick. | 25 Sets |
| 58 | Universal Milling Machine with Standard Accessories | Table size – 1000x260mm (minimum),Number of T-slots-3 (Minimum),T-slot size -14-16mm,Spindle to table-0-350mm,Number of spindle speeds – 12 steps, Spindle speed range – 50 to 1200 rpm, Swivel of table 45ᵒ,Spindle taper: Horizontal: ISO40,Vertical: ISO40,Table travel Vertical -400-450mm,Table Travel Cross – 280-300mm,Main spindle motor – 3-3.5KW 3Ph,Coolant pump, Machine Weight – 1100 Kg (minimum), without any attachment and accessories. Accessories: • Milling Machine vice, • Tail stock, • Milling arbor 400mmxΦ27mm, • Milling arbor 400mm x Φ22mm, • Stub arbor Φ27mm, • Stub arbor Φ22mm. Standard Accessories: Tool Kit(Wrench 8”,10”,12”, Gear Puller, Screw Driver Set, Plier, Nose Plier, Grip Plier),spanner Set ,Collet holders & collets, L-Key set, Coolant System, Machine light, Centralized lubrication, Table guard,3 Axis Digital Read Out, chip tray for Base, Dividing head (Ratio 40:1) Swivel based, 3 jaw chuck (outer and inner jaws), Gear set of 12 gears, Spiral index system, Standard indexing plates, Differential and direct indexing, • Slotting head,• Universal Head (Vertical),• Rotary Table | 2 Nos, |
| 59 | Standard Milling Machine Attachments (Sets) | Milling Machine vice, Swivel Base vice, Tailstock, Milling arbor 400mmxΦ27mm, Milling arbor 400mm x Φ22mm, Stub arbor Φ27mm, Stub arbor Φ22mm, Tool Kit(Wrench 8”,10”,12”, Gear Puller, Screw Driver Set, Plier, Nose Plier, Grip Plier), spanner Set, Collet holders & collets, L-Key set,3 Axis Digital Read Out, Dividing head (Ratio 40:1) Swivel based, 3 jaw chuck (outer and inner jaws), Gear set of 12 gears, Spiral index system, Standard indexing plates, Differential and direct indexing, Slotting head, Universal Head (Vertical). | 2 Nos. |
| 60 | File Brush | 3 inches | 25 Nos. |
| 61 | Digital Vernier Caliper | 160mm | 02 Nos |
| 62 | Counter sink | 18x60⁰, 12.5x50⁰ | 5 set |
| 63 | Counter Bore | 6, 10, 12 mm | 5 set |
| 64 | Digital Micro Mete | 0-25 mm | 02 Nos |
| 65 | Digital Micrometer | 25-50 mm | 02 Nos |
| 66 | Chisel Flat, Groove, Crosscut | 8 inches | 5 Each |
| 67 | Number Punch | 4 mm | 02 Sets |
| 68 | Letter Punch | 4 mm | 02 Sets |
| 69 | Lathe Machine (with standard accessories) | Center height – 160-200 mm, Maximum swing over bed – 320-400mm, Maximum swing over saddle – 180-190mm, Maximum swing in gap – 460-470mm, Length of gap – 200-250mm, Distance between centers - 750mm, Spindle bore – 38mm, Spindle taper – MT5, Tailstock taper – MT3, Bed width – 180-190mm, Number of spindle speeds – 8, Range of spindle speeds – 70-2000 rpm, Metric threads (26 pitches) – 0.4-7mm, Imperial threads – (34 pitches) – 4-60 TPI, Lead screw dia./pitch – 22/3mm, Travel of compound slide: 75mm, Travel of cross slide – 130mm, Tailstock quill travel – 100mm, Main Motor – 1.5-2 KW 3phase, Coolant Motor, Tool Section – 16mm, Net Weight of machine – 525 to 550 Kg without any attachment and accessories. Standard Accessories: 160mm 3 jaw chuck(Universal), 200mm 4 jaw chuck(Independent), 250mm face plate, Fixed steady, Traveling steady, Electrically interlocked chuck guard, 4 way indexing tool post stand, Tool post chip guard, Rear splash guard, Spindle foot brake, Thread dial indicator, Coolant system, Low volt machine light, Tool box(Spanner Set, L-Key Set, Wrench 10”, Puller, Screw Driver Set, Plier, Nose Plier, Grip Plier). | 10 Nos. |
| 70 | Pedestal Grinder with Tools, Cutting Angle Support | Ball bearing mounted spindle, suitable for tool grinding, protection glass/clear plastic eye shield with lamp, grinding wheel = 250 x 30 x 25 mm, Spindle speed = 2500-3000 rpm, Motor rating 3 hp/440V/50 Hz = 600–750-Watt, Quick angle setting of = 30-90 degree. Attachment: Wheel dresser star=Standard type / Diamond, Complete in all respects. | 2 Nos. |
| 71 | Surface Grinding Machine | Working surface of Table (W x L) 250 x 500mm Minimum, Table travel (y,x)-250 x 550mm Minimum, Minimum distance spindle centre-table-400mm, Magnetic Chuck-size 200 x 450mm, Longitudinal feeds 5-25m/min, Automatic Cross Feed of Saddle 0.5-10mm, Grinding wheel size- 180 x 13 x 31.75mm, Speed-2800 rpm Minimum, Spindle motor -1.5-2 KW, 3phase, Hydraulic pump motor-0.75 KW, Coolant pump motor, Elevating motor, Cross feed motor, Total rated power- 3KW, 3phase, Maximum table load incl.chuck-120 Kg, Net weight of machine-750 Kg Minimum. STANDARD ACCESSORIES: Wheel balancing base, Wheel balancing arbor, Grinding wheel, Diamond dresser fit on the top of machine spindle grinding wheel, Toolbox (Spanner Set, L-Key Set, Wrench 10”, Puller, Screw Driver Set, Plier, Nose Plier, Grip Plier), Low volt machine light, Built in demagnetizer, Coolant system, 200 x 450mm electro-magnetic chuck. | 2 Nos. |
| 72 | Cylindrical Grinding Machine | Distance between center = 500-600mm, Center height = 120-150 mm, Max dia. grind 100mm, Spindle taper MT4, Maximum transverse Travel = 120-150mm, Swiveling angle 0-10˚, Spindle speed = 2500-3000 RPM, Tail stock taper MT4, Motor 4Kw, Internal Grinding motor 1-1.25Kw, Work head motor 0.5-1Kw, Hydraulic longitudinal feed, Wheel head manual feed, Wheel head & Work head (Swivel Type). | 2 Nos. |
| 73 | Tool/Cutter Grinding Machine | Suitable to grind all types of tool and cutters, Clamping area of table (mm) =820x 140, Longitudinal table travel (mm) =450, Lateral table travel (mm) = 225,Distance between centers (mm) = 600-650,Height of center (mm) =125-130,Spindle speed (rpm) =2800- 5600,Motor power 3ph = 1 kW, Table angle setting (degrees) =±450,Grinding head tilting on vertical axis (degrees) = ±3600,Grinding head tilting on horizontal axis (degrees) = ±150. Attachments: Face milling cutter grinding attachment (mm) = 450, Radius grinding attachment (mm) = 0-25,Cylindrical grinding attachment (mm) = 250,Internal grinding attachment (mm) =12-50,Indexing attachment = 01,Extended grinding wheel spindle (mm) = 75,Universal vice (mm) = 100x25x75,Drill point grinding attachment (mm) = 3-25,Lathe and planer tool grinding attachment (mm) =10-38,Set of spring collets (mm) = 6, 8, 10, 12 ,16, 20. Accessories : Work head = 1,Left hand tail stock = 1,Right hand tail stock = 1,Diamond wheel dresser = 1,Setoff wheel guards = 1,Universal tooth rest = 1,Center height gauge = 1.Collect wrench = 1,Ejector rod = 1,T- wrench = 1,Pin wrench = 1,Grinding wheel sleeves = 5,Set of grinding wheels different shapes = 7,Spindle extension 100 mm =1,Belt for spindle drive = 1,Suitable tool cabinet = 1,Instruction / operation manual, complete in all respect, COMPLETE AND COMPATIBLE IN ALL RESPECTS, OR EQUIVALENT OF BEST QUALITY. | 1 No. |
| 74 | Centerless Grinding Machine along with Standard Accessories | Speed = 20-220 RPM (Step less Variable), Grinding Wheel = Diameter - 350 mm x 100 Width, Bore Size = 127mm,Grinding Capacity = Dia. - 2mm (min), 70mm (max),Machine Weight = 2200 KG (Approx.),Coolant Pump Motor = 0.12 KW, Regulating Wheel Motor = 0.75 KW DC, Width = 100 mm, Regulating Wheel = Diameter - 230 mm, Swivel Angle = Swivel of grinding dresser - +/- 5 Degree, Regulating Swivel Angle = +/- 5 Degree Horizontal, Dresser Operation = Grinding & Regulating Wheel - Hydraulics, Power Requirement = Grinding Wheel Motor - 5.5 KW, Power Pack Pump = 0.18 KW. | 1 No. |
| 75 | Pillar Drilling Machines | PILLAR DRILL MACHINE: - V-belt drive with 12 spindle speeds, table raise/lower by means of gear and rack. Chuck capacity (mm) = 16 Depth of feed (mm) = 80 Distance pillar to spindle center(mm) = 300 Table size (mm) = 230 x 240. Spindle speeds (rpm) = 220 – 3000. Spindle Morse taper number = MT-2. Motor 3-Ph./440V/50Hz = 0.75KW. Drill Press Vice = 100 mm. Drill chuck with arbor = 1-16mm. Magnet stator switch of on/off | 1 No. |
| 76 | Power Hacksaw Machine | Hydraulic Power Hacksaw, variable cutting pressure, quick return of frame, cooling pump, Capacity Ø = 180-225 mm, Capacity ◊ = 230 x 230 mm, Miter 450 = 190mm,Strokes/mint = 40-160,Blade size = 450 x 32 x 1.6mm,Motor rating 3 ph 1440v/50 Hz.=0.75-1.5 KW, Adjustable material stand = 450-650mm,Width of stand = 350mm,Separate motor operated 440v, 3Ph, 50Hz 2.5 to 3 Kw, coolant supply attachment motor operated, Complete in all respect and standard accessories, COMPLETE AND COMPATIBLE IN ALL RESPECTS, , OR EQUIVALENT OF BEST QUALITY. | 2 No. |
| 77 | Shaper Machine with Standard Accessories | Stroke length = 400-450mm, No. of speeds =2-8, Strokes per mint = 20-130, Vertical Travel of tool slide=150mm or greater, Table automatic feed (5)=0.2 to 2 mm/stroke, Motor rating 3 ph/440v/50 Hz =1.25-1.5 KW, Shaper Vice (Swivel Base)=250-300 mm width of jaws, Note: - Along with all standard accessories machine handles, set of spanner. Ellen Key set and complete in all respect. | 2 No. |
| 78 | Work Bench and Stools (Set) | Work-Bench L X W X H = 5 X 3 X 2.5 ft. Angle Iron used for legs 64x5mm Wooden Top 40mm thick Stools Four legs made of MS pipe with wall thickness 3mm, 2ft high 1.25 x 1.25 ft square Wooden seat 200mm thick. | 5 Nos. |
| 79 | Machine Vices, Swivel Vices (Each) | MACHINE VICE Material = Close grain cast Iron Jaw width = 150 mm Jaw Height = 50 mm Max. opening = 400 mm SWIVEL VICES Material = Close grain cast Iron Jaw width = 150 mm Jaw height = 50 mm Max. opening = 400 mm With swivel base able to rotate up to 360 degrees. | 2 Nos. |
| 80 | Measuring and Marking Tools (Assorted Range) (Sets) | Scribers (straight and off-set), punches, Try Square, Dividers, Spirit Level, Folding Rule, Measuring Tape, inside calipers, outside calipers, odd leg calipers, steel rule, Vernier Calipers, Digital Vernier Calipers, Micrometer, Thread Pitch Gauge, v-block with clamp, Bevel Protractor, surface gauge, Layout ink/dye. | 2 Nos. |
| 81 | Work Holding Devices and Attachments | MACHINE VICE Material = Close grain cast Iron Jaw width = 150 mm Jaw height = 50 mm Max. opening = 400 mm | 2 Nos. |
|  |  | SWIVEL VICES Material = Close grain cast Iron Jaw width = 150 mm Jaw height = 50 mm Max. opening = 400 mm With swivel base able to rotate up to 360 deg | 4 Nos. |
|  |  | C-CLAMP Max. Opening = 8 in. Body made of cast iron | 10 Nos. |
|  |  | U-CLAMP Max. Holding capacity = 200mm | 10 Nos. |
| 82 | Plug and Snape Gauges (Sets) | Material: Tool Steel/chrome steel/carbide Set of 4 pcs 12, 15 ,18, 20 mm Material: Tool Steel/chrome steel/carbide Sizes: 10, 20, 30 mm according to ANSI Standard | 2 Nos. |
| 83 | Twist Drill Bits and Boring Bars (Assorted Range) | TWIST DRILL SET (set): - Made of H.S.S 118° lip angle, hardened and ground set of twist drills in steel Box. Twist drill range = 1 to 20 mm Number of drills = 26 pcs | 3 Nos. |
|  |  | Boring Bars Miscellaneous Sizes with H.S.S. tool bits (set) | 10 Nos. |
| 84 | Side and Face Cutters, End Mill Cutters, Ball Mill Cutters, Shell-End Mill Cutters, Slotting Cutters, Grooving Cutters etc. (Assorted Range) Each | Side and Face Cutters = 100 x 8 x 27, 100 x 10 x 27, 100 x 12 x 27, 100 x 14 x 27 End Mill Cutters = 5, 6, 8, 10, 12 , 14 , 16 Ball Mill Cutters = 5, 6, 8, 10, 12 , 14 , 16 Shell-End Mill Cutters = 40 x 40 x 27, 60 x60 x 27, 100 x 50 x 27 Slotting Cutters = 80 x 3 x 27, 80 x 5 x 27 Grooving Cutters = 63 x 2 x 22 Made of H.S.S. and cutter bore(22 or 27mm) compatible with milling arbor diameter. | 10 Nos. |
| 85 | Common Kinds and Sizes Of Files (Assorted Range) | FLAT FILE Length of file (mm) = 300, Type of cut = Double cut. FLAT FILE Length of file (mm)= 300, Type of cut = Single cut FLAT FILE Length of file (mm) = 300, Type of cut= Rasp cut HALF ROUND FILE Length of file (mm)= 300, Type of cut = Double cut ROUND FILE Length of file (mm) = 200, Type of cut= Double cut TRIANGULAR FILE Length of file (mm)=150, Type of cut = Double cut. SQUARE FILE Length of file (mm) = 200, Type of cut = Double cut. KNIFE EDGE FILE Length of file (mm) = 250, Type of cut = Double cut. NEEDLE FILE SET(Set of 6 pieces) Length of file (mm) = 200, Type of cut = Double cut. All files made of alloy steel, hardened with wooden/Plastic file handle. | 25 Nos. |
| 86 | Steel Cabinet and Steel Almirah (03 for Each Machine) Each | Cabinet No. of Door = 2 Dimension = 2.5 x 2 x 1.5 Material = MS Sheet SWG 22 Steel Almirah No. of Door = 2 Dimension = 6 x 3.5 x 1.5 Material = MS Sheet SWG 22 | 25 Nos. |
| 87 | Hammers (Assorted Range) Each | HAMMER Made of Forged steel hardened and ground peen and face. Suitable wooden handle. Weight of hammer (g) = 250, 500, 1000 | 5 Nos. |
| 88 | Personal Protective Equipment (Sets) | Overalls, safety helmet, wide brimmed hat, safety footwear, safety glasses and goggles, gloves, respirators, masks, earmuffs and earpieces. | 30 Nos. |
| 89 | First Aid Box | Suitable size having large and small gauze pads, adhesive bandages, wound cleaning agent, scissors, tweezers, adhesive tape, latex gloves, pocket mask, elastic wraps, splints | 1 Nos. |
| 90 | Maintenance Tool Kit, General Repairing Tool Kit & Hands Tool Kit (Sets) | Machine Repairing Tool Kits containing Open End Spanner Set, Allen Key Set, measuring tape, Screw Driver Set, LED Flashlight, Hammers, Pliers, Hacksaw with blade, Adjustable wrench, punches, utility knife, super glue, safety gloves, safety glasses, mask, ear plugs | 2 Nos. |
| 91 | Radius Gauge - Concave & Convex (Assorted Range) | Radius Gauge - Concave & Convex Individually marked, rust proof steel Range: 1-3 mm R---0.25 step & 3.5-7mm R---0.5 step No. of Plates: 30 | 5 Nos. |
| 92 | Dial Indicator with Magnet Stand | DIAL INDICATOR: - For surface roughness with magnetic stand, dial zero setting. Range (mm) = 10 Resolution (mm) = 0.01 | 2 Nos. |
| 93 | Involute Gear Cutter Sets | H.S.S. M1 PA14, H.S.S. M1.75 PA14, H.S.S. M2 PAPA14, H.S.S. M2.5 PA14 08 cutters in each module (1 Set (Each Module) | 1 Nos. |
| 94 | Hob Cutters Sets | H.S.S. M1 PA20, H.S.S. M1.75 PA20, H.S.S. M2 PA20, H.S.S. M2.5 PA20 (1 Set (Each Module) | 1 Nos. |
| 95 | Tooth Vernier | TOOTH VERNIER: - Suitable to measure gear tooth thickness in module system. Horizontal beam (mm)= 100 Vertical beam (mm) = 80 LC (mm) = 0.02 | 2 Nos. |
| 96 | Lathe Chucks and Steady Rests (Assorted Range) Sets | 3-jaw self-centering chuck = 160 mm 4-jaws independent chuck = 200 mm Steady rest | 2 Nos. |
| 97 | Turning, Parting, Grooving and Forming Tools etc. (Assorted Range) Sets | Made of H.S.S. 200 mm long with square and rectangular cross sections | 10 Nos. |
| 98 | Universal Milling Machine with Standard Accessories | Table size – 1000x260mm (minimum),Number of T-slots-3 (Minimum),T-slot size -14-16mm,Spindle to table-0-350mm,Number of spindle speeds – 12 steps, Spindle speed range – 50 to 1200 rpm, Swivel of table 45ᵒ,Spindle taper: Horizontal: ISO40,Vertical: ISO40,Table travel Vertical -400-450mm,Table Travel Cross – 280-300mm,Main spindle motor – 3-3.5KW 3Ph,Coolant pump, Machine Weight – 1100 Kg (minimum), without any attachment and accessories. Accessories: • Milling Machine vice, • Tail stock, • Milling arbor 400mmxΦ27mm, • Milling arbor 400mm x Φ22mm, • Stub arbor Φ27mm, • Stub arbor Φ22mm. Standard Accessories: Tool Kit(Wrench 8”,10”,12”, Gear Puller, Screw Driver Set, Plier, Nose Plier, Grip Plier),spanner Set ,Collet holders & collets, L-Key set, Coolant System, Machine light, Centralized lubrication, Table guard,3 Axis Digital Read Out, chip tray for Base, Dividing head (Ratio 40:1) Swivel based, 3 jaw chuck (outer and inner jaws), Gear set of 12 gears, Spiral index system, Standard indexing plates, Differential and direct indexing, • Slotting head,• Universal Head (Vertical),• Rotary Table | 6 Nos. |
| 99 | Power Hacksaw Machine with Blade | Hydraulic Power Hacksaw, variable cutting pressure, quick return of frame, cooling pump, Capacity Ø = 180-225 mm, Capacity ◊ = 230 x 230 mm, Miter 450 = 190mm,Strokes/mint = 40-160,Blade size = 450 x 32 x 1.6mm,Motor rating 3 ph 1440v/50 Hz.=0.75-1.5 KW, Adjustable material stand = 450-650mm,Width of stand = 350mm,Separate motor operated 440v, 3Ph, 50Hz 2.5 to 3 Kw, coolant supply attachment motor operated, Complete in all respect and standard accessories, COMPLETE AND COMPATIBLE IN ALL RESPECTS, , OR EQUIVALENT OF BEST QUALITY | 1 Nos. |
| 100 | Standard Milling Machine Attachments (Sets) | Milling Machine vice, Swivel Base vice, Tail stock, Milling arbor 400mmxΦ27mm,Milling arbor 400mm x Φ22mm,Stub arbor Φ27mm,Stub arbor Φ22mm,Tool Kit(Wrench 8”,10”,12”, Gear Puller, Screw Driver Set, Plier, Nose Plier, Grip Plier),spanner Set, Collet holders & collets, L-Key set,3 Axis Digital Read Out, Dividing head (Ratio 40:1) Swivel based, 3 jaw chuck (outer and inner jaws), Gear set of 12 gears, Spiral index system, Standard indexing plates, Differential and direct indexing, Slotting head, Universal Head (Vertical). | 6 Nos. |
| 101 | Boring Head with Boring Tools (Sets) | Compatible with vertical milling machine able to perform boring operation using various boring tools | 2 Nos. |
| 102 | Gear Generator Machine, Gear Hobbing Machine along with Standard Accessories | Vertical differential type machine capable to cut spur gears, helical gears and worm gears, heaving even, odd, prime number of teeth. Built in coolant and lubrication pump, Maximum module for gears (mm) = 6,Maximum diameter of gear hob wit out outer support (mm) = 400-500,Maximum diameter of helical gears hobbed (mm) = 370,Hob diameter (mm) = 120,Hob arbor diameter (mm) = 22, 27, 32.Work mandrel diameter (mm) = 30,Spindle speeds (8) rpm = 50-300,Hob down feed per rev. of job (mm) = 0.24-4,Main drive motor 3Ph 1420 rpm = 3kw,With all standard accessories, tool kit, tool cabinet, operation manual, Maintenance manual etc, COMPLETE AND COMPATIBLE IN ALL RESPECTS, , OR EQUIVALENT OF BEST QUALITY. | 2 Nos. |
| 103 | Lathe Machine along with Standard Accessories | Centre height – 160-200 mm, Maximum swing over bed – 320-400mm,Maximum swing over saddle – 180-190mm,Maximum swing in gap – 460-470mm,Length of gap – 200-250mm,Distance between centers - 750mm,Spindle bore – 38mm,Spindle taper – MT5,Tailstock taper – MT3,Bed width – 180-190mm,Number of spindle speeds – 8,Range of spindle speeds – 70-2000 rpm, Metric threads (26 pitches) – 0.4-7mm,Imperial threads – (34 pitches) – 4-60 TPI, Lead screw dia./pitch – 22/3mm,Travel of compound slide : 75mm,Travel of cross slide – 130mm,Tailstock quill travel – 100mm,Main Motor – 1.5-2 KW 3phase,Coolant Motor ,Tool Section – 16mm,Net Weight of machine – 525 to 550 Kg without any attachment and accessories, Standard Accessories,160mm 3 jaw chuck(Universal),200mm 4 jaw chuck(Independent),250mm face plate, Fixed steady, Traveling steady, Electrically interlocked chuck guard,4 way indexing tool post stand, Tool post chip guard, Rear splash guard, Spindle foot brake, Thread dial indicator, Coolant system, Low volt machine light, Tool box(Spanner Set, L-Key Set, Wrench 10”, Puller, Screw Driver Set, Plier, Nose Plier, Grip Plier). | 6 Nos. |
| 104 | Power Hacksaw Machine along with Standard Accessories | Hydraulic Power Hacksaw, variable cutting pressure, quick return of frame, cooling pump, Capacity Ø = 180-225 mm, Capacity ◊ = 230 x 230 mm, Miter 450 = 190mm,Strokes/mint = 40-160,Blade size = 450 x 32 x 1.6mm, Motor rating 3 ph 1440v/50 Hz.=0.75-1.5 KW, Adjustable material stand = 450-650mm, Width of stand = 350mm, Separate motor operated 440v, 3Ph, 50Hz 2.5 to 3 Kw, coolant supply attachment motor operated, Complete in all respect and standard accessories, COMPLETE AND COMPATIBLE IN ALL RESPECTS, OR EQUIVALENT OF BEST QUALITY. | 2 Nos. |
| 105 | Pedestal Grinder with Tools Cutting Angle Support | Ball bearing mounted spindle, suitable for tool grinding, protection glass/clear plastic eye shield with lamp, grinding wheel = 250 x 30 x 25 mm, Spindle speed = 2500-3000 rpm, Motor rating 3 ph/440V/50 Hz = 600–750-Watt, Quick angle setting of = 30-90 degree. Attachment: Wheel dresser star=Standard type / Diamond, Complete in all respect. | 2 Nos. |

List of Consumables

| Sr. No | Name | Qty. Required |
| --- | --- | --- |
| 1 | Cleaning Supplies (E.G., Wipes, Disinfectants) | Sufficient Supply |
| 2 | Safety Posters | 10-15 Nos. |
| 3 | Markers And Highlighters | 30 Nos. |
| 4 | Compostable Bags | 10 Rolls |
| 5 | First Aid Kit | 10 Nos. |
| 6 | Fire Extinguishers | 5 Nos. |
| 7 | Rags And Wipes | 40 Kg |
| 8 | Gloves | 100 Pairs |
| 9 | First Aid Supplies | 5 Kits |
| 10 | Masking Tape | 50 Rolls |
| 11 | WD-40 or Equivalent Lubricant | 5 Cans |
| 12 | Paper Towels | 50 Rolls |
| 13 | Lubricating Oil | 100 Liters |
| 14 | Grease | 3 Kg |
| 15 | Machine Oil | 50 Liters |
| 16 | Trash Bags | 50 Nos. |
| 17 | Disposable Gloves | 100 Pairs |
| 18 | Cleaning Detergent | 3 Kg |
| 19 | Safety Goggles | 50 Nos. |
| 20 | Kerosene Oil | 20 Liter |
| 21 | Lubricating Oil | 70 Liters |
| 22 | Sandpaper | 150 Sheets |
| 23 | Polishing Compound | 5 Nos. |
| 24 | Polishing Cloths | 50 Nos. |
| 25 | Ear Protection | 25 Nos. |
| 26 | Safety Gloves | 50 Pairs |
| 27 | Hand Sanitizer | 10 Bottles |
| 28 | Disposable Masks | 50 Nos. |
| 29 | Hand Grinding Disk | 30 Nos. |
| 30 | Twist Drill Set | 10 Set |
| 31 | File Brush | 10 Nos. |
| 32 | Grinding Wheels | 5 Nos. |
| 33 | Cutting Oil | 50 Liter |
| 34 | Aprons | 30 Nos. |
| 35 | Surface Grinding Wheels | 2 Nos. |
| 36 | Cylindrical Grinding Wheels | 2 Nos. |
| 37 | Tool/Cutter Grinding Wheels Different Types of Sets | 2 Sets. |
| 38 | Paper Towels | 50 Nos. |
| 39 | Rags And Wipes | 10 Kg |
| 40 | Carbide Tools of Different Shapes and Sizes | 50 Nos. |
| 41 | Tool Bits of Different Shapes and Sizes | 50 Nos. |
| 42 | Chips Cleaning Brushes | 50 Nos. |
| 43 | Shaper Cutting Tools of Different Types and Shapes | 25 Nos. |
| 44 | Tool Bits of Different Sizes and Shapes | 100 Nos. |
| 45 | Milling Inserts Different Types | 100 Nos. |
| 46 | Milling Cutters of Different Types and Shape | 100 Nos. |

- Members of Curriculum Development Committee,

The Curriculum Development, Committees consisted of the following members:

| S. # | Name | Designation | Organization |
| --- | --- | --- | --- |
| 1. | Engr. Inayat ur Rehman | DACUM Facilitator | Retired from KP-TEVTA |
| 2. | Engr. Hafiz Naveed Rashid | Instructor Machinist | GTVC (B) Mansehra KP-TEVTA |
| 3. | Engr. Muhammad Dawood ul Hassan | Jr. Instructor Mechanical | P-TEVTA |
| 4. | Mr. Faheem Naeem | Instructor Machinist | P-TEVTA |
| 5. | Mr. Babar Hussain | Instructor Machinist | P-TEVTA |
| 6. | Mr. Asim Jamal | DD SS&C (Coordinator) | NAVTTC HQ. Islamabad |
| 7. | Mr. Muhammad Ishaq | DD SS&C (Coordinator) | NAVTTC HQ. Islamabad |
| 8. |  |  |  |
| 9. |  |  |  |
| 10 |  |  |  |